3GPP TSG-CT Meeting #109	CP-252212

# Beijing, P.R.China, 15-16 September 2025	(was C1-255686)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4441 | rev | 4 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Collision of TAU and MT SM procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If a tracking area updating procedure is ongoing, no SM procedure shall be initiated. While for the UE this is defined (see clause 6.1.1), a similar requirement is missing for the network. Rev-4 Update to CT #109: In the previous version (rev-3) at CT1 #156, in C1-255686, an incorrect CR was included. Instead of the 24.301 version of the CR (#4441), erroneously the 24.501 version of the CR (#6942 from C1-255687) was included. Thus, the tdoc numbers C1-255686 and C1-255687 were swapped at CT1 #156. Instead of C1-255686 including the 24.301 version of the CR, it included the 24.501 version…and C1-255687 ended up including the 24.301 version of the CR, instead of including the 24.501 version. The error has since been corrected in this revision to CT #109. There has been no change to the technical content of the CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If handover or inter-system handover to S1 mode is ongoing and the TA of the target cell is not included in the TAI list, the MME shall suspend the transmission of ESM messages to the UE until the tracking area updating procedure has been successfully completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk that MT SM procedures which are initiated while a tracking area updating procedure is ongoing are not handled at all. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-4 Update to CT #109: In the previous version (rev-3) at CT1 #156, in C1-255686, an incorrect CR was included. Instead of the 24.301 version of the CR, erroneously the 24.501 version of the CR (from C1-255687) was included. Thus, the tdoc numbers C1-255686 and C1-255687 were swapped at CT1 #156. Instead of C1-255686 including the 24.301 version of the CR, it included the 24.501 version…and C1-255687 ended up including the 24.301 version of the CR, instead of including the 24.501 version. The error has since been corrected in this revision to CT #109. There has been no change to the technical content of the CR. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 4.2	Linkage between the protocols for EPS mobility management and EPS session management

During the EPS attach procedure, the network can activate a default EPS bearer context (i.e. if the UE requests PDN connectivity in the attach request). Additionally, the network can activate one or several dedicated EPS bearer contexts in parallel for PDN connections of IP or Ethernet PDN type. To this purpose the EPS session management messages for the default EPS bearer context activation can be transmitted in an information element in the EPS mobility management messages. In this case, the UE and the network execute the attach procedure, the default EPS bearer context activation procedure, and the dedicated EPS bearer context activation procedure in parallel. The UE and network shall complete the combined default EPS bearer context activation procedure and the attach procedure before the dedicated EPS bearer context activation procedure is completed. If EMM-REGISTERED without PDN connection is not supported by the UE or the MME, then the success of the attach procedure is dependent on the success of the default EPS bearer context activation procedure. If the attach procedure fails, then the ESM procedures also fail.

A UE using EPS services with control plane CIoT EPS optimization can initiate transport of user data via the control plane. For this purpose a UE in EMM-IDLE mode can initiate the service request procedure and transmit the ESM DATA TRANSPORT message in an information element in the CONTROL PLANE SERVICE REQUEST message.

Except for the attach procedure and the service request procedure, during EMM procedures the MME shall suspend the transmission of ESM messages. During the service request procedure the MME may suspend the transmission of ESM messages.

If a S1 mode to S1 mode handover or inter-system handover to S1 mode is ongoing and the TA of the target cell is not included in the TAI list, the MME shall suspend the transmission of ESM messages to the UE until the tracking area updating procedure has been completed.

Except for the attach procedure and the service request procedure for UE initiated transport of user data via the control plane, during EMM procedures the UE shall suspend the transmission of ESM messages.

* * * End of Changes * * * *