3GPP TSG-CT1 Meeting #157	C1-256212

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.186 | CR | 0103 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing editorial changes and correct SIP responses for clarity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Providing editorial changes and correct SIP responses for clarity |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Based on the agreed CR C1-255383 and TS 24.228 Clause AC.10.2.2, if the data channel application is not downloaded, the UE shall download the data channel application …. Change 2: Application data channel application is not a defined term in Clause 3.1. Change 3: Note 2 in clause 9.3.3.1.6.2A and 9.3.3.1.6.3 is corrected for indentation and the text “data channel application” Change 4: SIP response codes 180 (Ringing) and 183 (Session Progress) are corrected in clause 9.3.3.1.6.3, 9.3.3.2.4.1A and 9.3.3.2.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Necessary editorial changes and correct SIP responses will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.1.6.2A, 9.3.3.1.6.3, 9.3.3.2.4.1A, 9.3.3.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

###### 9.3.3.1.6.2A	Adding application data channel during standalone IMS data channel session establishment

Upon receipt of the initial INVITE request with the SDP offer with only the media description for the bootstrap data channel and the UE accepts the bootstrap data channel, the UE shall return a 183 (Session Progress) response to the initial INVITE request with the SDP answer generated as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1].

NOTE 1:	When receiving the incoming INVITE request, the UE can alert the user that there is a standalone data channel establishing based on the UE's implementation for user consent.

Upon receipt of the UPDATE request with the updated SDP offer containing the established bootstrap data channel and a requested application data channel with the associated DC application binding information (provided within the "a=3gpp-req-app" SDP attribute) according to 3GPP TS 26.114 [4] to establish an application data channel, the UE shall determine whether the data channel application has been downloaded or not. If the data channel application:

- has not been downloaded, the UE shall:

1)	download the data channel application;

2)	alert the user and return the 180 (Ringing) once the data channel application is downloaded;

3)	generate the SDP answer that contains the media description for the application data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 200 (OK) response to the UPDATE request; and

4)	return 200 (OK) response to the INVITE request; or

b)	has been downloaded, the UE shall:

1)	alert the user and return the 180 (Ringing);

2)	generate the SDP answer that contains media descriptions for both the bootstrap data channel and application data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 200 (OK) response to the UPDATE request; and

3)		return 200 (OK) response to the INVITE request.

NOTE 2:	Based on the UE's implementation for user consent, the UE can alert the user that data channel application is downloaded.

# *** Next Change ***

###### 9.3.3.1.6.3	Combined standalone bootstrap data channel and application data channel establishment

Upon receipt of the initial INVITE request with the SDP offer with the media description for both the bootstrap data channel and the application data channel associated with the application binding information in "a=3gpp-req-app" line, the UE shall determine whether the data channel application has been downloaded or not. If the data channel application:

a)	has not been downloaded, the UE:

1)	shall generate SDP answer for the data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] accepting the bootstrap data channel and rejecting the application data channel by setting the port number of m=line of application data channel to zero, indicate the data channel application is desired to be downloaded according to 3GPP TS 23.228 [3] clause AC.10.2.3 and return a 183 (Session Progress) response with the generated SDP answer to the initial INVITE request;

NOTE 1:	When receiving the incoming INVITE request, the UE can alert the user that there is a standalone data channel establishing based on the UE's implementation for user consent.

2)	download the data channel application;

3)	 upon receipt of the UPDATE request with the updated SDP offer to establish the corresponding application data channel,

-	alert the user and return the 180 (Ringing) once the data channel application is downloaded;

-	 generate the SDP answer as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 200 (OK) response to the UPDATE request;

-	return 200 (OK) to the INVITE request; or

b)	has been downloaded, the UE shall:

1)	generate the SDP answer that contains both the bootstrap data channel and application data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 183 (Session Progress) response to the network;

2)	alert the user and return the 180 (Ringing) response; and

3)	 return the 200 (OK) response to the INVITE request.

NOTE 2:	Based on the UE's implementation for user consent, the UE can alert the user that data channel application is downloaded.

# *** Next Change ***

###### 9.3.3.2.4.1A	Subsequent standalone bootstrap data channel and application data channel establishment

Upon receipt of the initial INVITE request from the originating network towards a terminating UE with the SDP offer which only includes bootstrap data channels and the 183 (Session Progress) response to the INVITE request with the SDP answer, from the terminating UE, the IMS AS shall follow the procedure in clause 9.3.3.2.1 to handle the bootstrap data channel media.

Upon receipt of the UPDATE request from the originating network with an updated SDP offer that contains a data channel media description for the established bootstrap data channel, as well as the requested application data channel and the associated DC application binding information (provided within the "a=3gpp-req-app" SDP attribute) and the 200 (OK) response to the UPDATE request from the terminating UE, the IMS AS shall follow the procedures upon receipt of the re-INVITE request and the response to the re-INVITE request in clause 9.3.2.2.2.2 to handle the application data channel media.

Upon receipt of the 200 (OK) response to the initial INVITE request from the terminating UE, the IMS AS shall notify the DCSF about the session establishment success event and forward the 200 (OK) response to the originating network.

# *** Next Change ***

###### 9.3.3.2.4.3	Combined standalone bootstrap data channel and application data channel establishment

Upon receipt of the initial INVITE request from the originating network to a terminating UE with SDP offer which includes the media description for both bootstrap data channel and application data channel and the 183 (Session Progress) response to the initial INVITE request from the terminating UE, the IMS AS shall follow the procedure in clause 9.3.3.2.1 to handle the bootstrap data channel media and clause 9.3.2.2.2.2 to handle the and application data channel media.

Upon receipt of the UPDATE request from the originating network with an updated SDP offer that contains a data channel media description for the established bootstrap data channel, as well as the requested application data channel and the associated DC application binding information (provided within the "a=3gpp-req-app" SDP attribute) and the 200 (OK) response to the UPDATE request from the terminating UE, the IMS AS shall follow the procedure in clause 9.3.3.2.2.2 to handle the application data channel media.

Upon receipt of the 200 (OK) response to the initial INVITE request from the terminating UE, the IMS AS shall notify the DCSF about the session establishment success event and forward the 200 (OK) response to the originating network.

# *** End of Changes ***