3GPP TSG-CT WG1 Meeting #157	C1-256313

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Security parameter encoding of AIoT NAS messages

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

The security aspects applicable for AIoT NAS protocol have been captured in 33.369 v1.0.0 such that the encoding for the AIoT NAS messages can be determined. The following key aspects apply:

- The INVENTORY REPORT message is not security protected;

- Integrity protection is mandatory in command messages;

- There is only one integrity protection algorithm specified, and no additional algorithms will ever be added;

- Encryption is optional in command messages;

- There are two encryption algorithms specified, of which one is the null ciphering algorithm;

Following 1) and 2) it can be concluded that both unprotected and security protected messages need to be supported. For the receiver to determine the content and whether the received message is unprotected or security protected, it is proposed that the first octet indicates this property. As for traditional NAS protocols, the leading part of a security protected message contains the parameters needed to security protect the message, which is followed by the specific part for the message, including the Message type IE. The security parameters are proposed to be the security header and the message authentication code.

Following 2) and 3), there is no additional information needed related to integrity protection for security protected messaged, these are always integrity protected and as there is only one algorithm supported there is no need to indicate used algorithm. It also implies that all command messages always need to be security protected messages.

Following 3) and 4), there is a need to indicate whether the message is ciphered or not. Additionally, as are two supported algorithms, it should be possible for the sender to indicate which algorithm was used when ciphering is indicated, i.e. NEA0 or 128-NEA2.

For the specification of unprotected vs security protected messages, it can be considered to follow the 5GS NAS method and define a specific security protected AIoT NAS message for the part related to security, which can include a “plain” AIoT NAS message. The benefit of this method it to avoid repetition of the security part in all messages and to support that any specific message can be sent unprotected or security protected. This second case is however not applicable following 33.369. As the security part is limited, the security header is needed in both unprotected and security protected messages, and the only additional security parameter in security protected messages is the MAC, it is proposed to explicitly include the security part in all message definitions, as applicable.

3. Conclusions

AIoT NAS message encoding can be specified and it is proposed to specify unprotected and security protected AIoT NAS message structures, the needed IEs, and updated message definitions..

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

### 7.1.1	General

Within the AIoT NAS protocol defined in the present document, every message is a standard L3 message as defined in 3GPP TS 24.007 [3]. This means that the message consists of the following parts:

1)	if the message is an unprotected AIoT NAS message:

a)	security header;

b)	message type;

c)	other information elements, as required.

2)	if the message is a security protected AIoT NAS message:

a)	security header;

b)	message authentication code;

c)	message type;

c)	other information elements, as required.

The organization of an unprotected AIoT NAS message is illustrated in the example shown in figure 7.1.1-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security header |  |  |  |  |  |  |  | octet 1 |
| Message type |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 7.1.1-1: General message organization example for an unprotected AIoT NAS message

The INVENTORY REPORT message is an unprotected AIoT NAS message.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security header |  |  |  |  |  |  |  | octet 1 |
| Message authentication code |  |  |  |  |  |  |  | octet 2 octet 5 |
| Message type |  |  |  |  |  |  |  | octet 6 |
|  |  |  |  |  |  |  |  | octet 7 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

Figure 7.1.1-2: General message organization example for a security protected AIoT NAS message

All AIoT NAS messages except the INVENTORY REPORT message are security protected AIoT NAS messages.

Editor's note:	Additional content of AIoT NAS general message organization is FFS and can be added as normative requirements are available.

* * * Next Change * * *

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity. The INVENTORY REPORT message is an unprotected AIoT NAS message.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
|  | AIoT device identity | AIoT device identity 7.2.6 | M | V | n |

* * * Next Change * * *

##### 7.1.3.1	Message definition

The READ COMMAND message is sent by the AIOTF to the AIoT device to read the AIoT data from the memory.

See table 7.1.3.1-1.

Message type:	READ COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.3.1-1: READ COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Read command message identity | Message type 7.2.2 | M | V | 1 |
|  | Read offset | Offset 7.2.9 | M | V | 2 |
|  | Read length | AIoT data length 7.2.5 | M | V | TBD |

* * * Next Change * * *

#### 7.1.4.1	Message definition

The READ COMPLETE message is sent by the AIoT device to the AIOTF to provide the AIoT data that has been read from the memory.

See table 7.1.4.1-1.

Message type:	READ COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.4.1-1: READ COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Read complete message identity | Message type 7.2.2 | M | V | 1 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

* * * Next Change * * *

#### 7.1.5.1	Message definition

The READ COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of reading the AIoT data from the memory.

See table 7.1.5.1-1.

Message type:	READ COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.5.1-1: READ COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Read command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Read cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * *

##### 7.1.6.1	Message definition

The WRITE COMMAND message is sent by the AIOTF to the AIoT device to write the indicated AIoT data to the memory.

See table 7.1.6.1-1.

Message type:	WRITE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.6.1-1: WRITE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Write command message identity | Message type 7.2.2 | M | V | 1 |
|  | Write offset | Offset 7.2.9 | M | V | 2 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

* * * Next Change * * *

#### 7.1.7.1	Message definition

The WRITE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT data has been successfully written to the memory.

See table 7.1.7.1-1.

Message type:	WRITE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.7.1-1: WRITE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Write complete message identity | Message type 7.2.2 | M | V | 1 |

* * * Next Change * * *

#### 7.1.8.1	Message definition

The WRITE COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of writing the AIoT data from the memory.

See table 7.1.8.1-1.

Message type:	WRITE COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.8.1-1: WRITE COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Write command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Write cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * *

##### 7.1.9.1	Message definition

The PERMANENT DISABLE COMMAND message is sent by the AIOTF to the AIoT device to permanently disable the communication capability of AIoT device.

See table 7.1.9.1-1.

Message type:	PERMANENT DISABLE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.9.1-1: PERMANENT DISABLE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Permanent disable command message identity | Message type 7.2.2 | M | V | 1 |

* * * Next Change * * *

#### 7.1.10.1	Message definition

The PERMANENT DISABLE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT device communication capability has been disabled.

See table 7.1.10.1-1.

Message type:	PERMANENT DISABLE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.10.1-1: PERMANENT DISABLE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.x1 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.x2 | M | V | 4 |
|  | Permanent disable complete message identity | Message type 7.2.2 | M | V | 1 |

* * * Next Change * * *

### 7.2.x1	Security header

The purpose of the security header information element is to indicate security related properties for the message in which the information element is included as first octet.

The security header information element is coded as shown in figure 7.2.x1-1 and table 7.2.x1-1.

The cause is a type 3 information element with length of 2 octets.

| 8 | 7 | 6 | 5 |  | 4 | 3 |  | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Security header IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| Spare |  |  |  | CA |  |  | SPI |  |  | octet 2 |

Figure 7.2.x1-1: Cause information element

Table 7.2.x1-1: Cause information element

| Security protection indication (SPI) (octet 2, bit 1 to 3) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |
| 3 | 2 | 1 |  |  |  |  |
| 0 | 0 | 0 |  |  | Unprotected AIoT NAS message |  |
|  |  |  |  |  | Security protected AIoT NAS message: |  |
| 0 | 0 | 1 |  |  | Integrity protected |  |
| 0 | 1 | 0 |  |  | Integrity protected and ciphered |  |
| All other values are reserved. |  |  |  |  |  |  |
| Ciphering algorithm (CA) (octet, bit 4) The CA indicates which algorithm was used at ciphering of the AIoT NAS message. This indication is ignored unless the SPI field indicates "Integrity protected and ciphered" Bit |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 0 |  |  |  | NEA0 |  |
| 1 |  |  |  | 128-NEA2 |  |

* * * Next Change * * *

### 7.2.x2	Message authentication code

The purpose of the Message authentication code information element is to protect the integrity of an AIoT NAS message.

The Message authentication code is a type 3 information element with 6 octets length.

The Message authentication code information element is coded as shown in figure 7.2.x2-1 and table 7.2.x2-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message authentication code IEI |  |  |  |  |  |  |  | octet 1 |
| Message authentication code value |  |  |  |  |  |  |  | octet 2 octet 5 |

Figure 7.2.x2-1: Message authentication code information element

Table 7.2.x2-1: Message authentication code information element

Message authentication code value (octet 2 to 5) This field contains the 32 bit message authentication code calculated for AIoT integrity protection. Bit 1 of octet 2 contains the most significant bit, and bit 8 of octet 5 the least significant bit of these 4 octets.

* * * End of Changes * * *