3GPP TSG-CT WG1 Meeting #157	C1-256314

Sophia Antipolis, France, 13-17 October 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on deletion of Primary authentication and key agreement procedure clause

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

Work has started in CT1 on AmbientIoT-CT and a new TS for AIoT NAS between AIoT device and AIOTF is to be developed.

2. Reason for Change

In 33.369 the authentication is specified in clause 5.2.2 “Authentication procedure”. However, the specification of this procedure is achieved by authentication related enhancements of the Inventory and Command procedures in 23.369. This is captured in 33.369:

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Thus, from an AIoT NAS protocol perspective, there is no separate authentication procedure and the AIoT NAS procedures for Inventory and Command will cover also authentication aspects.

It is proposed to delete the current clause 4.3, Primary authentication and key agreement procedure, keep the general parts for authentication in clause 4.2.2, Authentication, and capture and procedure parts of authentication in clause 5.2, Inventory procedure, and clause 5.3, Command procedures, respectively. Additionally, it is proposed to add a note for clarification of the mapping of the Authentication procedure in 33.369 to the Inventory and Command procedures.

3. Conclusions

Following SA3 progress and 33.369 v1.0.0, it can be concluded that there is no explicit authentication of the network by the AIoT devive in the Inventory procedure, and the abnormal case in 24.369 can be deleted.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0

* * * First Change * * *

### 4.2.2	Authentication

Authentication of the AIoT device is done in the inventory procedure. A network generated RANDAIOT_n is provided by the AIOTF and received by the AIoT device in the paging request. The AIoT device which is target of the paging request (see clause 5.2.3), uses the RANDAIOT_n and its long-term KAIOT to derive a RESAIOT and includes the RESAIOT and a RANDAIOT_d in the INVENTORY REPORT message to the AIOTF. The network verifies the RESAIOT to authenticate the AIoT device.

The requirements for the AIoT device to derive RESAIOT at receiving RANDAIOT_n from lower layers are described in 3GPP TS 33.369 [6].

The requirements for parameters and algorithms used at AIoT authentication are specified in 3GPP TS 33.369 [6].

NOTE:	The procedure parts for authentication as specified in 3GPP TS 33.369, clause 5.2.2, are specified in clauses 5.2, Inventory procedure, and clause 5.3, Command procedures.

Editor's note:	Details on authentication will be aligned with stage 2 normative requirements when available.

* * * Next Change * * *

* * * End of Changes * * *