3GPP TSG-CT WG1 Meeting #156	C1-256332

# Sophia-Antipolis, France 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.193 | CR | 0232 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of UE requested MA PDU session with invalid capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For deterministic network response to a case when at a UE request related to an MA PDU session which includes invalid UE ATSSS capabilities, it is proposed to add a new case in missing information elements that specifies which cause value to include in the reject. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change of note to normative words and added clarifying text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Normative text for MME is missing and reference to table C.1 of TS 29.274 ia missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3b.1, 5.3b.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************** First Change *******************************

### 5.3b.1	General

Activating a multi-access PDU (MA PDU) connectivity service may fail due to the UE and the network having incompatible supports for ATSSS steering functionalities during session setup. This clause specifies the necessary actions for both the UE and the network when this problem occurs.

****************************** Next Change *******************************

#### 5.3b.2.1	Network procedures

Activating the MA PDU connectivity service may fail when the UE attempts to establish PDU session over 5GS as described in clause 5.2.1, if the SMF receives the PDU SESSION ESTABLISHMENT REQUEST message with the 5GSM capability IE with the ATSSS-ST field set to "ATSSS not supported", the ATSSS-LL field set to "ATSSS-LL functionality not supported" and the MPQUIC-IP bit set to"MPQUIC-IP functionality with any steering mode supported" from the UE, and the SMF determines that the "MPQUIC-IP functionality" is not allowed based on the DNN configuration, then the SMF shall send a PDU SESSION ESTABLISHMENT REJECT message with the 5GSM cause set to #31 "request rejected, unspecified" as specified in clause 6.4.1.4.1 of 3GPP TS 24.501 [6] to the UE. In addition, the SMF shall include the ATSSS container IE containing the ATSSS status set to"MPQUIC-IP functionality not allowed" (see table 6.1.5A.2-1) in the PDU SESSION ESTABLISHMENT REJECT message.

Activating the MA PDU connectivity service may fail when the UE attempts to establish PDN connection over:

a)	E-UTRAN connected to EPC as a user-plane resource of an MA PDU session to be established as described in clause 5.3.1; or

b)	untrusted non-3GPP access network connected to EPC as a user-plane resource of an MA PDU session to be established as described in clause 5.3a.2,

if the PGW-C+SMF receives Create Session Request message containing the Extended protocol configuration options IE with the ATSSS request PCO parameter indicating "ATSSS not supported", "ATSSS-LL functionality not supported" and "MPQUIC-IP functionality with any steering mode supported" from the UE, and the PGW-C+SMF determines that the "MPQUIC-IP functionality" is not allowed based on the DNN configuration, then the PGW-C+SMF shall reject by constructing the Create Session Response message with a reject cause specified in 3GPP TS 29.274 [18], and include the ATSSS status as the ATSSS parameter set to"MPQUIC-IP functionality not allowed" (see table 6.1.5A.2-1) in the ATSSS response with the length of two octets PCO parameter container contents in the Extended protocol configuration options IE.

In case of the procedure a) above, if the MME receives the Create Session Response message with a reject cause value including the Extended protocol configuration options IE from the SGW as specified in 3GPP TS 29.274 [18], the MME shall include the received Extended protocol configuration options IE in the PDN CONNECTIVITY REJECT message with an appropriate error cause and send to the UE as specified in 3GPP TS 24.301 [4]. The MME determines which ESM cause value is included in the PDN CONNECTIVITY REJECT message based on the reject cause value received in the Create Session Response message and the mapping of the rejection cause values listed in table C.1 in 3GPP TS 29.274 [18].

In case of the procedure b) above, if the ePDG receives the Create Session Response message with a reject cause value including the Extended protocol configuration options IE from the PGW-C+SMF as specified in 3GPP TS 29.274 [18], the ePDG shall construct the ATSSS_RESPONSE Notify payload as specified in 3GPP TS 24.302 [17], copy the ATSSS response with the length of two octets PCO parameter container contents into the ATSSS response information field of the ATSSS_RESPONSE Notify payload, and include the ATSSS_RESPONSE Notify payload in the IKE_AUTH response message. The ePDG shall also include a Notify payload with a private notify message to the UE, indicating an error in the IKE_AUTH response message.

NOTE:	It is up to the implementation of the ePDG, which private notify message error type value is included in the IKE_AUTH response message based on the reject cause value received in the Create Session Response message as specified in 3GPP TS 29.274 [18].

****************************** End of Change *******************************