3GPP TSG-CT WG1 Meeting #157	C1-256373

Sophia Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Pseudo-CR on AIoT encoding of security information

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

SA3 agreed on the security aspects of integrity and ciphering of AIoT NAS messages.

2. Reason for Change

TS 33.369 agreements w.r.t security procedures were agreed. Following requirements need to be captured in CT1 specification as well.

4.2.1.3	Requirements for command protection

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

5.3.2	Security procedure on information protection during command procedure

The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN.

3. Conclusions

Following considerations & conclusions based on above SA3 agreements were made while proposing changes in this pCR:

a. Only 1 type of integrity protection algorithm to be used, mostly pre-configured, so no negotiation or indication of the integrity algorithm is required.

b. Negotiation of ciphering algorithm required. Existing encoding scheme used for support of more/ new supported algorithms.

c. Encoding scheme needs to be aligned with current encoding scheme of plain NAS messages as in 24.301 and 24.501. No MAC-I inclusion aspect required to be considered, while considering encoding of plain AIoT NAS message. Security header type to only indicate whether message is plain/ integrity protected/ integrity protected and ciphered. Read / Write command reject messages can be plain messages, if there is failure in integrity check of R2D messages.

d. For D2R messages, selected ciphering algorithm does not need to be replayed, as this is indicated by NW in the R2D message and the device uses the same algorithm.

e. There are no identified security related parameters as per SA3, in the inventory report message from the device.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

### 7.1.2	Inventory report

##### 7.1.2.1	Message definition

The INVENTORY REPORT message is sent by the AIoT device to the AIOTF to report the AIoT device identity.

See table 7.1.2.1-1.

Message type:	INVENTORY REPORT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | RANDAIOT_d | RAND 7.2.10 | M | V | TBD |
|  | RESAIOT | Authentication response parameter 7.2.7 | M | V | TBD |
|  | AIoT device identity | AIoT device identity 7.2.6 | M | V | n |

* * * Next Change * * * *

### 7.1.3	Read command

##### 7.1.3.1	Message definition

The READ COMMAND message is sent by the AIOTF to the AIoT device to read the AIoT data from the memory.

See table 7.1.3.1-1.

Message type:	READ COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.3.1-1: READ COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Ciphering algorithm type | Ciphering algorithm type 7.x.2 | M | V | 1/2 |
|  | Read command message identity | Message type 7.2.2 | M | V | 1 |
|  | Read offset | Offset 7.2.9 | M | V | 2 |
|  | Read length | AIoT data length 7.2.5 | M | V | TBD |

* * * Next Change * * * *

### 7.1.4	Read complete

#### 7.1.4.1	Message definition

The READ COMPLETE message is sent by the AIoT device to the AIOTF to provide the AIoT data that has been read from the memory.

See table 7.1.4.1-1.

Message type:	READ COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.4.1-1: READ COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 7.x.1 | M | V | 1/2 |
|  | Read complete message identity | Message type 7.2.2 | M | V | 1 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

* * * Next Change * * * *

### 7.1.5	Read command reject

#### 7.1.5.1	Message definition

The READ COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of reading the AIoT data from the memory.

See table 7.1.5.1-1.

Message type:	READ COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.5.1-1: READ COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 7.x.1 | M | V | 1/2 |
|  | Read command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Read cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * * *

### 7.1.6	Write command

##### 7.1.6.1	Message definition

The WRITE COMMAND message is sent by the AIOTF to the AIoT device to write the indicated AIoT data to the memory.

See table 7.1.6.1-1.

Message type:	WRITE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.6.1-1: WRITE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Ciphering algorithm type | Ciphering algorithm type 7.x.2 | M | V | 1/2 |
|  | Write command message identity | Message type 7.2.2 | M | V | 1 |
|  | Write offset | Offset 7.2.9 | M | V | 2 |
|  | AIoT data | AIoT data 7.2.4 | M | TBD | TBD |

* * * Next Change * * * *

### 7.1.7	Write complete

#### 7.1.7.1	Message definition

The WRITE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT data has been successfully written to the memory.

See table 7.1.7.1-1.

Message type:	WRITE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.7.1-1: WRITE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 7.x.1 | M | V | 1/2 |
|  | Write complete message identity | Message type 7.2.2 | M | V | 1 |

* * * Next Change * * * *

### 7.1.8	Write command reject

#### 7.1.8.1	Message definition

The WRITE COMMAND REJECT message is sent by the AIoT device to the AIOTF to indicate the cause of the failure of writing the AIoT data from the memory.

See table 7.1.8.1-1.

Message type:	WRITE COMMAND REJECT

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.8.1-1: WRITE COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 7.x.1 | M | V | 1/2 |
|  | Write command reject message identity | Message type 7.2.2 | M | V | 1 |
|  | Write cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * * *

### 7.1.9	Permanent disable command

##### 7.1.9.1	Message definition

The PERMANENT DISABLE COMMAND message is sent by the AIOTF to the AIoT device to permanently disable the communication capability of AIoT device.

See table 7.1.9.1-1.

Message type:	PERMANENT DISABLE COMMAND

Significance:	dual

Direction:	AIOTF to AIoT device

Table 7.1.9.1-1: PERMANENT DISABLE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Ciphering algorithm type | Ciphering algorithm type 7.x.2 | M | V | 1/2 |
|  | Permanent disable command message identity | Message type 7.2.2 | M | V | 1 |

* * * Next Change * * * *

### 7.1.10	Permanent disable complete

#### 7.1.10.1	Message definition

The PERMANENT DISABLE COMPLETE message is sent by the AIoT device to the AIOTF to indicate that the AIoT device communication capability has been disabled.

See table 7.1.10.1-1.

Message type:	PERMANENT DISABLE COMPLETE

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.10.1-1: PERMANENT DISABLE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header type | Security header type 7.2.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 7.x.1 | M | V | 1/2 |
|  | Permanent disable complete message identity | Message type 7.2.2 | M | V | 1 |

* * * Next Change * * * *

### 7.2.3	Security header type

Bits 1 to 4 of the first octet of every AIoT NAS message contain the Security header type IE. This IE includes control information related to the security protection of a NAS message. The total size of the Security header type IE is 4 bits.

The Security header type IE can take the values shown in table 7.2.3.1.

Table 7.2.3.1: Security header type

| Security header type (octet 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | Plain NAS message, not security protected |
|  |  |  |  | Security protected AIoT NAS message: |
| 0 | 0 | 0 | 1 | Integrity protected |
| 0 | 0 | 1 | 0 | Integrity protected and ciphered |
| 1 | 1 | 1 | 1 |  |
| All other values are reserved. |  |  |  |  |

* * * Next Change * * * *

### 7.x	Other information elements

#### 7.x.1	Spare half octet

This element is used in the description of AIoT NAS messages when an odd number of half octet type 1 information elements are used. This element is filled with spare bits set to zero and is placed in bits 5 to 8 of the octet unless otherwise specified.

* * * Next Change * * * *

#### 7.x.2	Ciphering algorithm

The purpose of the Ciphering algorithm information element is to indicate the algorithm to be used for ciphering of the AIoT NAS message.

The NAS security algorithms information element is coded as shown in figure 7.x.2.1 and table 7.x.2.1.

The NAS security algorithms is a type 1 information element with a length of 1 octet.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ciphering algorithm IEI |  |  |  | 0 spare | Type of ciphering algorithm |  |  |

Figure 7.x.2.1: Additional update result information element

Table 7.x.2.1: Additional update result information element

| Type of ciphering algorithm (octet 1, bit 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 |  | EPS encryption algorithm EEA0 (null ciphering algorithm) |
| 0 | 0 | 1 |  | EPS encryption algorithm 128-EEA1 |
| 0 | 1 | 0 |  | EPS encryption algorithm 128-EEA2 |
| 0 | 1 | 1 |  | EPS encryption algorithm 128-EEA3 |
| 1 | 0 | 0 |  | EPS encryption algorithm EEA4 |
| 1 | 0 | 1 |  | EPS encryption algorithm EEA5 |
| 1 | 1 | 0 |  | EPS encryption algorithm EEA6 |
| 1 | 1 | 1 |  | EPS encryption algorithm EEA7 |
| Bit 4 is spare and shall be coded as zero. |  |  |  |  |

* * * End of Changes * * * *