3GPP TSG-CT WG1 Meeting #157	C1-256378

Sophia Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Pseudo-CR on AIoT security procedures update

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

This pseudo-CR updates the general sections for integrity protection and ciphering of AIoT NAS messages based on SA3 agreements.

2. Reason for Change

SA3 has agreed on the integrity protection and ciphering of AIoT NAS messages. Additionally, SA3 has clarified exactly which part of the AIoT NAS message needs to be ciphered, when NULL-ciphering algorithm is not used for protecting AIoT NAS messages.

The corresponding updates of AIoT NAS procedures need to be made in CT1 specification.

NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text.

3. Conclusions

Adding: a) Integrity protection clause for AIoT NAS message; and 2) Further information related to ciphering of AIoT NAS messages.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

### 4.2.4	Integrity protection of AIoT NAS signalling messages

The AIoT device and the AIOTF shall support integrity protection of AIoT NAS messages. Except the INVENTORY REPORT message the integrity protection of AIoT NAS messages is mandatory in the AIoT device and the AIOTF. The complete AIoT NAS message is integrity protected.

The AIoT device shall support required integrity algorithms as described in clause 4.2.1.3 of 3GPP TS 33.369 [6]. The derivation of the integrity keys required are described in clause A.4 of 3GPP TS 33.369 [6].

* * * Next Change * * * *

### 4.2.5	Ciphering of AIoT NAS signalling messages

The use of ciphering in a network is an operator option subject to AIOTF configuration. When operation of the network without ciphering is configured, the AIOTF shall indicate the use of "null ciphering algorithm" NEA0.

All AIoT NAS messages, except the INVENTORY REPORT message, shall be sent ciphered. If the "null ciphering algorithm" NEA0 has been selected as a ciphering algorithm, the AIoT NAS message is regarded as ciphered. If the "null ciphering algorithm" NEA0 has not been selected as a ciphering algorithm, the AIoT NAS message is partly ciphered, with the selected ciphering algorithm included as a clear text information.

The algorithm to calculate the ciphering of AIoT NAS messages is described in annex D.2 in 3GPP TS 33.501 [9]. In addition to the data that is to be ciphered, the constant BEARER, DIRECTION bit, COUNT, KEY and LENGTH are input to the ciphering algorithm. These parameters are described in 3GPP TS 33.369 [6].

* * * End of Changes * * * *