3GPP TSG-CT WG1 Meeting #157	C1-256401

# Sophia Antipolis, France, 13-17 October 2025

Source:	Ericsson

Title:	Pseudo-CR on AIoT identification information

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

2. Reason for Change

As discussed in C1-256400, the AIoT identification information can be:

1. Filtering Information for group paging;

2. One T-ID for individual paging when privacy protection is used;

3. One AIoT device permanent identifier for individual paging when privacy protection is not used; or

4. All AIoT devices.

And such AIoT identification information type information is needed for the AIoT device NAS to perform AIoT device identifier matching, it is proposed to add the handling for the AIoT device to perform matching based on the AIoT identification information type, and add the encoding for the AIoT identification information in 24.369.

And as agreed in the C4-253570 to 23.003, the Structure of Filtering Information contains a list of the filtering elements, rather than one filtering element, so the group paging matching shall be updated accordingly.

3. Conclusions

It is proposed to add the handling for the AIoT device to perform matching based on the received AIoT identification information type, and add the encoding for the AIoT identification information in 24.369.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369.

***** change *****

### 5.2.3	Inventory procedure completion

The AIoT device determines to respond to the AIoT paging:

a)	based on the indication from lower layer as specified in 3GPP TS 38.391 [8]; or

b)	based on whether the AIoT identification information as specified in clause 7.2.xx received from the lower layer is matched or not.

If the AIoT device determines to respond to the AIoT paging, the AIoT device shall send an INVENTORY REPORT message to the AIOTF.

The AIoT device shall derive a RANDAIOT_d value as specified in 3GPP TS 33.369 [6] and shall include the derived RANDAIOT_d  value in the in the RANDAIOT_d IE in the INVENTORY REPORT message.

The AIoT device shall derive a RESAIOT value, using the long-term configured KAIoT and the value of the RANDAIOT_n included in the paging message provided by lower layers as specified in 3GPP TS 33.369 [6]. The derived RESAIOT value shall be included in the in the RESAIOT IE in the INVENTORY REPORT message.

The AIoT device shall include the AIoT device identity IE in the INVENTORY REPORT message.

Editor's note:	Details of the AIoT device identity IE is FFS and depends on SA3 progress.

Based on the received AIoT identification information, the AIoT device determines whether the AIoT identification information is matched or not as follows:

a)	if the paging type is set to "individual paging", and the single device identity type is set to "AIoT device permanent identifier":

1)	if the privacy protection configured in the AIoT device indicates that privacy protection is used, then the AIoT device determines the AIoT identification information is not matched; or

2)	if the privacy protection configured in the AIoT device indicates that privacy protection is not used:

A)	if the AIoT device permanent identifier stored in the AIoT device is the same as the AIoT device permanent identifier in the AIoT identification information, the AIoT device determines that the AIoT identification information is matched; or

B) 	otherwise, the AIoT device determines the AIoT identification information is not matched;

b)	if the paging type is set to "individual paging", and the single device identity type is set to "T-ID":

1)	if the privacy protection configured in the AIoT device indicates that privacy protection is not used, then the AIoT device determines the AIoT identification information is not matched;

2)	if the privacy protection configured in the AIoT device indicates that privacy protection is used:

A) 	if the T-ID type is set to "concealed type based on permanent identifier", the AIoT device shall derive T-ID from the AIoT device permanent identifier stored in the AIoT device as specified in annex B.1 in 3GPP TS 33.369 [6], and:

i)	if the derived T-ID is the same as the T-ID in the AIoT identification information, the AIoT device determines that the AIoT identification information is matched;

ii) 	otherwise, the AIoT device determines the AIoT identification information is not matched;

B)	if the T-ID type is set to "concealed type based on stored T-ID", the AIoT device shall derive a new T-ID from the stored T-ID as specified in annex B.1 in 3GPP TS 33.369 [6], and:

i)	if the derived T-ID is the same as the T-ID in the AIoT identification information, the AIoT device determines that the AIoT identification information is matched;

ii) 	otherwise, the AIoT device determines the AIoT identification information is not matched; or

C)	if the T-ID type is set to "stored type":

i)	if the stored T-ID is the same as the T-ID in the AIoT identification information, the AIoT device determines that the AIoT identification information is matched;

ii) 	otherwise, the AIoT device determines the AIoT identification information is not matched;

c)	if the paging type is set to "group paging", then the AIoT device identity information contains a list of the filtering elements, for each filtering element:

1)	based on ID type field included in the filtering element, the AIoT device determines that the filtering element is matched if:

A)	the PLMN ID field is not included in the filtering element, or the PLMN ID field is included in the filtering element and:

i)	the PLMN ID field is included in the AIoT device permanent identifier; and

ii)	the PLMN ID field in the AIoT device permanent identifier is the same as the PLMN ID field within the filtering element;

B)	the NID field is not included in the filtering element, or the NID field is included in the filtering element and:

i)	the NID field is included in the AIoT device permanent identifier; and

ii)	the NID field in the AIoT device permanent identifier is the same as the NID field within the filtering element;

C)	the third party ID field is not included in the filtering element, or the third party ID field is included in the filtering element and:

i)	the third party ID field is included in the AIoT device permanent identifier; and

ii)	the third party ID field in the AIoT device permanent identifier is the same as the third party ID field within the filtering element; and

D)	the identification information filter field is not included in the filtering element, or the identification information filter field is included in the filtering element, the component value within the identification information filter is the same as the corresponding bitstring from the AIoT device permanent identifier based on offset field and length field; or

2)	otherwise, the AIoT device determines the AIoT identification element is not matched;

If all the AIoT identification elements are matched, then the AIoT device determines the AIoT identification information is matched; otherwise if any of the AIoT identification elements is not matched, the AIoT device determines the AIoT identification information is not matched; or

d)	if the paging type is set to "paging all", the AIoT device determines that the AIoT identification information is matched.

NOTE 1:	The AIoT identification information corresponds to paging ID, as defined in 3GPP TS 38.391 [8]).

Based on whether the AIoT identification information is matched:

a)	if the AIoT identification information is matched, the AIoT device shall indicate to the lower layer that the AIoT identification information is matched, and determine to respond to the AIoT paging; or

b)	otherwise, the AIoT device shall ignore the AIoT identification information, indicate to the lower layer that the filtering information is not matched, and determine not to respond to the AIoT paging.

Upon all the selected NG-RAN indicating the inventory procedure has been completed, the inventory procedure is considered as completed by the AIOTF as specified in 3GPP TS 23.369 [2].

NOTE:	How NG-RAN indicates the inventory procedure has been completed is defined in 3GPP TS 38.300 [7].

Editor's note:	The reference to TS 38.300 may be updated according to RAN2 progress.

Editor's note:	AIoT .device authentication of the network is FFS.

***** change *****

### 7.2.xx	AIoT identification information

The purpose of the AIoT identification information information element is to provide the AIoT identification information to the AIoT device to perform the AIoT device identifier matching in paging.

The AIoT identification information information element is coded as shown in figure 7.2.xx-1, figure 7.2.xx-2, figure 7.2.xx-3, figure 7.2.xx-4, figure 7.2.xx-5 and table 7.2.xx-1.

The AIoT identification information is a non-standard layer 3 information element with a minimum length of 18 octets and maximum length of 49 octets.

| 8 | 7 | 6 | 5 | 4 |  | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | T-ID type |  |  | Single device identity type | Paging type |  |  | octet 1 |
| AIoT device identity information |  |  |  |  |  |  |  |  | octet 2 octet m |

Figure 7.2.xx-1: AIoT identification information

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RANDAIOT_n |  |  |  |  |  |  |  | octet 2 octet 17 |

Figure 7.2.xx-2: AIoT device identity information for paging type = "paging all"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RANDAIOT_n |  |  |  |  |  |  |  | octet 2 octet 17 |
| Filtering information elements |  |  |  |  |  |  |  | octet 18 octet m(see NOTE) |

NOTE:	The maximum value of m is 49.

Figure 7.2.xx-3: AIoT device identity information for paging type = "group paging"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RANDAIOT_n |  |  |  |  |  |  |  | octet 2 octet 17 |
| Length of AIoT device identity information contents |  |  |  |  |  |  |  | octet 18 |
| AIoT device permanent identifier |  |  |  |  |  |  |  | octet 19 octet m(see NOTE) |

NOTE:	The maximum value of m is 47.

Figure 7.2.xx-4: AIoT device identity information for paging type = "individual paging", and single device identity type = "AIoT device permanent identifier"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RANDAIOT_n |  |  |  |  |  |  |  | octet 2 octet 17 |
| T-ID |  |  |  |  |  |  |  | octet 18 octet 49 |

Figure 7.2.xx-5: AIoT device identity information for paging type = "individual paging", and single device identity type = "T-ID"

Table 7.2.xx-1: AIoT identification information

| Paging type (octet 1 bits 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 3 | 2 |  | 1 |  |
| 0 | 0 |  | 0 | Paging all |
| 0 | 0 |  | 1 | Group paging |
| 0 | 1 |  | 0 | Individual paging |
| The other values are spare and not used. The AIoT device shall ignore the paging if other values are received. When the paging type is set to "paging all" or "group paging", the single device identity type field, the T-ID type field shall be ignored. When the paging type is set to "individual paging", the single device identity type field, the T-ID type field shall be set. |  |  |  |  |
| Single device identity type (octet 1 bit 4) |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | AIoT device permanent identifier |
| 1 |  |  |  | T-ID |
| T-ID type (octet 1 bits 5 to 6) |  |  |  |  |
| Bit |  |  |  |  |
| 6 |  | 5 |  |  |
| 0 |  | 0 |  | Concealed type based on permanent identifier |
| 0 |  | 1 |  | Concealed type based on stored T-ID |
| 1 |  | 0 |  | Stored type |
| The other values are spare and not used. The AIoT device shall ignore the paging if other values are received. |  |  |  |  |
| RANDAIOT_n (octet 2 to octet 17) |  |  |  |  |
| This field is provided to the AIoT device for the security calculation for authentication as specified in 3GPP TS 33.369 [6]. |  |  |  |  |
| AIoT device permanent identifier (octets 19 to m) |  |  |  |  |
| This field is encoded as the structure of AIoT device permanent identifier as specified in clause 31.2 in 3GPP TS 23.003 [5]. When the ID Type is set to NID part is included, 0000 are padded in the bits 5 to 8 of the last octet of the NID field. |  |  |  |  |
| T-ID (octets 18 to 49) |  |  |  |  |
| This field contains the binary encoded T-ID value as specified in clause 31.3 in 3GPP TS 33.369 [6]. |  |  |  |  |
| Filtering information elements (octets 18 to m) |  |  |  |  |
| This field is encoded as the structure of filtering information as specified in clause 31.3 in 3GPP TS 23.003 [5]. |  |  |  |  |