3GPP TSG-CT WG1 Meeting #158	C1-257070

Dallas, US, 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.281 | CR | 0293 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of functional alias determination in MCVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Firstnet, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2025-10-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedure to subscribe to functional alias statuses in MCVideo does not work. The procedure use a request-type element set to a value of “functional-alias-status-determination”, however the request-type is an enumration that has not defined “functional-alias-status-determination”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The issue is corrected by adding a new req-type element that can be extended with new types define as string, boolean or URI’s. In this case a new string “functional-alias-status-determination” is defined. Backward compatible The CR is backward compatible. The feature functional alias status determination will by this CR work. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature functional alias status determination will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 20.2.1.3, 20.2.2.2.4, F.1.2, F.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 20.2.1.3	Functional alias status determination procedure

NOTE 1:	The MCVideo UE also uses this procedure to determine which functional alias have been successfully activated for the MCVideo ID.

In order to discover functional aliases:

1)	which are activated for the MCVideo user; or

2)	which another MCVideo user has activated;

the MCVideo client shall generate an initial SIP SUBSCRIBE request according to TS 24.229 [11], IETF RFC 3856 [13], and IETF RFC 6665 [16].

In the SIP SUBSCRIBE request, the MCVideo client:

1)	shall set the Request-URI to the public service identity identifying the originating participating MCVideo function serving the MCVideo user;

2)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body. In the application/vnd.3gpp.mcvideo-info+xml MIME body, the MCVideo client shall include:

a)	the <mcvideo-request-uri> element set to the MCVideo ID of the targeted MCVideo user; and

b)	the <req-type> element in the <anyExt> element of the <mcvideo-Params> element of the <mcvideoinfo> element set to the value "functional-alias-status-determination";

3)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14];

4)	if the MCVideo client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [16], to 4294967295;

NOTE 2:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [15].

5)	if the MCVideo client wants to fetch the current state only, shall set the Expires header field according to IETF RFC 6665 [16], to zero;

6)	shall include an Events header field set to "presence"; and

7)	shall include an Accept header field containing the application/pidf+xml MIME type.

In order to re-subscribe or de-subscribe, the MCVideo client shall generate an in-dialog SIP SUBSCRIBE request according to TS 24.229 [11], IETF RFC 3856 [13], and IETF RFC 6665 [16]. In the SIP SUBSCRIBE request, the MCVideo client:

1)	if the MCVideo client wants to receive the current status and later notification, shall set the Expires header field according to IETF RFC 6665 [16], to 4294967295;

NOTE 3:	4294967295, which is equal to 232-1, is the highest value defined for Expires header field in IETF RFC 3261 [15].

2)	if the MCVideo client wants to de-subscribe, shall set the Expires header field according to IETF RFC 6665 [16], to zero;

3)	shall include an Events header field set to "presence"; and

4)	shall include an Accept header field containing the application/pidf+xml MIME type.

Upon receiving a SIP NOTIFY request according to TS 24.229 [11], IETF RFC 3856 [13], and IETF RFC 6665 [16], if SIP NOTIFY request contains an application/pidf+xml MIME body indicating per-user functional alias information  constructed according to clause 20.3.1, then the MCVideo client shall determine the status of the MCVideo user for each functional alias in the MIME body. If the <p-id-fa> child element of the <presence> root element of the application/pidf+xml MIME body of the SIP NOTIFY request is included, the <p-id-fa> element value indicates the SIP PUBLISH request which triggered sending of the SIP NOTIFY request.

If the MCVideo client detected a functional alias activation or deactivation, it shall perform the procedure specified in clause 8.2.1.7.

* * * Next Change * * *

##### 20.2.2.2.4	Receiving subscription to functional alias status procedure

Upon receiving a SIP SUBSCRIBE request such that:

1)	Request-URI of the SIP SUBSCRIBE request contains either the public service identity identifying the originating participating MCVideo function serving the MCVideo user, or the public service identity identifying the terminating participating MCVideo function serving the MCVideo user;

2)	the SIP SUBSCRIBE request contains an application/vnd.3gpp.mcvideo-info+xml MIME body containing:

a)	the<mcvideo-request-uri> element which identifies an MCVideo ID served by the MCVideo server; and

b)	the <req-type> element in the <anyExt> element of the <mcvideo-Params> element of the <mcvideoinfo> element set to the value "functional-alias-status-determination";

3)	the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in TS 24.229 [11]), in a P-Asserted-Service header field according to IETF RFC 6050 [14]; and

4)	the Event header field of the SIP SUBSCRIBE request contains the "presence" event type;

the MCVideo server:

1)	shall identify the served MCVideo ID in the <mcvideo-request-uri> element of the application/vnd.3gpp.mcvideo-info+xml MIME body of the SIP SUBSCRIBE request;

2)	if the Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the originating participating MCVideo function serving the MCVideo user, shall identify the originating MCVideo ID from public user identity in the P-Asserted-Identity header field of the SIP SUBSCRIBE request;

3)	if the Request-URI of the SIP SUBSCRIBE request contains the public service identity identifying the terminating participating MCVideo function serving the MCVideo user, shall identify the originating MCVideo ID in the <mcvideo-calling-user-id> element of the application/vnd.3gpp.mcvideo-info+xml MIME body of the SIP SUBSCRIBE request;

4)	if the originating MCVideo ID is different than the served MCVideo ID and the originating MCVideo ID is not authorized to modify functional alias status of the served MCVideo ID, shall send a SIP 403 (Forbidden) response and shall not continue with the rest of the steps; and

5)	shall generate a SIP 200 (OK) response to the SIP SUBSCRIBE request according to TS 24.229 [11], IETF RFC 6665 [16].

For the duration of the subscription, the MCVideo server shall notify the subscriber about changes of the information of the served MCVideo ID, as described in clause 20.2.2.2.5.

* * * Next Change * * *

## F.1.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:mcvideoInfo:1.0"

xmlns:mcvideoinfo="urn:3gpp:ns:mcvideoInfo:1.0"

elementFormDefault="qualified"

attributeFormDefault="unqualified"

xmlns:xenc="http://www.w3.org/2001/04/xmlenc#"

xmlns:mvgktp="urn:3gpp:ns:mcvideoGKTP:1.0">

<xs:import namespace="http://www.w3.org/2001/04/xmlenc#"/>

<xs:import namespace="urn:3gpp:ns:mcvideoGKTP:1.0"/>

<!-- root XML element -->

<xs:element name="mcvideoinfo" type="mcvideoinfo:mcvideoinfo-Type" id="info"/>

<xs:complexType name="mcvideoinfo-Type">

<xs:sequence>

<xs:element name="mcvideo-Params" type="mcvideoinfo:mcvideo-ParamsType"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcvideoinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mcvideo-ParamsType">

<xs:sequence>

<xs:element name="mcvideo-access-token" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="session-type" type="xs:string" minOccurs="0"/>

<xs:element name="mcvideo-request-uri" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="mcvideo-calling-user-id" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="mcvideo-called-party-id" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="mcvideo-calling-group-id" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="required" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="emergency-ind" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="alert-ind" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="imminentperil-ind" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="broadcast-ind" type="xs:boolean" minOccurs="0"/>

<xs:element name="mc-org" type="xs:string" minOccurs="0"/>

<xs:element name="associated-group-id" type="xs:string" minOccurs="0"/>

<xs:element name="originated-by" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="MKFC-GKTPs" type="mvgktp:singleTypeGKTPsType" minOccurs="0"/>

<xs:element name="mcvideo-client-id" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="alert-ind-rcvd" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="multiple-devices-ind" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:element name="video-pull-url" type="mcvideoinfo:contentType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcvideoinfo:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--    anyExt elements for MCVideo-Params-->

<xs:element name="release-reason" type="mcvideoinfo:releaseReasonType"/>

<xs:simpleType name="releaseReasonType">

<xs:restriction base="xs:string">

<xs:enumeration value="private-call-expiry"/>

<xs:enumeration value="administrator-action"/>

<xs:enumeration value="not selected for call"/>

<xs:enumeration value="call-request-for-viewed-to-client"/>

<xs:enumeration value="call-request-initiated-by-viewed-to-client"/>

<xs:enumeration value="authentication of the MIKEY-SAKE I_MESSAGE failed"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="request-type" type="mcvideoinfo:requestTypeType"/>

<xs:simpleType name="requestTypeType">

<xs:restriction base="xs:string">

<xs:enumeration value="group-selection-change-request"/>

<xs:enumeration value="fa-group-binding-req"/>

</xs:restriction>

</xs:simpleType>

<!-- The request-type and response-type elements uses the enumeration as values, which cant be extended for new values to be added. Hence the new req-type element is defined and going forward all the new values are added to this element -->

<xs:element name="req-type" type="mcvideoinfo:contentType"/>

<xs:element name="response-type" type="mcvideoinfo:responseTypeType"/>

<xs:simpleType name="responseTypeType">

<xs:restriction base="xs:string">

<xs:enumeration value="group-selection-change-response"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="selected-group-change-outcome" type="mcvideoinfo:selectedGroupChangeOutcomeType"/>

<xs:simpleType name="selectedGroupChangeOutcomeType">

<xs:restriction base="xs:string">

<xs:enumeration value="success"/>

<xs:enumeration value="fail"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="affiliation-required" type="xs:boolean"/>

<xs:element name="ambient-viewing-type" type="mcvideoinfo:ambientViewingType"/>

<xs:simpleType name="ambientViewingType">

<xs:restriction base="xs:string">

<xs:enumeration value="remote-init"/>

<xs:enumeration value="local-init"/>

</xs:restriction>

</xs:simpleType>

<xs:element name="video-push-url" type="xs:anyURI"/>

<xs:element name="functional-alias-URI" type="mcvideoinfo:contentType"/>

<xs:element name="user-requested-priority" type="xs:nonNegativeInteger"/>

<xs:element name="emergency-alert-area-ind" type="xs:boolean"/>

<xs:element name="call-to-functional-alias-ind" type="xs:boolean"/>

<xs:element name="group-geo-area-ind" type="xs:boolean"/>

<xs:element name="binding-ind" type="xs:boolean"/>

<xs:element name="binding-fa-uri" type="xs:anyURI"/>

<xs:element name="unbinding-fa-uri" type="xs:anyURI"/>

<xs:simpleType name="protectionType">

<xs:restriction base="xs:string">

<xs:enumeration value="Normal"/>

<xs:enumeration value="Encrypted"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="contentType">

<xs:choice>

<xs:element name="mcvideoURI" type="xs:anyURI"/>

<xs:element name="mcvideoString" type="xs:string"/>

<xs:element name="mcvideoBoolean" type="xs:boolean"/>

<xs:any namespace="##other" processContents="lax"/>

<xs:element name="anyExt" type="mcvideoinfo:anyExtType" minOccurs="0"/>

</xs:choice>

<xs:attribute name="type" type="mcvideoinfo:protectionType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

## F.1.3	Semantic

The <mcvideoinfo> element is the root element of the XML document. The <mcvideoinfo> element can contain subelements.

NOTE 1:	The subelements of the <mcvideo-info> are validated by the <xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> particle of the <mcvideo-info> element

If the <mcvideoinfo> contains the <mcvideo-Params> element then:

1)	the <mcvideo-access-token>, <mcvideo-request-uri>, <mcvideo-calling-user-id>, <mcvideo-called-party-id>, <mcvideo-calling-group-id>, <emergency-ind>, <alert-ind>, <imminentperil-ind>, <originated-by> and <mcvideo-client-id>, <functional-alias-URI>, <req-type>  and <multiple-devices-ind> elements can be included with encrypted content;

2)	for each element in 1) that is included with content that is not encrypted:

a)	the element has the "type" attribute set to "Normal";

b)	if the element is the <mcvideo-request-uri>, <mcvideo-calling-user-id>, <mcvideo-called-party-id> or <mcvideo-calling-group-id>, <originated-by> or <functional-alias-URI> then the <mcvideoURI> element is included;

c)	if the element is <mcvideo-access-token>, <req-type> or <mcvideo-client-id>, then the <mcvideoString> element is included; and

d)	if the element is <emergency-ind>, <alert-ind>, <imminentperil-ind> or <multiple-devices-ind> then the <mcvideoBoolean> element is included;

3)	for each element in 1) that is included with content that is encrypted:

a)	the element has the "type" attribute set to "Encrypted";

b)	the <xenc:EncryptedData> element from the "http://www.w3.org/2001/04/xmlenc#" namespace is included and:

i)	can have a "Type" attribute can be included with a value of "http://www.w3.org/2001/04/xmlenc#Content";

ii)	can include an <EncryptionMethod> element with the "Algorithm" attribute set to value of "http://www.w3.org/2009/xmlenc11#aes128-gcm";

iii)	can include a <KeyInfo> element with a <KeyName> element containing the base 64 encoded XPK-ID; and

iv)	includes a <CipherData> element with a <CipherValue> element containing the encrypted data.

NOTE 2:	When the optional attributes and elements are not included within the <xenc:EncryptedData> element, the information they contain is known to sender and the receiver by other means.

If the <mcvideoinfo> contains the <mcvideo-Params> element then:

1)	the <mcvideo-access-token> element can be included with the access token received during authentication procedure as described in 3GPP TS 24.482 [52];

2)	the <session-type> element can be included and set to:

a)	"chat" to indicate that the MCVideo client wants to join a chat group call

b)	"prearranged" to indicate the MCVideo client wants to make a prearranged group call;

c)	"private" to indicate the MCVideo client wants to make a private call;

d)	"ambient-viewing" to indicate the MCVideo client wants to make an ambient viewing call;

e)	"pull-from-server" to indicate the MCVideo client wants to pull video file from MCVideo server;

f)	"pull-from-user" to indicate the MCVideo client wants to to pull video media from another MCVideo client;

g)	"push-to-server" to indicate the MCVideo client wants to push video media to MCVideo server, save as a file;

h)	"one-to-one video push" to indicate the MCVideo client wants to push video media to another MCVideo client; or

i)	"one-to-server video push" to indicate the MCVideo client wants to push video media to a MCVideogroup;

3)	the <mcvideo-request-uri> element can be included with:

a)	the value set to an MCVideo group ID or temporary MCVideo group ID when the <session-type> element is set to a value of "prearranged" or "chat"; and

b)	the value set to the MCVideo ID of the called MCVideo user when the <session-type> element is set to a value of "private";

4)	the <mcvideo-calling-user-id> element can be included, set to the MCVideo ID of the originating user;

5)	the <mcvideo-called-party-id> element can be included, set to the MCVideo ID of the terminating user;

6)	the <mcvideo-calling-group-id> element can be included to indicate the MCVideo group identity to the terminating user;

7)	the <required> element can be included in a SIP 183 (Session Progress) from a non-controlling MCVideo function of an MCVideo group to inform the controlling MCVideo function that the group on the non-controlling MCVideo function has group members in the group document which are marked as <on-network-required>, as specified in 3GPP TS 24.481 [24];

8)	the <emergency-ind> element can be set to:

a)	"true" to indicate that the call that the MCVideo client is initiating is an emergency MCVideo call; or

b)	"false" to indicate that the MCVideo client is cancelling an emergency MCVideo call (i.e. converting it back to a non-emergency call)

9)	the <alert-ind> element can be set to:

a)	"true" in an emergency call initiation to indicate that an alert to be sent; or

b)	"false" when cancelling an emergency call which requires an alert to be cancelled also

10)	if the <session-type> element is set to "chat" or "prearranged":

a)	the <imminentperil-ind> element can be set to "true" to indicate that the call that the MCVideo client is initiating is an imminent peril group MCVideo call;

11)	the <broadcast-ind> element can be set to:

a)	"true" indicates that the MCVideo client is initiating a broadcast group call; or

b)	"false" indicates that the MCVideo client is initiating a non-broadcast group call;

12)	the <mc-org> element can be:

a)	set to the MCVideo user's Mission Critical Organization in an emergency alert sent by the MCVideo server to terminating MCVideo clients;

13)	Void;

14)	the <associated-group-id> element:

a)	if the <mcvideo-request-uri> element contains a group identity then this element can include an MCVideo group ID associated with the group identity in the <mcvideo-request-uri> element. E.g. if the <mcvideo-request-uri> element contains a temporary group identity (TGI), then the <associated-group-id> element can contain the constituent MCVideo group ID;

15)	the <originated-by> element:

a)	can be included, set to the MCVideo ID of the originating user of an MCVideo emergency alert when being cancelled by another authorised MCVideo user;

16)	the <MKFC-GKTPs> element:

a)	contains a group key transport payload carrying one or more MKFC(s) and MKFC-ID(s) as described in3GPP TS 24.481 [24] clause 7.4, to be used for protection of multicast transmission control signalling when the UE operates on the network;

NOTE 3:	A GMS (Group Management Server) compliant to Release 14 of the present document does not send a group key transport payload carrying MKFC and MKFC-ID. A GMC (Group Management Client) can receive MKFC and MKFC-ID from a GMS compliant only to Release 13 of the present document.

17)	the <mcvideo-client-id> element:

a)	can be included, set to the MCVideo client ID of the MCVideo client that originated a SIP INVITE request, SIP REFER request, SIP REGISTER request, SIP PUBLISH request or SIP MESSAGE request.

18)	the <alert-ind-rcvd> element:

a)	can be set to true and included in a SIP MESSAGE to indicate that the emergency alert or cancellation was received successfully;

18a)	the <multiple-devices-ind> element set to:

a)	"true" to indicate to the client that multiple clients are registered for the MCVideo user; or

b)	"false" to indicate to the client that no other clients are registered for the MCVideo user;

18b)	the <video-pull-url> element:

a)	can be set to the URL of the video file located in the MCVideo server; and

19)	the <anyExt> can be included with the following elements:

a)	a <release-reason> element set to:

i)	"authentication of the MIKEY-SAKE I_MESSAGE failed" by a MCVideo client when the signature cannot be verified;

ii)	"private-call-expiry" when the ambient viewing call is release due to the expiry of the private call timer;

iii)	"administrator-action" when the ambient viewing call is released by an MCVideo administrator;

iv)	"call-request-for-viewed-to-client" when there is a call request targeted to the viewed-to client; or

v)	"call-request-initiated-by-viewed-to-client" when there is a call request initiated by the viewed-to client;

b)	a <request-type> element set to:

i)	"group-selection-change-request" when a client initiates a group selection change request; or

ii)	"fa-group-binding-req" when a client initiates a request for binding of a functional alias with the MCVideo group(s) for the MCVideo user;

c)	a <response-type> element set to:

i)	"group-selection-change-response" when a client responds to a group selection change request;

d)	a <selected-group-change-outcome> element set to:

i)	"success" when a client reports that it has successfully changed its selected group as requested by a received group selection change request; or

ii)	"fail" when a client reports that it has failed to change its selected group as requested by a received group selection change request;

e)	an<affiliation-required> element set to:

i)	"true" when received by a client in a group-selection-change-request indicates that the client needs to affiliate to the specified group;

f)	an <ambient-viewing-type> element set to:

i)	"remote-init" when the viewing MCVideo user of an ambient viewing call initiates the call; or

ii)	"local-init" when the viewed-to MCVideo user of an ambient viewing call initiates the call;

g)	an <video-push-url> element:

i)	set to the URL of the video file located in the MCVideo server; and

h)	a <functional-alias-URI> element set to the value of the functional alias that is used together with the "mcvideo-calling-user-id";

i)	an <emergency-alert-area-ind> element set to:

i)	"true" when the MCVideo client has entered an emergency alert area; or

ii)	"false" when the MCVideo client has exited an emergency alert area;

j)	a <group-geo-area-ind> element set to:

i)	"true" when the MCVideo client has entered a group geographic area; or

ii)	"false" when the MCVideo client has exited a group geographic area:

k)	a <binding-ind> element set to:

i)	"true" when the user wants to create a binding of a particular functional alias with the specified list of MCVideo groups for the MCVideo client; or

ii)	"false" when the user wants to remove a binding of a particular functional alias from the specified list of MCVideo groups for the MCVideo client;

l)	a <binding-fa-uri> element set to:

i)	a URI of a functional alias that shall be bound with the specified list of MCVideo groups for the MCVideo client;

m)	a <unbinding-fa-uri> element set to:

i)	a URI of a functional alias that shall be unbound from the specified list of MCVideo groups for the MCVideo client;

n)	a <call-to-functional-alias-ind> element set to:

i)	"true" when the MCVideo client is using a functional alias to identify the MCVideo IDs of the potential target MCVideo users; or

ii)	"false" when the MCVideo client is using MCVideo IDs to identify the potential target MCVideo users;

o)	a <user-requested-priority> element set to the non-negative integer value requested by the user as priority; and

p)	a <req-type> element set to:

i)	"functional-alias-status-determination" when a MCVideo client initiates a subscription to functional alias status.

Absence of the <emergency-ind>, <alert-ind> and <imminentperil-ind> elements in a SIP INVITE request indicates that the MCVideo client is initiating a non-emergency private call or non-emergency group call.

Absence of the <broadcast-ind> element in a SIP INVITE request indicates that the MCVideo client is initiating a non-broadcast group call.

Absence of the <call-to-functional-alias-ind> in a SIP INVITE or a SIP REFER request for a first-to-answer call indicates the use of the MCVideo IDs of the potential target MCVideo users.

The recipient of the XML ignores any unknown element and any unknown attribute.

* * * End of Change * * *