3GPP TSG-CT WG1 Meeting #158	C1-257077

Dallas, US, 17-21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.281 | CR | 0297 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCVideo Regroup XML schema |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Firstnet, AT&T, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh3MCPTT-CT |  |  |  |  |  | Date: |  |  | 2025-10-20 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are XML schemas for regroup using preconfigured group in 24.379, 24.281 and 24.282, all using the same target namespace: - targetNamespace="urn:3gpp:ns:preconfiguredRegroup:1.0" All three XML schemas define some identical types: - preconfig-group-Type - groups-for-regroup-Type - users-for-regroup-Type - anyExtType Since the schemas have the same namespace these types occur duplicated, which cause a problem when compiling the schemas. The issue occurs only when the schema files are compiled in one common MC system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MCvideo regroup schema is update to include the type definition from the MCPTT schema in TS 24.379. The duplicated types are removed in the MCVideo schema. Backward Compatibility This change does not cause any backward compatibility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCVideo XML schema for regroup can not be used in combination with the MCData and MCPTT XML schemas. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## F.6.2	XML schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:preconfiguredRegroup:1.0"

xmlns:mcvideorgrp="urn:3gpp:ns:preconfiguredRegroup:1.0"

attributeFormDefault="unqualified" elementFormDefault="qualified">

<!-- The schema includes common type definition from 3GPP TS 24.379 clause F.7.2-->

<xs:include schemaLocation="mcpttregroup.xsd"/>

<!-- root XML element -->

<xs:element name="mcvideoregroup" type="mcvideorgrp:mcvideoregroup-Type" id="info"/>

<xs:complexType name="mcvideoregroup-Type">

<xs:sequence>

<xs:element name="mcvideoregroup-Params" type="mcvideorgrp:mcvideoregroup-ParamsType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcvideorgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mcvideoregroup-ParamsType">

<xs:sequence>

<xs:element name="preconfig-group-id" type="mcvideorgrp:preconfig-group-Type"/>

<xs:element name="mcvideo-regroup-uri" type="mcvideorgrp:mcvideo-regroup-uri-Type"/>

<xs:element name="groups-for-regroup" type="mcvideorgrp:groups-for-regroup-Type" minOccurs="0"/>

<xs:element name="users-for-regroup" type="mcvideorgrp:users-for-regroup-Type" minOccurs="0"/>

<xs:element name="regroup-action" type="xs:string"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcvideorgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="mcvideo-regroup-uri-Type">

<xs:sequence>

<xs:element type="xs:anyURI" name="mcvideo-regroup-uri"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcvideorgrp:anyExtType" minOccurs="0"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:schema>

* * * End of Change * * *