3GPP TSG-CT WG1 Meeting #158	C1-257502

Dallas, US , 17-21 November 2025	Revision of C1-257094

Source:	OPPO

Title:	Pseudo-CR on STATUS cause values

Spec:	3GPP TS 24.369 v1.1.0

Agenda item:	19.70 (AmbientIoT-CT)

Document for:	Agreement

1. Reason for Change

In the current version of the specification, AIoT device receives only a single type of the AIoT NAS messages – a Command message. All Command messages are integrity protected, while they may or may not be ciphered. A STATUS message may be sent only as a response to a Command message, see clause 6.3. Therefore, for AIOTF to process the received STATUS message, it shall also be integrity protected and ciphered, if the triggering Command was ciphered.

In Annex A.1 specifies that cause #x1 is used to indicate that the requested read or write command is not supported by the AIoT device. The problem is, if AIoT device does not support the requested read or write command, then it cannot send this value ever. Rather, the AIoT device shall send cause #97 Message type non-existent or not implemented. Therefore, cause #x1 must be removed from Annex A.1. Clarification is also added to the annex.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.1.0.

*******

* * * First Change * * * *

#### 5.4.1.1	General

The purpose of the sending of the STATUS message is to report certain error conditions detected in a received AIoT NAS message by the AIoT device (see example in figure 5.4.1.1.1). The STATUS message shall be integrity protected. The same ciphering shall apply to the STATUS message as the ciphering in the received AIoT message that triggers the STATUS message.

AIoT device

AIOTF

STATUS

Figure 5.4.1.1.1: Status procedure

* * * 2nd Change * * * *

### 7.1.11	STATUS

#### 7.1.11.1	Message definition

The STATUS message is sent by the AIoT device to the AIOTF to report certain error conditions listed in clause 6.

See table 7.1.11.1-1.

Message type:	STATUS

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.11.1-1: STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Security header | Security header 7.2.3 | M | V | 1 |
|  | Message authentication code | Message authentication code 7.2.9 | M | V | 4 |
|  | Status message identity | Message type 7.2.2 | M | V | 1 |
|  | Status cause | Cause 7.2.8 | M | V | 1 |

* * * End of Changes * * * *