3GPP TSG-CT WG1 Meeting #158	draft C1-257615

Dallas, USA 17-21 Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4591 | rev | 4 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | A NAS notification to AS when NAS procedures cannot be initiated in a camped on S&F cell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Sateliot, Novamint |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  |  |  |  |  | Date: |  |  | 2025-11-17 |  |  |  |
| Category: | F |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-19 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |  |  |
| Reason for change: |  | In the context of S&F satellite operation, a UE may not initiate any NAS procedures when camped on an S&F satellite E-UTRA cell. Reasons for not initiating NAS procedures could be due to, for example, restrictions associated with the handling of the timer T3451 and/or due to how the UE uses the S&F monitoring list. When this situation arises, the UE may remain camped on a cell which is considered useable as per AS conditions but renders useless as per NAS conditions, preventing the UE from considering other potential candidate cells. To avoid such a situation, it is proposed that the NAS layer may provide an indication to the AS layer to inform that NAS procedures cannot be initiated in a camped on cell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | As an implementation option, notes are added to the effect such that when NAS detects that it cannot initiate any NAS procedure over an S&F satellite E-UTRA cell, NAS should indicate this to AS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A situation could arise where a UE remains camped on a cell which is considered useable as per AS conditions but renders useless as per NAS conditions, preventing the UE from considering other potential candidate cells while draining its battery energy. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.5.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 4.11.5.2	Handling of timers

The UE supporting S&F satellite operation shall have no more than one timer T3451 running at a time.

The UE maintains timer T3451, which is started with the value of the S&F wait time duration received from the network during an attach procedure, a tracking area updating procedure or a service request procedure.

NOTE 1:	The S&F Wait Timer value takes into account the time taken to synchronize the UE’s context between MME(s) as specified in subclause O.2 of 3GPP TS 23.401 [10]).

NOTE 2: As an implementation option for the UE, if NAS detects that NAS procedures cannot be initiated in the current cell because T3451 is running, NAS can indicate this to AS.

The UE shall stop the timer T3451, if running, when the UE:

1)	successfully performs an attach procedure or a tracking area updating procedure:

a)	via a cell which is not a S&F satellite E-UTRAN cell;

b) 	via a cell being in a TA which is not in the list of TAs; or

c)	in a new PLMN;

2)	responds to paging; or

3)	successfully performs intersystem change from S1 mode to N1 mode or A/Gb mode or Iu mode.

The UE stores the S&F wait time duration in the non-volatile memory of the ME as described in annex C.

* * * End of first Change * * *