3GPP TSG-CT WG1 Meeting #157	C1-256792

Sophia Antipolis, France, 13-17 October 2025	was C1-256194

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24.554 | CR |  | 0841 |  |  |  |  | rev |  |  | 1 |  |  | Current version: |  |  |  |  |  |  | 19.3.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  | UICC apps |  |  |  |  |  |  | ME | X |  | Radio Access Network |  |  |  |  |  |  | Core Network |  |  |
| Title: |  |  | RRC container Related Updates to the Multi-hop UE-to-Network Relay Discovery Procedures and Messages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  | NIST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  | CT1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  | 5G_ProSe_Ph3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 2025-10-1 |  |  |  |  |  |
| Category: |  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-19 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |  |  |  |  |
| Reason for change: |  |  |  |  | RP-251941 captuures the following agreement from RAN2 #131, which assumes the presence of RRC container in the discovery response msg: - “Working assumption on having RRC_CONNECTED state indication for the announcing/responding UE (only) in the discovery AS container is confirmed. No SA2/CT1 impact is expected.” However, RRC container is not mentioned in the discovery procedure (8b.2.1.3.3.2, 8b.2.1.3.3.3) or discovery response message (10.2.1) for multi-hop U2N relay with model B yet. Thus the proposed 2nd and 3rd changes. In addition, RP-251941 also captures RAN2 #131 agreement that lower layers will decide whether to keep or modify the content of RRC container: - “The RRC_CONNECTED indicator in the discovery message RRC container reflects the state of the UE that sends the discovery message. The UE modifies the container if necessary to include this information, but this does not preclude a UE from forwarding the container unchanged if it has the same RRC_CONNECTED status as the UE from which it received the container. Detailed spec impact can be discussed in the running CR implementation.” Thus the proposed 1st change. The 4th change is proposed for the consistency. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  | Update multi-hop U2N relay discovery procedures and message definitions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  | Inconsistency with RAN2#131 agreements on multihop U2N Relay discovery related to RRC container. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 8b.2.1.2.3.2, 8b.2.1.3.3.2, 8b.2.1.3.3.3, 10.2.1, 10.2.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  | x | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  | x | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  | x | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  | S2-2509243 is submitted to SA2 on TS 23.304 to reflect the agrements at RAN2 #131 cited above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 8b.2.1.2.3.2	ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring

This procedure is performed by:

a)	a 5G ProSe intermediate UE-to-network relay UE to monitor a 5G ProSe multi-hop UE-to-network relay UE; or

b)	a 5G ProSe multi-hop child UE-to-network relay UE to monitor a 5G ProSe multi-hop parent UE-to-network relay UE.

The UE is authorised to perform the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring if:

a)	the following is true:

1)	if the UE is not served by NG-RAN, the UE is authorised to perform 5G ProSe direct discovery monitoring when the UE is configured with the radio parameters to be used for 5G ProSe direct discovery when not served by NG-RAN; or

2)	if the UE is served by NG-RAN, the UE is authorised to perform 5G ProSe direct discovery monitoring in at least one PLMN; and

b)	the UE is configured with the relay service code parameter identifying the connectivity service to be monitored, as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE.

otherwise, the UE is not authorised to perform the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring.

Figure 8b.2.1.2.3.2-1 illustrates the interaction of the UEs in the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring.

Figure 8b.2.1.2.3.2-1: ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring

When the UE is triggered by the upper layers to monitor proximity of a connectivity service provided by a multi-hop UE-to-network relay or a intermediate UE-to-network relay or when the UE decides to perform 5G ProSe multi-hop UE-to-network relay reselection as specified in clause 8b.2.3, and if the UE is authorised to perform the ProSe intermediate UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery monitoring, then the UE shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages with the default destination layer-2 ID as specified in clause 5.2.8.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement according to clause 10.2.1, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to monitor, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the monitoring operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement.

NOTE 1:	The use of an erroneous UTC-based counter for processing received PROSE PC5 DISCOVERY messages at the ProSe-enabled UE can cause MIC check failure after DUIK is used for integrity check and malformed contents after DUSK is used for unscrambling or DUCK is used for deciphering. How a ProSe-enabled UE ensures the accuracy of the UTC-based counter is left to UE implementation.

NOTE 2:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the relay service code parameter configured as specified in clause 5 for the connectivity service being monitored;

b)	the target announcer info is not provided by upper layers for the connectivity service being monitored, or the announcer info parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the target announcer info if the target announcer info is provided by upper layers for the connectivity service being monitored;

c)	the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is less than the configured maximum number of allowed hops, as specified in clause 5.2.8; and

d) 	the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is less than the hop limit parameter of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement (if available),

then

a)	the UE may measure the signal strength of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement for relay selection or reselection;

b)	if a 5G ProSe UE-to-Network relay discovery entry corresponding to the relay service code parameter and the root relay info parameter of the received PROSE PC5 DISCOVERY message is not stored at the UE:

1)	the UE shall create and store a 5G ProSe UE-to-Network relay discovery entry corresponding to the received PROSE PC5 DISCOVERY message. The 5G ProSe UE-to-Network relay discovery entry:

i)	shall include RSC Info, set to the relay service code parameter of the received PROSE PC5 DISCOVERY message;

ii)	shall include the Announcer Info, set to the Announcer Info parameter of the received PROSE PC5 DISCOVERY message;

iii)	shall include Hop Count Info, set to the hop count parameter of the received PROSE PC5 DISCOVERY message;

iv)	may include Root Relay Info, set to the root relay info parameter of the received PROSE PC5 DISCOVERY message;

v)	may include the Accumulated QoS for PC5 link Info, based on the Accumulated QoS for PC5 link information parameter of the received PROSE PC5 DISCOVERY message and the QoS of the PC5 link between the UE and the sender of the message;

vi)	may include the NCGI Info, set to the NCGI parameter of the received PROSE PC5 DISCOVERY message;

vii)	 may include the RRC Container Info if provided by the lower layers; and

viii)	shall include the hop limit, set to the hop limit parameter of the received PROSE PC5 DISCOVERY message, if available; and

c)	if a 5G ProSe UE-to-Network relay discovery entry corresponding to the relay service code parameter and the root relay info parameter of the received PROSE PC5 DISCOVERY message is already stored, then the UE shall:

1)	check if the hop count parameter of the received PROSE PC5 DISCOVERY message is less than the Hop Count Info of the stored 5G ProSe UE-to-Network relay discovery entry. If yes, the UE:

i)	shall update the Announcer Info of the stored 5G ProSe UE-to-network relay discovery entry to the Announcer Info parameter of the received PROSE PC5 DISCOVERY message;

ii)	shall update the Hop Count Info of the stored 5G ProSe UE-to-network relay discovery entry to the hop count parameter of the received PROSE PC5 DISCOVERY message; and

iii)	may update the Accumulated QoS for PC5 link Info, based on the Accumulated QoS for PC5 link information parameter of the received PROSE PC5 DISCOVERY message and the QoS of the PC5 link between the UE and the sender of the message; and

2)	check if the hop limit parameter of the received PROSE PC5 DISCOVERY message (if available) is different with the Hop Limit Info of the stored 5G ProSe UE-to-network relay discovery entry. If yes, the UE shall update the Hop Limit Info of the stored 5G ProSe-UE-to-network relay discovery entry to the hop limit parameter of the received PROSE PC5 DISCOVERY message.

If the hop count parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement is the same as the configured maximum number of allowed hops as specified in clause 5.2.8; or is the same as the hop limit parameter of the PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement (if available), then the UE shall discard the received PROSE PC5 DISCOVERY message.

NOTE 3:	Based on UE implementation, the UE may decide to update the 5G ProSe UE-to-network relay discovery entry based on the Accumulated QoS for PC5 link Info instead of the Hop Count Info, or a combination of the two information elements.

NOTE 4:	Based on UE implementation, the UE sets a locally configured timer for each 5G ProSe UE-to-network relay discovery entry. When the timer expires, if the UE does not have an active PC5 link with the 5G ProSe multi-hop UE-to-network relay UE and does not receive any new PROSE PC5 DISCOVERY message from the multi-hop UE-to-network relay UE, the UE removes 5G ProSe UE-to-network relay discovery entry of the 5G ProSe multi-hop UE-to-network relay UE.

* * * Second Change * * * *

###### 8b.2.1.3.3.2	Discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe multi-hop UE-to-network relay UE towards a 5G ProSe intermediate UE-to-network relay UE or a 5G ProSe multi-hop remote UE.

The UE is authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe multi-hop UE-to-network relay UE in the PLMN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe multi-hop layer-3 UE-to-network relay UE,

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code and the maximum number of allowed hops as specified in clause 5.2.8; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1 illustrates the interaction of the UEs in the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1: Discoveree UE procedure for intermediate UE-to-network Relay discovery

When the UE is triggered by an upper layer application to start responding to solicitation on proximity of a connectivity service provided by the 5G ProSe multi-hop UE-to-network relay UE and if the UE is authorised to perform the discoveree UE-to-network relay UE procedure for multi-hop UE-to-network relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, for the relay service code of the connectivity service which the UE is authorized to respond, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation.

NOTE 1:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is the same as the relay service code parameter configured as specified in clause 5.2.8 for the connectivity service;

b)	the target discoveree info parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is the same as the user info ID for the multi-hop UE-to-network relay discovery provided by upper layers or same as the configured user info ID for the multi-hop UE-to-network relay discovery as specified in clause 5.2.8, if the target discoveree info parameter is included in the received PROSE PC5 DISCOVERY message; and

c)	if the hop count parameter in the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation is not greater than the maximum number of allowed hops as specified in clause 5.2.8;

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, the UE:

1)	shall set the Discoveree info parameter to the configured User info ID for the multi-hop UE-to-network Relay discovery parameter, as specified in clause 5.2.8;

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation. The relay service code parameter is set to relay service code specific for emergency service if the UE is triggered by an upper layer application to announce availability of an emergency connectivity service provided by a UE-to-network relay, and either of the following:

i)	for 5G ProSe multi-hop layer-3 UE-to-network relay UE, the EMC bit in the 5GS network feature support IE of the REGISTRATION ACCEPT message is not set to "Emergency services not supported" as specified in 3GPP TS 24.501 [11] and the UE is neither using its own emergency service nor relaying the emergency services from one 5G ProSe multi-hop layer-3 remote UE;

3)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

4)	shall include the MIC filed computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

5)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

6)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.10;

7)	shall include the Multi-hop path info IE and include in it the ordered list of the User info IDs of the selected path, starting from the User info ID of the 5G ProSe multi-hop remote UE and ending with the User info ID of the last 5G ProSe intermediate UE-to-network relay UE;

Editor’s note [5G_ProSe_Ph3, CR#0680]:	It is FFS whether the Layer-2 IDs are included in the Multi-hop path info IE, together with the list of the User info IDs (or as a separate IE), due to the following statement from clause 6.3.2.5.3 in 3GPP TS 23.304: "the Relay Discovery Response message is forwarded along the path, where each 5G ProSe Intermediate UE-to-Network Relay determines the next hop information (e.g. User Info ID and Layer-2 ID) to forward the Relay Discovery Response message".

8)	may include the hop count parameter to indicate the number of hops between the 5G ProSe multi-hop remote UE and the 5G ProSe multi-hop UE-to-network relay UE on the path selected by the 5G ProSe multi-hop UE-to-network relay UE;

9)	may include the Accumulated QoS IE that includes the QoS supported over all the PC5 links along the path from the 5G ProSe multi-hop remote UE to the 5G ProSe multi-hop UE-to-network relay UE; and

10)	shall include the PLMN ID set to its HPLMN ID and in cleartext; and

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

11) if acting as 5G ProSe layer-2 multi-hop UE-to-network relay UE, shall set the RRC container to the RRC container if provided by the lower layers;

c)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

d)	shall set the destination layer-2 ID to the source layer-2 ID received from the discoverer UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery response message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

e)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

NOTE 3:	It is up to the implementation of the 5G ProSe multi-hop UE-to-network relay UE how it decides when to send the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, e.g. due to considering the need to wait till it gets multiple PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation from different paths. The 5G ProSe multi-hop UE-to-network relay UE can decide on the selected path based on different factors such as the PC5 signal strength, number of hops to the 5G ProSe multi-hop remote UE, the path information in the Multi-hop path info IE, the Accumulated QoS for PC5 link.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once generating the PROSE PC5 DISCOVERY message. Each self-assigned source layer-2 ID can be associated with only one relay service code.

NOTE 4:	If the UE is processing a PROSE DIRECT LINK ESTABLISHMENT REQUEST message from the same source layer-2 ID of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, it depends on UE implementation to avoid the conflict of destination layer-2 ID (e.g. send a PROSE DIRECT LINK ESTABLISHMENT REJECT message containing PC5 signalling protocol cause value #3 "conflict of layer-2 ID for unicast communication is detected", or ignore the PROSE DIRECT DISCOVERY message for multi-hop UE-to-network relay discovery solicitation).

###### 8b.2.1.3.3.3	Discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery initiation

This procedure is performed by:

a)	a 5G ProSe intermediate UE-to-network relay UE towards a 5G ProSe multi-hop remote UE; or

b)	a 5G ProSe multi-hop parent UE-to-network relay UE towards a 5G ProSe multi-hop child UE-to-network relay UE.

The UE is authorised to perform the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery if:

a)	the UE is authorised to act as a 5G ProSe intermediate UE-to-network relay UE in the PLMN indicated by the serving cell, and

1)	the UE is served by NG-RAN; or

2)	the UE is not served by NG-RAN and intends to use the provisioned radio resources for UE-to-network relay discovery;

b)	the UE is configured with:

1)	the relay service code parameter identifying the connectivity service to be solicited or responded to as specified in clause 5.2.8 and the indicated security procedure for the relay service code is supported by the UE. For 5G ProSe layer-3 intermediate UE-to-network relay UE,

i)	the S-NSSAI associated with that relay service code shall belong to the allowed NSSAI of the UE; and

ii)	if the UE is camped on a cell whose TAI is in the list of "non-allowed tracking areas" or is camped on a cell whose TAI is not in the list of "allowed tracking areas", then the relay service code shall be associated with high priority access as defined in clause 5.3.5 of 3GPP TS 24.501 [11];

2)	an indication that multi-hop relay is allowed for the relay service code; and

3)	the User info ID for the multi-hop UE-to-network relay discovery parameter, as specified in clause 5.2.8; and

c)	the back-off timer T3346 used for NAS mobility management congestion control as specified in clause 5.3.9 of 3GPP TS 24.501 [11] is not running at the UE;

otherwise, the UE is not authorised to perform the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1 illustrates the interaction of the UEs in the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery.

Figure 8b.2.1.3.3.2.1: Discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery

When the UE is triggered by an upper layer application to start responding to responses of proximity of a connectivity service provided by the 5G ProSe multi-hop UE-to-network relay UE or another 5G ProSe intermediate UE-to-network relay UE and if the UE is authorised to perform the discoveree intermediate UE procedure for multi-hop UE-to-network relay discovery, then the UE:

a)	if the UE is served by NG-RAN and the UE in 5GMM-IDLE mode needs to request resources for sending PROSE PC5 DISCOVERY messages as specified in 3GPP TS 38.331 [13], shall perform a service request procedure as specified in 3GPP TS 24.501 [11]; and

b)	shall instruct the lower layers to start monitoring for PROSE PC5 DISCOVERY messages.

Upon reception of a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, for the relay service code of the connectivity service which the UE is authorized to respond, if the received PLMN ID in the message is not associated with the target relay service code of the connectivity service which the UE is authorized to respond, the UE shall ignore the message. If the received PLMN ID in the message is associated with the target relay service code of the connectivity service which the UE is authorized to respond the UE shall use the associated DUSK, if received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used) and the UTC-based counter obtained during the reception operation to unscramble the PROSE PC5 DISCOVERY message as described in 3GPP TS 33.503 [34]. Then, if a DUCK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUCK and the UTC-based counter to decrypt the configured message-specific confidentiality-protected portion, as described in 3GPP TS 33.503 [34]. Finally, if a DUIK is received from the 5G DDNMF or 5G PKMF (if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used), the UE shall use the DUIK and the UTC-based counter to verify the MIC field in the unscrambled PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response.

NOTE 1:	The UE can determine the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is for 5G ProSe direct discovery based on an indication from the lower layer.

Then, if:

a)	the relay service code parameter of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response is the same as the relay service code parameter configured as specified in clause 5.2.8 for the connectivity service; and

b)	the User info ID of the receiving UE is included in the list of User info IDs included in the Multi-hop path info IE in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

then the UE:

a)	shall obtain a valid UTC time for the discovery transmission from the lower layers and generate the UTC-based counter corresponding to this UTC time;

b)	shall generate a PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. In the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, the UE:

1)	shall set the Discoveree info parameter to the User info ID obtained from the Discoveree info included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

2)	shall set the relay service code parameter to the relay service code parameter of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response. The relay service code parameter is set to relay service code specific for emergency service if the UE is triggered by an upper layer application to announce availability of an emergency connectivity service provided by a multi-hop UE-to-network relay, and either of the following:

i)	for 5G ProSe multi-hop layer-3 UE-to-network relay UE, the EMC bit in the 5GS network feature support IE of the REGISTRATION ACCEPT message is not set to "Emergency services not supported" as specified in 3GPP TS 24.501 [11] and the UE is neither using its own emergency service nor relaying the emergency services from one 5G ProSe multi-hop layer-3 remote UE;

3)	shall set the Resource Status Indicator bit of the status indicator parameter to indicate whether or not the UE has resources available to provide a connectivity service for additional ProSe-enabled UEs;

4)	shall include the MIC filed computed as described in 3GPP TS 33.503 [34] by using the UTC-based counter and the DUIK contained in the <UNR-discovery-security-parameters-accept> element of the PROSE_SECURITY_PARAM_RESPONSE message, if security procedure over user plane for 5G ProSe multi-hop UE-to-network relay is used, or the PROSE_SECURITY_MATERIAL_RESPONSE message, if security procedure over control plane for 5G ProSe multi-hop UE-to-network relay is used;

5)	shall set the UTC-based counter LSB parameter to the 4 least significant bits of the UTC-based counter;

6)	shall set the ProSe direct discovery PC5 message type parameter as specified in table 10.2.1.10;

7)	shall include the Multi-hop path info IE and include in it the same list of User info IDs that were included in the Multi-hop path info IE included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

8)	may include the hop count parameter set to the same hop count parameter included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

9)	may include the Accumulated QoS IE set to the Accumulated QoS included in the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response;

10)	shall include the PLMN ID set to the PLMN ID value obtained from the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response, and in cleartext; and

NOTE 1:	The confidentiality security mechanism ensures that the PLMN ID IE is not encrypted as specified in 3GPP TS 33.503 [34]. Also the scrambling security mechanism ensures that the PLMN ID IE is not scrambled as specified in 3GPP TS 33.503 [34].

11) if acting as 5G ProSe layer-2 intermediate UE-to-network relay UE, shall set the RRC container to the RRC container if provided by the lower layers;

c)	shall apply the DUIK, DUSK, or DUCK with the associated Encrypted Bitmask, along with the UTC-based counter to the PROSE PC5 DISCOVERY message for whichever security mechanism(s) configured to be applied, e.g., integrity protection, message scrambling or confidentiality protection of one or more above parameters, as specified in 3GPP TS 33.503 [34];

d)	shall set the destination layer-2 ID to the source layer-2 ID received from the discoverer UE used in the transportation of the PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation and self-assign a source layer-2 ID for sending the multi-hop UE-to-network relay discovery response message; and

NOTE 2:	The UE implementation ensures that the value of the self-assigned source layer-2 ID is different from any other self-assigned source layer-2 ID(s) in use for 5G ProSe direct communication as specified in clause 7.2 and is different from any other provisioned destination layer-2 ID(s) as specified in clause 5.2.

e)	shall pass the resulting PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response along with the source layer-2 ID, destination layer-2 ID and an indication that the message is for 5G ProSe direct discovery to the lower layers for transmission over the PC5 interface.

The UE shall maintain the association(s) between the self-assigned source layer-2 ID and relay service code once generating the PROSE PC5 DISCOVERY message. Each self-assigned source layer-2 ID can be associated with only one relay service code.

NOTE 3:	If the UE is processing a PROSE DIRECT LINK ESTABLISHMENT REQUEST message from the same source layer-2 ID of the received PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation, it depends on UE implementation to avoid the conflict of destination layer-2 ID (e.g. send a PROSE DIRECT LINK ESTABLISHMENT REJECT message containing PC5 signalling protocol cause value #3 "conflict of layer-2 ID for unicast communication is detected", or ignore the PROSE DIRECT DISCOVERY message for multi-hop UE-to-network relay discovery solicitation).

* * * Third Change * * * *

### 10.2.1	Message definition

This message is sent by the UE over the PC5 interface for open 5G ProSe direct discovery and restricted 5G ProSe direct discovery. See table 10.2.1.1, table 10.2.1.2, table 10.2.1.3, table 10.2.1.4, table 10.2.1.5, table 10.2.1.6, table 10.2.1.7, table 10.2.1.8, table 10.2.1.9, table 10.2.1.10, table 10.2.1.11, table 10.2.1.12, table 10.2.1.13, table 10.2.1.14, table 10.2.1.15, table 10.2.1.16, table 10.2.1.17, table 10.2.1.18 table 10.2.1.19, table 10.2.1.20, table 10.2.1.21, table 10.2.1.22, table 10.2.1.23 and table 10.2.1.24.

Message type:	PROSE PC5 DISCOVERY

Significance:	dual

Direction:	UE to peer UE

Table 10.2.1.1: PROSE PC5 DISCOVERY message content for open 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe application code | ProSe application code 11.2.2 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Open discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.2: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe restricted code | ProSe restricted code 11.2.3 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Announcement". |  |  |  |  |  |

Table 10.2.1.3: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe query code | ProSe restricted code 11.2.3 | M | V | 23 |
| 2A | Discoveree user info | Application layer ID 11.2.15 | O | TLV | 3-257 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "Solicitation". |  |  |  |  |  |

Table 10.2.1.4: PROSE PC5 DISCOVERY message content for restricted 5G ProSe direct discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | ProSe response code | ProSe restricted code 11.2.3 | M | V | 23 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery" and the content type is set to "response". |  |  |  |  |  |

Table 10.2.1.5: PROSE PC5 DISCOVERY message for group member discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-257 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.6: PROSE PC5 DISCOVERY message for group member discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
| 28 | Target user info | User info ID 11.2.7 | O | TV | 7 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.7: PROSE PC5 DISCOVERY message for group member discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.14 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Application layer group ID | Application layer group ID 11.2.6 | M | LV | 2-256 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
| 7A | Metadata | Metadata 11.2.13 | O | TLV-E | 4-8195 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Group member discovery announcement/group member discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.8: PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model A". NOTE 2: If the announcing UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.9: PROSE PC5 DISCOVERY message for UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.10: PROSE PC5 DISCOVERY message for UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 2) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-network relay discovery announcement/UE-to-network relay discovery response" and the discovery model is set to "Model B". NOTE 2: If the discoveree UE works as a 5G ProSe Layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.11: PROSE PC5 DISCOVERY message for relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 51 | Relay TAI | TAI 11.2.10 | O | TV | 4 |
| 10 | NID | NID 11.2.23 | O | TLV | 8 |
| NOTE: The discovery type is set to "Restricted discovery", the content type is set to "Relay discovery additional information" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.12: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 15-8323 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery announcement" and the discovery model is set to "Model A". |  |  |  |  |  |

Table 10.2.1.13: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 12-520 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 8- | Announce prohibited indication | Announce prohibited indication 11.2.19 | O | TV | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery solicitation" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.14: PROSE PC5 DISCOVERY message for UE-to-UE relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Direct discovery set | Direct discovery set 11.2.17 | M | LV-E | 12-520 |
| 2C | UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| 2D | Status indicator | Status indicator 11.2.9 | O | TV | 2 |
| 8- | Announce prohibited indication | Announce prohibited indication 11.2.19 | O | TV | 1 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "UE-to-UE relay discovery response" and the discovery model is set to "Model B". |  |  |  |  |  |

Table 10.2.1.15: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code (NOTE 3) | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
| 10 | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 11 | Root relay info | User info ID 11.2.7 | O | TV | 7 |
| 12 | Accumulated QoS | PC5 QoS flow descriptions 11.3.5 | O | TLV | 6-65538 |
| 52 | NCGI | NCGI 11.2.12 | O | TV | 9 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-network relay discovery announcement”. NOTE 3: If the announcing UE works as a 5G ProSe multi-hop layer-3 UE-to-network relay UE, the S-NSSAI associated with the relay service code belongs to the allowed NSSAI of the UE. |  |  |  |  |  |

Table 10.2.1.16: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery additional information

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Announcer info | User info ID 11.2.7 | M | V | 6 |
|  | Hop count | Hop count 11.2.21 | M | V | 1 |
| 8- | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 29 | Root relay info | User info ID 11.2.7 | O | TV | 7 |
| 52 | NCGI of multi-hop UE-to-network relay | NCGI 11.2.12 | O | TV | 9 |
| 10 | NCGI info set of intermediate UE-to-network relay | NCGI info set 11.2.25 | O | TLV | 16 |
| 51 | Relay TAI | TAI 11.2.10 | O | TV | 4 |
| 53 | NID | NID 11.2.23 | O | TLV | 8 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-network relay discovery additional information”. |  |  |  |  |  |

Table 10.2.1.17: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery solicitation

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| 20 | Multi-hop path info | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 21 | Intermediate relays | List of user info ID 11.3.47 | O | TLV | 8-257 |
| 22 | Hop count | Hop count 11.2.21 | O | TV | 1 |
| 23 | Hop limit | Hop limit 11.2.22 | O | TV | 1 |
| 24 | Accumulated QoS | PC5 QoS flow descriptions 11.3.5 | O | TLV | 6-65538 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to "Multi-hop UE-to-network relay discovery solicitation". |  |  |  |  |  |

Table 10.2.1.18: PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | PLMN ID | PLMN ID 11.2.20 | M | V | 3 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | Status indicator | Status indicator 11.2.9 | M | V | 1 |
|  | Multi-hop path info | List of user info ID 11.3.47 | M | LV | 7-257 |
| 10 | Hop count | Hop count 11.2.21 | O | TV | 1 |
| 12 | Accumulated QoS | PC5 QoS flow descriptions 11.3.5 | O | TLV | 6-65538 |
| 63 | RRC container | RRC container 11.2.14 | O | TLV | 3-257 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to "Multi-hop UE-to-network relay discovery response". |  |  |  |  |  |

Table 10.2.1.19: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery announcement for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.20: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 29 | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery solicitation for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.21: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for IP data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery response for IP data unit type”. |  |  |  |  |  |

Table 10.2.1.22: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery announcement for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Multi-hop UE-to-UE relay UE info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | M | LV-E | 2-65537 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model A". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery announcement for Ethernet and unstructured unit type”. |  |  |  |  |  |

Table 10.2.1.23: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
| 10 | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | O | TLV-E | 3-65538 |
| 29 | Target discoveree info | User info ID 11.2.7 | O | TV | 7 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery solicitation for Ethernet and unstructured data unit type”. |  |  |  |  |  |

Table 10.2.1.24: PROSE PC5 DISCOVERY message for multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ProSe direct discovery PC5 message type (NOTE 1) | ProSe direct discovery PC5 message type 11.2.1 | M | V | 1 |
|  | ProSe direct discovery PC5 message content type extensions (NOTE 2) | ProSe direct discovery PC5 message content type extensions 11.2.1a | M | V | 1 |
|  | UTC-based counter LSB | UTC-based counter LSB 11.2.11 | M | V | 1 |
|  | MIC | MIC 11.2.4 | M | V | 4 |
|  | Discoveree info | User info ID 11.2.7 | M | V | 6 |
|  | Discoverer info | User info ID 11.2.7 | M | V | 6 |
|  | Relay service code | Relay service code 11.2.8 | M | V | 3 |
|  | List of multi-hop UE-to-UE relay paths | List of multi-hop UE-to-UE relay paths 11.2.24 | M | LV-E | 2-65537 |
| NOTE 1: The discovery type is set to "Restricted discovery", the content type is set to "Extended content type" and the discovery model is set to "Model B". NOTE 2: The extended content type is set to “Multi-hop UE-to-UE relay discovery response for Ethernet and unstructured data unit type”. |  |  |  |  |  |

* * * Fourth Change * * * *

### 10.2.6	RRC container

The RRC container information element shall be included in:

a)	PROSE PC5 DISCOVERY message for UE-to-network relay discovery announcement as in table 10.2.1.8;

b)	PROSE PC5 DISCOVERY message for UE-to-network relay discovery response as in table 10.2.1.10;

c)	PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery announcement as in table 10.2.1.15; or

d)	PROSE PC5 DISCOVERY message for multi-hop UE-to-network relay discovery response as in table 10.2.1.18.

to indicate the RRC container information if it is received from the lower layers and the UE acts either as a 5G ProSe layer-2 UE-to-network relay UE for UE-to-network relay discovery, or as a 5G ProSe layer-2 multi-hop UE-to-network relay UE or a 5G ProSe layer-2 intermediate UE-to-network relay for multi-hop UE-to-network relay discovery.

* * * End of Changes * * * *