3GPP TSG-CT WG1 Meeting #157	C1-256808

Sophia-Antipolis, France, 13-17 October 2025

Source:	Samsung, InterDigital

Title:	Service operation on spatial maps notification

Spec:	3GPP TS 24.550 v1.1.0

Agenda item:	19.42

Document for:	Agreement

1. Introduction

This p-CR proposes spatial maps notification to 3GPP TS 24.550.

2. Reason for Change

In the current TS 24.550v1.1.0, the spatial map notification service operation is yet to be defined. This pCR defines service operation for spatial maps notification as per 3GPP TS 23.437, which enables the spatial map server to share the notification information about the spatial map(s), to those spatial map client(s) subscribed for these updates.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.550 v1.1.0.

* * * First Change * * * *

##### 5.3.1.2.1	Introduction

The service operation defined for SS_SmManagement API is shown in the table 5.3.1.2.1-1.

Table 5.3.1.2.1-1: Operations of the SS_SmManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SmManagement_Create | This service operation is used by the SM client to create the spatial map on SM server. | SM client, VAL Server |
| SS_SmManagement_Update | This service operation is used by the SM client to update the spatial map on SM server. | SM client |
| SS_SmManagement_Delete | This service operation is used by the SM client to delete the spatial map on SM server. | SM client |
| SS_SmManagement_Subscribe | This service operation is used by SM client to subscribe with the SM server to receive notifications related to spatial maps. | SM client |
| SS_SmManagement_UpdateSubscription | This service operation is used by SM client to update a subscription with the SM server to receive notifications related to spatial maps. | SM client |
| SS_SmManagement_Unsubscribe | This service operation is used by SM client to unsubscribe with the SM server to receive notifications related to spatial maps. | SM client |
| SS_SmManagement_Get | This service operation is used by SM client to retrieve to spatial map information from the SM server. | SM client |
| SS_SmManagement_Notify | This service operation is used by the spatial map server to notify the spatial map(s) related notifications to the spatial map client. | SM server |

* * * Next Change * * * *

##### 5.2.1.2.y	SS_SmManagement_Notify

###### 5.2.1.2.y.1	General

This service operation is used by the spatial map server to notify the spatial map(s) related notifications to the spatial map client.

###### 5.2.1.2.y.2	SM server notifying the spatial map notification to SM client

The SM server shall send an HTTP POST request to the SM client with the request URI set to the "{notifUri}" URI as specified in the clause 6.2.1.5.2.2. The body of the HTTP POST request shall include the "SpatialMapsNotif" data structure as specified in the clause 6.2.1.5.2.3.1.

Upon reception of the HTTP POST request from the SM server, the SM client shall:

a)	validate if there is an active spatial map subscription with SM server; in case of none the HTTP "406 Not Acceptable" response is sent;

b)	process the "SpatialMapsNotif" data structure as specified in clause 6.2.1.6.2.18 shared in the POST request body and send the HTTP "204 No Content" response to the SM server; and

c)	notify the data spatial map notification to the VAL client.

* * * End of Changes * * * *