3GPP TSG-CT WG1 Meeting #156	C1-255166

Goteberg, Sweden 25-29 August 2025	Revision of C1-254613

Source:	China Mobile

Title:	Pseudo-CR on the inventory not accept and abnormal cases at the device

Spec:	3GPP TS 24.369 v0.1.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR provides the requirement for the inventory not accept and abnormal cases at the device in the 3GPP TS 24.369 specification subclause 5.2.

2. Reason for Change

The requirement for the inventory not accept and abnormal cases at the device needs to be specified for the 3GPP TS 24.369 specification subclause 5.2.

The stage 2 requirement that “A permanently disabled AIoT Device does not respond to the Inventory Procedure.” needs to be considered as a not accepted case.

During the inventory procedure, if the authentication cannot be fufilled, it’s useless for the device to respond to the paging, because the NW will treat its report as invalid.

The device needs to handle the case#1(it’s moved from reader 1 area to reader 2 area), the case#2(both 2 inventory requests target it) in case the NW cannot avoid the collision issue.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v0.1.0.

* * * First Change * * * *

5.2.4	Inventory procedure not accepted by AIoT device

If the AIoT device has completed a disable command procedure, the AIoT device shall not respond to the AIoT paging message.

* * * Next Change * * * *

5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

- a)	Failure of authentication.

- If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [x], the AIoT device shall not respond to the AIoT paging message.

- b)	Collision of AIoT paging.

- If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall proceed with the ongoing inventory procedure and shall process the new AIoT paging after the completion of the inventory procedure.

* * * End of Changes * * * *