3GPP TSG-CT WG1 Meeting #156	C1-255174

Goteberg, Sweden 25-29 August 2025	Revision of C1-254619

Source:	China Mobile, SHARP

Title:	Pseudo-CR on Timers for the procedures

Spec:	3GPP TS 24.369 v0.1.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR proposes to consider the timers in the AIOTF used in the procedures in the 3GPP TS 24.369 specification.

This p-CR is related to the subclauses for inventory procedure initiation/complete, read/write/disable command procedure initiation/complete and abnormal cases in the AIOTF.

2. Reason for Change

The timers in the AIOTF used in the procedures needs to be discussed for the 3GPP TS 24.369 specification.

According to TS 23.369:

***

5.2.2.3	Permanent Disable Command

...

The Permanent Disable command is sent to an AIoT Device when an authorized AF uses the Permanent Disable command service operations as described in clause 5.2.2.1, or if the network determines to disable the AIoT Device.

...

***

The inventory procedure and command procedure could be triggered not only by the AF but also by the NW, the AIOTF needs an timer for the inventory procedure and command procedure to handle abnormal cases (e.g. no response from the AIoT device) and determines the target AIoT device is undiscovered in the target areas and the command is not accepted/fulfied by the device.

In addition, TS 24.501 defines 5GMM timer T3513 for paging, and timers not only for retransmission but also for state/context management, e.g.Implicit de-registration timer, Mobile reachable timer, Implementation specific timer for onboarding services.

It is suggested to define the timers for the procedures for abnormal case handling and determine the result of the procedures for device management by the NW.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v0.1.0.

* * * First Change * * * *

8	List of system parameters

8.1	General

This clause describes the definition of the timers for AIoT.

8.x	Timers of command procedure

Timer of command procedure are shown in table 8.3.

Table 8.3: Timers of command procedure – AIOTF side

| TIMER NUM. | TIMER VALUE | DEFINITION | CONDITION OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Tx | NOTE | Timer for the command procedures | Command procedure initiated | - Command procedure completed | - Abort the command procedure and consider the command procedure is unsuccessful. |
| NOTE: The value of this timer is network dependent. |  |  |  |  |  |

* * * End of Changes * * * *