3GPP TSG-CT WG1 Meeting #157	C1-256500

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4556 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for overhead reduction applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NORDAT_CP |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In CT1#156 the scope of NORDAT_CP work item was discussed to resolve the following Editor’s Note in the WID: Editor’s note: Whether the feature applies only to satellite access or to both satellite access and terrestrial access is FFS. It was agreed to remove the Editor’s Note without any related updates to limit the scope for the work item in EPS. Thus, the introduced functionality is applicable for both TN and NTN accesses. However, there are two sentences in 24.301 that informs of the intended use of EMM data transport procedure and service request procedure with overhead reduction over satellite access. Even if this is informative there is no corresponding normative limitation for type of access, it can result in incorrect interpretation for the use of the feature. It is therefore proposed to delete these informative sentences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Informative sentences not aligned to the scope of the WI are deleted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect interpretation for the applicability of the feature is possible which could restrict the feature being implemented and deployed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.1.2.3, 5.6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.6.1.2.3	UE is using EPS services with control plane CIoT EPS optimization with overhead reduction

The UE shall send a EMM TRANSPORT message, start T3417 and enter the state EMM-SERVICE-REQUEST-INITIATED.

For response to paging, sending user data, sending SMS message or sending LCS message, the UE may use EMM TRANSPORT message.

For case a in clause 5.6.1.1, the UE may include the Data container IE in the EMM TRANSPORT message.

For case b in clause 5.6.1.1:

-	if the UE has pending user data, the EPS bearer identity and optionally the DDX, provided by the ESM layer. that are to be sent via the control plane radio bearers, the UE shall include the user data in the data payload field, include the EPS bearer identity, set the DDX field, if present, of the Data container IE of the EMM TRANSPORT message, and set the data type field to "Control plane user data"; and

-	if the UE supports the CP-EDT (see 3GPP TS 36.300 [20]), the UE shall provide the EMM TRANSPORT message in the NAS request to the lower layer to establish an RRC connection as specified in clause 5.3.1.1.

For case c in clause 5.6.1.1, if the EMM TRANSPORT message is:

-	for sending SMS message, the UE shall include the SMS message in the data payload field and set the data type field to "SMS" of the Data container IE; and

-	for sending LCS message, the UE shall include the LCS message and the additional data, if any, in the data payload field, set the data type field to "Location services message container" and set the DDX field, if present, of the Data container IE.

* * * Next Change * * * *

#### 5.6.5.1	General

The purpose of the EMM data transport procedure is to provide a transport of:

a)	user data;

b)	SMS; and

c)	location services message,

in an encapsulated form between the MME and the UE. The procedure may be initiated by the UE or the network and can only be used when:

1)	the UE is attached for EPS services; and

2)	both the UE and the network have indicated the support of control plane CIoT EPS optimization with overhead reduction during the attach procedure or tracking area updating procedure.

The procedure may be initiated by the UE or the network when the UE is in EMM-CONNECTED mode.

* * * End of Changes * * * *