3GPP TSG-CT WG1 Meeting #158	C1-257563

Dallas, US , 17-21 November 2025

Source:	Samsung, China Mobile, SHARP, CEWiT, InterDigital, vivo

Title:	Pseudo-CR on AIoT Timer related to Write procedure

Spec:	3GPP TS 24.369 v1.1.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

Missing timer related texts in specification.

2. Reason for Change

This pCR captures the relevant AIOTF timer T2 which will be started during the WRITE COMMAND procedure. Corresponding timers have been agreed for READ and PERMANENT DISABLE COMMAND.

3. Conclusions

Adding relevant text missed out during merging.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.1.0.

* * * First Change * * * *

#### 5.3.3.2	Write command procedure initiation

The AIOTF initiates the write command procedure by sending a WRITE COMMAND message to the AIoT device and starting the timer T2. In the WRITE COMMAND message, the AIOTF:

a)	shall include the Write offset IE set to the offset from the start of the AIoT device's user memory indicating where to write the AIoT data; and

b)	shall include the AIoT data IE set to the AIoT data to be written.

* * * Next Change * * * *

#### 5.3.3.4	Write command procedure completion by the AIOTF

Upon receiving the WRITE COMPLETE message, the AIOTF shall consider the write command procedure as successfully completed and stop the timer T2.

* * * End of Changes * * * *