3GPP TSG-CT WG1 Meeting #157	C1-256530

# Sophia Antipolis, France, 13-17 October 2025	(was C1-256251)

Source:	Apple, InterDigital Inc.

Title:	Pseudo-CR on "Addition of status procedure"

Spec:	3GPP TS 24.369

Agenda item:	19.70

Document for:	Decision

1. Introduction

--

2. Reason for Change

Similar to existing NAS protocols a status procedure is missing for the AIoT NAS protocol.

3. Conclusions

A status procedure is introduced for:

1) unknown or unforeseen message type

2) non-semantical mandatory information element errors

If the read or write command is not supported by the AIoT device, the STATUS message shall be sent with a cause set to #97 "message type non-existent or not implemented".

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

#### 5.3.2.1	General

The purpose of the read command procedure is to retrieve AIoT data from the AIoT device (see example in figure 5.3.2.1.1). The support of the read command procedure is optional for the AIoT device.

AIoT device

AIOTF

READ COMMAND

READ COMPLETE

OR

READ COMMAND REJECT

Figure 5.3.2.1.1: Read command procedure

* * * Next Change * * * *

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

#### 5.3.3.1	General

The purpose of the write command procedure is to write the AIoT data to the AIoT device's user memory (see example in figure 5.3.3.1.1). The support of the write command procedure is optional for the AIoT device.

AIoT device

AIOTF

WRITE COMMAND

WRITE COMPLETE

OR

WRITE COMMAND REJECT

Figure 5.3.3.1.1: Write command procedure

* * * Next Change * * * *

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

* * * Next Change * * * *

## 5.4	Common procedures

### 5.4.x	Status procedure

#### 5.4.x.1	General

The purpose of the sending of the STATUS message is to report at any time certain error conditions detected upon receipt of Ambient IoT NAS message by the AIoT device (see example in figure 5.3.x.1.1).

Editor's note: Ciphering and integrity protection for STATUS is FFS.

AIoT device

AIOTF

STATUS

Figure 5.3.x.1.1: Status procedure

#### 5.3.x.2	Status procedure procedure initiation

The AIoT device initiates the status procedure by sending a STATUS message to the AIOTF.

#### 5.3.x.2	Status procedure procedure completion by the AIOTF

If the AIOTF receives a STATUS message the AIOTF shall take different actions depending on the received cause value:

#97	message type non-existent or not implemented.

The AIOTF shall abort any ongoing AIoT NAS procedure related to the AIoT device and stop any related timer.

The local actions to be taken by the AIOTF on receipt of a STATUS message with any other cause values are implementation dependent.

* * * Next Change * * * *

# 6	Handling of unknown, unforeseen, and erroneous protocol data

## 6.1	General

The procedures specified in the present document apply to those messages which pass the checks described in this clause.

This clause also specifies procedures for the handling of unknown, unforeseen, and erroneous protocol data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the protocols.

Clauses 6.2 to 6.7 shall be applied in order of precedence.

Detailed error handling procedures in the network are implementation dependent and may vary from network to network. However, when extensions of this protocol are developed, networks will be assumed to have the error handling that is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the network is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [3], clause 11.4.2.

Editor's note:	Whether AIoT NAS protocol supports conditional IEs and comprehension required IEs is FFS.

Editor's note:	Whether AIoT NAS protocol supports TLV-E format is FFS.

## 6.2	Message too short or too long

### 6.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, c.f. 3GPP TS 24.007 [3].

### 6.2.2	Message too long

Editor's note:	Maximum AIoT NAS message length is FFS and to be defined by RAN3.

## 6.3	Unknown or unforeseen message type

If the the network receives an AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall ignore the AIoT NAS message.

If the AIoT device receives an AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the receiver, it shall return a STATUS message with cause #97 "message type non-existent or not implemented".

NOTE:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

## 6.4	Non-semantical mandatory information element errors

### 6.4.1	Common procedures

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error;

is diagnosed or when a message containing a syntactically incorrect mandatory IE is received, the AIoT device shall:

-	if the message is not one of the messages listed in clause 6.4.2, ignore the message and return a STATUS message with cause #96 "invalid mandatory information"; and

the network shall either:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message.

### 6.4.2	Specific AIoT procedures

The following procedures shall apply when the AIoT device encounters an error with a mandatory information element in an AIoT NAS message:

a)	If the message is a READ COMMAND, then a READ COMMAND REJECT message with cause #96 "invalid mandatory information", shall be returned; and

b)	if the message is a WRITE COMMAND, then a WRITE COMMAND REJECT message with cause #96 "invalid mandatory information", shall be returned.

## 6.5	Unknown and unforeseen IEs in the non-imperative message part

### 6.5.1	IEIs unknown in the message

The AIoT device shall ignore all IEs with unknown IEI in a message.

The network shall take the same approach.

### 6.5.2	Out of sequence IEs

The AIoT device shall ignore all out of sequence IEs in a message.

The network should take the same approach.

### 6.5.3	Repeated IEs

If an information element with format T, TV or TLV is repeated in a message, the AIoT device shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element.

The network should follow the same procedures.

## 6.6	Non-imperative message part errors

### 6.6.1	General

This category includes syntactically incorrect optional IEs.

### 6.6.2	Syntactically incorrect optional IEs

The AIoT device shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The network shall take the same approach.

## 6.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the AIoT device shall perform the foreseen reactions of the procedural part of clause 5. If, however no such reactions are specified, the UE shall ignore the message.

The network should follow the same procedure.

* * * Next Change * * * *

### 7.1.x	STATUS

#### 7.1.x.1	Message definition

The STATUS message is sent by the AIoT device to the AIOTF to report certain error conditions listed in clause 6.

See table 7.1.x.1-1.

Message type:	STATUS

Significance:	dual

Direction:	AIoT device to AIOTF

Table 7.1.x.1-1: STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Status message identity | Message type 7.2.2 | M | V | 1 |
|  | Status cause | Cause 7.2.8 | M | V | 1 |

* * * Next Change * * * *

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |
| 0 | 0 | 0 | 0 | x | x | x | x |  | Status |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

* * * Next Change * * * *

### 7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

Editor's note:	the possible cause values are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Error, unspecified |
| All other values are spare and if received shall be interpreted as 0110 1111, "error, unspecified". |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

Annex A (informative):
Cause values

# A.1	Causes related to command procedure

Cause #x1 – TBD

This cause is used to indicate that the requested read or write command is not supported by the AIoT device.

Cause #x2 – TBD

This cause is used to indicate that the command type specific parameters (e.g. the offset to read application data from the AIoT device user memory, the length of the application data to read, the offset where to write the application data, or the length of the application data to write) is invalid.

Cause #x3 – TBD

This cause is used to indicate that the requested read or write command was not executed successfully.

Cause #x4 – TBD

This cause is used to indicate that the application data to be read cannot be found by the AIoT device in the requested place of the user memory.

Cause #x5 – TBD

This cause is used to indicate that the write command procedure cannot be implemented because the energy will be run out.

Cause #96 – Invalid mandatory information

This cause is used to indicate that the equipment sending this cause has received a message with a non-semantical mandatory IE error.

Cause #97 – Message type non-existent or not implemented

This cause is used to indicate that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this cause.

* * * End of Changes * * * *