3GPP TSG-CT WG1 Meeting #156	C1-255195

# Goteborg, Sweden, 25-29 August 2025	(was C1-254893)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4480 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Access technology utilization control IE encoding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the access technology utilization control information element for the case that only octet 3 is included, but octet 4 is not include, the UE behavior is not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to define that if octet 3 is included always also octet 4 is included, otherwise the receiver shall ignore the entire IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous UE behavior if only octet 3 is included in the access technology utilization control information element. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.9.3.3A	Access technology utilization control

The purpose of the access technology utilization control information element is for the network to communicate the restricted access technologies information to the UE or to indicate to the UE to remove the access technology utilization control information.

The access technology utilization control information element is coded as shown in figure 9.9.3.3A.1 and table 9.9.3.3A.1.

The access technology utilization control is a type 4 information element with a minimum length of 2 octets and a maximum length of 5 octets.

If octet 3 is included in the access technology utilization control information element, then octet 4 shall also be included. If octet 3 is included but octet 4 is not included, the receiver shall ignore the information element.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 |  |  |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Access technology utilization control IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of Access technology utilization control contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | 0 spare |  |  |  | 0 spare | Type of access technology utilization control |  |  |  | octet 3* |
| 0 spare | 0 spare |  | Sat-NG-RAN |  | Sat-E-UTRAN |  | NG-RAN |  |  | E-UTRAN |  | UTRAN |  | GERAN |  | octet 4* |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | 0 spare |  |  | 0 spare |  | 0 spare |  | 0 spare |  | octet 5* |

Figure 9.9.3.3A.1: Access technology utilization control information element

Table 9.9.3.3A.1: Access technology utilization control information element

| Type of access technology utilization control (octet 3, bit 1 to 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 2 |  | 1 |  |  |
| 0 |  | 0 |  | current PLMN |
| 0 |  | 1 |  | current PLMN and its equivalent PLMN(s) |
| 1 |  | 0 |  | Unused, shall be interpreted as "current PLMN" if received by the UE |
| 1 |  | 1 |  | Unused, shall be interpreted as "current PLMN" if received by the UE |
| If Type of access technology utilization control = “00”, octet 4 applies to the current PLMN. If Type of access technology utilization control = “01”, octet 4 applies to the current PLMN and PLMN(s) included the equivalent PLMNs list, if any. |  |  |  |  |
| GERAN (octet 4, bit 1) |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | GERAN is not restricted |
| 1 |  |  |  | GERAN is restricted |
| UTRAN (octet 4, bit 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | UTRAN is not restricted |
| 1 |  |  |  | UTRAN is restricted |
| E-UTRAN (octet 4, bit 3) |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | E-UTRAN is not restricted |
| 1 |  |  |  | E-UTRAN is restricted |
| NG-RAN (octet 4, bit 4) |  |  |  |  |
| Bit 4 |  |  |  |  |
| 0 |  |  |  | NG-RAN is not restricted |
| 1 |  |  |  | NG-RAN is restricted |
| Sat-E-UTRAN (octet 4, bit 5) |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  | satellite E-UTRAN is not restricted |  |
| 1 |  |  | satellite E-UTRAN is restricted |  |
| Sat-NG-RAN(octet 4, bit 6) |  |  |  |  |
| Bit 6 |  |  |  |  |
| 0 |  |  | satellite NG-RAN is not restricted |  |
| 1 |  |  | satellite NG-RAN is restricted |  |
| All other bits in octet 4 and 5 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |

* * * End of Changes * * * *