3GPP TSG-CT WG1 Meeting #157	C1-25xxxx

# Sophia Antipolis, France, 13-17 October 2025	(was C1-256586, C1-256264, C1-254633)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 6943 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE actions upon 5G-GUTI deletion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The 5G-GUTI can be deleted via Application Toolkit (USAT) while the UE is registered. However, a valid 5G-GUTI is required to establish an N1 signalling connection in 5GMM-REGISTERED. If the UE detects that the 5G-GUTI is deleted it needs to initiate an initial registration to get a valid 5G-GUTI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that in state 5GMM-REGISTERED, if the UE detects that the 5G-GUTI is deleted (e.g. due to a REFRESH command from the UICC as specified in 3GPP TS 31.111 [22A]), the UE shall set the 5GS update status to 5U2 NOT UPDATED (and shall store it according to subclause 5.1.3.2.2) and shall delete any last visited registered TAI, TAI list and ngKSI. The UE shall enter the state 5GMM-DEREGISTERED. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot establish an N1-signalling connection and is stuck in 5GMM-REGISTERED without any services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.2.2, 5.3.15A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.1.3.2.2	5GS update status in the UE

In order to describe the detailed UE behaviour, the 5GS update (5U) status pertaining to a specific subscriber is defined.

If the UE is not operating in SNPN access operation mode (see 3GPP TS 23.501 [8]), the 5GS update status is stored in a non-volatile memory in the USIM if the corresponding file is present in the USIM, else in the non-volatile memory in the ME, as described in annex C.

If the UE is operating in SNPN access operation mode, the 5GS update status for each SNPN whose SNPN identity is included in the "list of subscriber data" configured in the ME (see 3GPP TS 23.122 [5]) is stored in the non-volatile memory in the ME as described in annex C.

The 5GS update status value is changed only after the execution of a registration, network-initiated de-registration, primary authentication and key agreement, service request, paging procedure, due to a 5G-GUTI deletion in state 5GMM-REGISTERED (see clause 5.3.15A) or due to change in the current TAI which does not belong to the current registration area while T3346 is running.

5U1: UPDATED

The last registration attempt was successful.

5U2: NOT UPDATED

The last registration, network-initiated de-registration, or service request attempt failed procedurally, e.g. no response or reject message was received from the AMF.

5U3: ROAMING NOT ALLOWED

The last registration, service request, primary authentication and key agreement, or network-initiated de-registration attempt was correctly performed, but the answer from the AMF was negative (because of roaming or subscription restrictions).

* * * Next Change * * * *

### 5.3.15A	Abnormal cases in the UE

The following abnormal case can be identified:

a)	5G-GUTI deletion in state 5GMM-REGISTERED

In state 5GMM-REGISTERED, if the UE detects that the 5G-GUTI is deleted (e.g. due to a REFRESH command from the UICC as specified in 3GPP TS 31.111 [22A]) and the last successful registration was in N1 mode, the UE should set the 5GS update status to 5U2 NOT UPDATED (and shall store it according to subclause 5.1.3.2.2) and should delete any last visited registered TAI, TAI list and ngKSI. The UE should enter the state 5GMM-DEREGISTERED. The UE shall delete the stored 5G-GUTI in the UE, if any.

* * * End of Changes * * * *