3GPP TSG CT WG3 Meeting #142	C3-253334

# Goteborg, SE, 25th – 29th August, 2025

Source:	Huawei

Title:	Pseudo-CR on completing the definition of the OpenAPI description of the MLR_MLModelManagement API

Spec:	3GPP TS 29.482 (V1.0.0)

Agenda item:	19.41 (AIML_App)

Document for:	Agreement

1. Introduction

The stage 2 requirements for the new MLR model management API have been defined in clause 8.11 of TS 23.482.

2. Reason for Change

Define the OpenAPI description of this new API in the corresponding new AIMLE Services TS to complete the definition of this API.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V 1.0.0.

* * * Start of Change * * * *

# A.4	MLR_MLModelManagement API

openapi: 3.0.0

info:

title: AIMLE Server ML Model Management

version: 1.0.0-alpha.1

description: |

AIMLE Server ML Model Management Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 V1.1.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/mlr-mlmm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/storages:

get:

summary: Retrieve one or several existing Individual ML Models Storage resource(s).

operationId: GetMLModelsStorages

tags:

- ML Models Storages (Collection)

parameters:

- name: storage-ids

in: query

description: Contains the identifier(s) of the targeted ML Models Storage resource(s).

required: false

schema:

type: array

items:

type: string

minItems: 1

- name: profile-ids

in: query

description: Contains identifier(s) of the profile(s) of the targeted stored ML Model(s).

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: supp-feats

in: query

description: >

Contains the list of supported features among the ones defined in clause 6.2.1.8.

This query parameter shall be present only when feature negotiation is required.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested Individual ML Models Storage resource(s) shall be returned.

If there are no available Individual ML Models Storage resource(s) fulfilling the

request, an empty array shall be returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MLModelsStorage'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a ML Models Storage.

operationId: CreateMLModelsStorage

tags:

- ML Models Storages (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

responses:

'201':

description: >

Created. The ML Models Storage is successfully created and a representation of the

created Individual ML Models Storage resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

headers:

Location:

description: >

Contains the URI of the created Individual ML Models Storage resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/storages/{storageId}:

parameters:

- name: storageId

in: path

description: >

Represents the identifier of the Individual ML Models Storage resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual ML Models Storage resource.

operationId: GetIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

responses:

'200':

description: >

OK. The requested Individual ML Models Storage resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual ML Models Storage resource.

operationId: UpdateIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

responses:

'200':

description: >

OK. The Individual ML Models Storage resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

'204':

description: >

No Content. The Individual ML Models Storage resource is successfully updated and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual ML Models Storage resource.

operationId: ModifyIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/MLModelsStoragePatch'

responses:

'200':

description: >

OK. The Individual ML Models Storage resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/MLModelsStorage'

'204':

description: >

No Content. The Individual ML Models Storage resource is successfully modified and no

content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual ML Models Storage resource.

operationId: DeleteIndMLModelsStorage

tags:

- Individual ML Models Storage (Document)

responses:

'204':

description: >

No Content. The Individual ML Models Storage resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

MLModelsStorage:

description: >

Represents a ML Models Storage.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelProfile'

minItems: 1

mlModelsAddresses:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

anyOf:

- required: [mlModels]

- required: [mlModelsAddresses]

MLModelProfile:

description: >

Represents the ML Model Profile.

type: object

properties:

mlModelProfId:

type: string

aimleServId:

type: string

aimleRepId:

type: string

mlModelInfo:

$ref: '#/components/schemas/MLModel'

mlModelUri:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

required:

- mlModelInfo

MLModel:

description: >

Represents a ML Model.

type: object

properties:

mlModelId:

type: string

adaeAnalyticsId:

type: string

mlModelSize:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

mlModelSrcId:

type: string

valServiceIds:

type: array

items:

type: string

minItems: 1

domain:

$ref: '#/components/schemas/MLModelDomain'

customDomain:

type: string

vendors:

type: array

items:

type: string

minItems: 1

interopInfo:

type: string

phaseInfo:

$ref: '#/components/schemas/MLModelPhaseInfo'

storeDiscReqs:

$ref: '#/components/schemas/MLModelStoreDiscReqs'

usageReqs:

$ref: '#/components/schemas/MLModelUsageReqs'

required:

- mlModelId

anyOf:

- required: [vendors]

- required: [interopInfo]

- required: [phaseInfo]

- required: [storeDiscReqs]

MLModelPhaseInfo:

description: >

Represents the ML Model phase related information.

type: object

properties:

phase:

$ref: '#/components/schemas/MLModelPhase'

observedPerf:

$ref: '#/components/schemas/MLModelPerf'

trainingInfo:

$ref: '#/components/schemas/MLModelTrainingInfo'

contTrainInd:

type: boolean

default: false

description: >

Indicates whether the ML Model can be trained continuously or not.

true indicates that the ML Model can be trained continuously.

false indicates that the ML Model cannot be trained continuously.

The default value is false when this attribute is omitted.

contTrainParams:

type: string

required:

- phase

MLModelStoreDiscReqs:

description: >

Represents the ML Model storage and discovery requirements.

type: object

properties:

duration:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

accessReqs:

$ref: '#/components/schemas/MLModelAccessReqs'

anyOf:

- required: [duration]

- required: [accessReqs]

MLModelUsageReqs:

description: >

Represents the ML Model usage requirements.

type: object

properties:

usages:

type: array

items:

$ref: '#/components/schemas/MLModelUsage'

minItems: 1

required:

- usages

MLModelPerf:

description: >

Represents the performance information of the ML Model.

type: object

properties:

resourceAddr:

type: integer

minimum: 0

maximum: 100

appSpecPerfData:

type: string

anyOf:

- required: [resourceAddr]

- required: [appSpecPerfData]

MLModelTrainingInfo:

description: >

Represents the ML Model training information.

type: object

properties:

dataSources:

type: string

dataVolume:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

freshness:

$ref: '#/components/schemas/DataFreshness'

baseModelId:

type: string

anyOf:

- required: [dataSources]

- required: [dataVolume]

- required: [freshness]

- required: [baseModelId]

MLModelAccessReqs:

description: >

Represents ML Model access requirements.

type: object

properties:

accessReqs:

type: array

items:

$ref: '#/components/schemas/AccessReqs'

minItems: 1

valServerIds:

type: array

items:

type: string

minItems: 1

valClientIds:

type: array

items:

type: string

minItems: 1

timePeriod:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- accessReqs

MLModelsStoragePatch:

description: >

Represents the requested modifications to a ML Models Storage.

type: object

properties:

mlModels:

type: array

items:

$ref: '#/components/schemas/MLModelProfile'

minItems: 1

mlModelsAddresses:

type: array

items:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

minItems: 1

#

# Simple data types

#

#

# ENUMERATIONS

#

MLModelDomain:

anyOf:

- type: string

enum:

- SPEECH_RECOGNITION

- IMAGE_RECOGNITION

- IMAGE_PROCESSING

- LOCATION_PREDICTION

- CUSTOM

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the domain of the ML Model.

Possible values are:

- SPEECH_RECOGNITION: Indicates that the domain of the ML Model is speech recognition.

- IMAGE_RECOGNITION: Indicates that the domain of the ML Model is image recognition.

- IMAGE_PROCESSING: Indicates that the domain of the ML Model is image processing.

- LOCATION_PREDICTION: Indicates that the domain of the ML Model is location prediction.

- CUSTOM: Indicates that the domain of the ML Model is a custom domain.

MLModelPhase:

anyOf:

- type: string

enum:

- NOT_TRAINED

- IN_TRAINING

- TRAINED

- IN_RETRAINING

- DEPLOYED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ML Model phase.

Possible values are:

- NOT_TRAINED: Indicates that the ML Model phase is that ML Model is not trained.

- IN_TRAINING: Indicates that the ML Model phase is that the ML Model is in training.

- TRAINED: Indicates that the ML Model phase is that the ML Model is trained.

- IN_RETRAINING: Indicates that the ML Model phase is that the ML Model is in re-training.

- DEPLOYED: Indicates that the ML Model phase is that the ML Model is deployed.

MLModelUsage:

anyOf:

- type: string

enum:

- TRAINING

- INFERENCE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ML Model usage.

Possible values are:

- TRAINING: Indicates that the ML Model usage is training.

- INFERENCE: Indicates that the ML Model usage is inference.

AccessReqs:

anyOf:

- type: string

enum:

- PUBLICLY_AVAILABLE

- RESTRICTED

- PRIVATE_USE_ONLY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the ML Model access requirements.

Possible values are:

- PUBLICLY_AVAILABLE: Indicates that the ML Model is publicly available.

- RESTRICTED: Indicates that the ML Model is restricted.

- PRIVATE_USE_ONLY: Indicates that the ML Model is for private use only.

DataFreshness:

anyOf:

- type: string

enum:

- REAL_TIME

- NEAR_REAL_TIME

- HOURLY

- DAILY

- WEEKLY

- MONTHLY

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the data freshness level.

Possible values are:

- REAL_TIME: Indicates that the data freshness level is real-time.

- NEAR_REAL_TIME: Indicates that the data freshness level is near real-time.

- HOURLY: Indicates that the data freshness level is hourly.

- DAILY: Indicates that the data freshness level is daily.

- WEEKLY: Indicates that the data freshness level is weekly.

- MONTHLY: Indicates that the data freshness level is monthly.

# Data types describing alternative data types or combinations of data types:

#

* * * * End of Changes * * * *