3GPP TSG CT WG3 Meeting #143	C3-254018

Sophia-Antipolis, France, 13 - 17 October, 2025

3GPP TSG-CT WG1 Meeting #156	C1-255326

# Goteberg, Sweden 25-29 August 2025

Title:	Reply on LS to 3GPP CT1 and CT3 on Reserved QoS Rule Precedence Values

Response to:	LS to 3GPP CT1 and CT3 on Reserved QoS Rule Precedence Values (C1-254600)

Release:	Release 19

Work Item:	-

Source:	CT1

To:	GSMA Terminal Steering Group, CT3

Cc:

Contact Person:

Name:	Yizhong Zhang

Tel. Number:	-

E-mail Address:	yizhong.zhang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1. Overall Description:

CT1 thanks GSMA’s Terminal Steering Group (TSG) for their LS on Reserved QoS Rule Precedence Values (C1-254600).

According to TS 24.501:

a) When creating a new derived QoS rule by reflective QoS in the UE, the UE locally sets the precedence value of the new derived QoS rule to 80 (decimal), cited below:

###### 6.2.5.1.4.3	Creating a derived QoS rule by reflective QoS in the UE

If the UE receives a DL user data packet marked with a QFI and an RQI, the DL user data packet belongs to a PDU session of IPv4, IPv6, IPv4v6 or Ethernet PDU session type, and the UE does not have a derived QoS rule with the same packet filter for UL direction as the packet filter for UL direction derived from the DL user data packet as specified in subclause 6.2.5.1.4.2, then the UE shall create a new derived QoS rule as follows:

a)	the QFI of the derived QoS rule is set to the received QFI;

b)	the precedence value of the derived QoS rule is set to 80 (decimal); and

c)	the packet filter for UL direction of the derived QoS rule is set to the derived packet filter for UL direction;

b) The codepoint for the precedence value 80 (decimal) is defined as 'reserved' value for signalled QoS rule, cited below:

###### 9.11.4.13	QoS rules

Table 9.11.4.13.1: QoS rules information element

QoS rule identifier (octet 4) … QoS rule precedence (octet m+1) The QoS rule precedence field is used to specify the precedence of the QoS rule among all QoS rules (both the signalled QoS rules as described in subclause 6.2.5.1.1.2 and the derived QoS rules as described in subclause 6.2.5.1.1.3) associated with the PDU session of the QoS flow. This field includes the binary coded value of the QoS rule precedence in the range from 0 to 255 (decimal). The higher the value of the QoS rule precedence field, the lower the precedence of that QoS rule is. For the "delete existing QoS rule" operation, the QoS rule precedence value field shall not be included. For the "create new QoS rule" operation, the QoS rule precedence value field shall be included. The value 80 (decimal) is reserved. …

From CT1 perspective,

- Precedence value 80 (decimal) is only used for a derived QoS rule. Derived QoS rule is used only when UE and network support reflective QoS. The derived QoS rule is only used locally in the UE and will not be included in any NAS signalling exchanged between UE and network.

- For signalled QoS rules, CT1 decided to define the codepoint for precedence value 80 (decimal) as 'reserved' since Rel-15, to prevent the collision cases where the UE has a derived QoS rule with precedence value 80 (decimal) and the network sending a signalled QoS rule with the precedence value 80 (decimal). This means the precedence value 80 (decimal) cannot be used for any signalled QoS rule, regardless whether reflective QoS is used or not.

- Upon receipt of a 'reserved' value, the receiving entity (UE or network) will immediately trigger the error handling.

Hence the observed UE behaviour is as intended, and there is no need for CT1 to update TS 24.501.

CT1 kindly asks GSMA’s Terminal Steering Group (TSG) and CT3 to take above information into account, and kindly asks CT3 to update their specifications if necessary.

2. Actions:

To GSMA’s Terminal Steering Group (TSG).

ACTION: 	CT1 kindly asks GSMA’s Terminal Steering Group (TSG) to take above information into account.

To CT3.

ACTION: 	CT1 kindly asks CT3 to take above information into account and update their specifications if necessary.

3. Date of Next CT1 Meetings:

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2026..................India