3GPP TSG-CT WG3 Meeting #143	C3-254349

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Ericsson

Title:	Discussion on the transform name for QUIC-aware proxying using HTTP

Document for:	Discussion & Agreement

# Agenda Item:	19.48

# Work Item / Release:	XRM_Ph2/Rel-19

Abstract of the contribution:

# 1	Decision/action requested

Discuss the proposal on the transform name for QUIC-aware proxying using HTTP

# 2	References

[1]	3GPP TS 33.501 "Security architecture and procedures for 5G system" v19.4.0

[2]	3GPP TS 29.561 "5G System; Interworking between 5G Network and external Data Networks;" v19.3.0

# 3	Discussion

The QUIC-aware proxying using HTTP method to realize the transport of XRM MRI information is jointly specified by 3GPP CT3 in 3GPP TS 29.561 [2] (Message format) and by 3GPP SA3 in 3GPP TS 33.501 [1] (Securing the format). The IANA registry for transform identification using string label, these labels are also used to negotiate the used transform if multiple transforms are supported.

QUIC-aware proxying’s mechanism for negotiation is realized using a one-dimensional preference ordered list, with the most preferred transform being listed first in a parameter “accept-transform” in the HTTP header "Proxy-QUIC-Forwarding". Note that including the transform in the parameter in this HTTP header is required 3GPP TS 33.501 [1].

The QUIC-aware proxying using HTTP method (clause 22.3.4 of TS 29.561 [2]) to carry MRI is dependent on that one also implements the Connect-UDP based proxying UDP in HTTP Datagram method implemented as described in clause 22.3.2 of TS 29.561 [2]. Thus, one will include the 3gpp-Connect-Req-Inband-Info to indicate purpose of MRI and register the context-ID used when not doing QUIC-Aware Proxying and instead forwarding End-to-End packets within the HTTP/3 connection and its security envelope.

When QUIC-aware proxying using HTTP method, the negotiation of the method and supported transform are indicated in the IETF specification defined header “Proxy-QUIC-Forwarding” and parameter “accept-transform”. As both the transform format and by SA3 specified protection method using AES CCM with truncated 8-byte authentication tag need to be supported that any extensions when it comes to the format or more likely the supported security cipher of the transform would be negotiated by defining a new transform label.

The reasons for defining a new transform rather than new headers or parameters are the following:

- Avoid having the information necessary to negotiate the preferred working point spread over multiple places, such as indicating the actual cipher in one header and the usage of the transform format in another:

  - This requires cross referencing another header or parameter to determine if one support the necessary cipher before accepting the QUIC-Aware proxying.

  - Forcing cross-referencing may make implementation more complex or make usage of some open-source implementations difficult.

  - Needs additional definition of how to interpret preference between transform label and the 3GPP Defined header’s parameter.

- Transform labels are practically unlimited so no reasons exist to avoid using new ones for each operation point that might be needed in the future.

Based on above analysis for discussions, it appears simplest to just use the mechanism that exists and use the label that 3GPP SA3 already defined in 3GPP TS 33.501 [1].

# 4	Proposal

Propose to discuss and agree Ericsson submitted TS 29.561 CR IANA registration for MRI packet transforms in CT3#143 which is based on the revision CR C3-253087 r2 discussed on CT3#142.