# 3GPP TSG CT WG3 Meeting #143	C3-254565

# Sophia-Antipolis, France, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1738 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS Session Management notification correlation Id |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Today ImsSessionManagement API is allowing to define at two different levels the notification URI used by the DC AS (AF) when creating an IMS Session monitoring, one inside ImsSession data structure and another one inside imsSessionInfo attribute. The notification URI and correlation Id included as part of imsSessionInfo attribute were defined in CT4 TS 29.175 for IMS AS to notify NEF. These attributes shall not be used by DC AS (AF) to avoid NEF implementations use DC AS (AF) attribute values. Actually, direct notification from IMS AS to AF is not required by stage 2 in 23.228. In order to facilitate the differentiation of the notifications sent by IMS AS to NEF from the ones sent by NEF to DC AS (AF) and to explain how to use the notification URIs defined at different levels, the following is proposed: - Include a notification correlation identifier at ImsSession level - Clarify the notification URI and correlation Id parameters defined by TS 29.175 in ImSessionInfo are not used by DC AS (AF) when creating the IMS Session monitoring in NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update ImsSession data structure to include a notification correlation identifier, and clarify attributes inside ImsSessionInfo are not used by AF. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible interoperability problems if attributes at different levels are used in different ways. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.42.5.2.2, A.40 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29522_ImsSessionManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 5.42.5.2.2	Type: ImsSession

Table 5.42.5.2.2-1: Definition of type ImsSession

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| afId | string | M | 1 | Contains the identifier of the AF that is sending the request. |  |
| imsSessionInfo | ImsSessionInfo | M | 1 | Contains the IMS session information. (NOTE). |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which IMS Session notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 5.42.6. This attribute shall be present only when feature negotiation is required. |  |
| notifCorrelationId | string | O | 0..1 | Contains the Notification Correlation ID assigned by the AF. |  |
| NOTE: The "notifUri" and "notifCorrelationId" attributes inside ImsSessionInfo data type are not applicable for "imsSessionInfo" attribute. |  |  |  |  |  |

# *** 2nd Change ***

# A.40	ImsSessionManagement API

openapi: 3.0.0

info:

title: 3gpp-ims-sm

version: 1.0.0-alpha.2

description: |

API for IMS Session Management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-ims-sm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/ims-sessions:

get:

summary: Request to retrieve all the active IMS Sessions managed by the NEF.

operationId: GetImsSessions

tags:

- IMS Sessions (Collection)

responses:

'200':

description: >

OK. All the active IMS Sessions resources managed by the NEF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ImsSession'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request to create a new IMS Session resource.

tags:

- IMS Session Creation (Collection)

operationId: CreateImsSession

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

responses:

'201':

description: >

Created. Successful creation of a new Individual IMS Session resource.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/3gpp-ims-sm/v1/ims-sessions/{sessionId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ImsSessionNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29175_Nimsas_ImsSessionManagement.yaml#/components/schemas/ImsSessionEventNotification'

responses:

'204':

description: >

No Content. The IMS EE notification is successfully received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/ims-sessions/{sessionId}:

parameters:

- name: sessionId

in: path

description: >

Represents the identifier of the Individual IMS Session resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual IMS Session resource.

operationId: GetIndImsSession

tags:

- Individual IMS Session (Document)

responses:

'200':

description: >

OK. The requested "Individual IMS Session" resource is successfully returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual IMS Session resource.

operationId: UpdateIndImsSession

tags:

- Individual IMS Session (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

responses:

'200':

description: >

OK. The Individual IMS Session resource is successfully updated and a representation

of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSession'

'204':

description: >

No Content. The Individual IMS Session Management resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual IMS Session resource.

operationId: DeleteIndImsSession

tags:

- Individual IMS Session (Document)

responses:

'204':

description: >

No Content. The Individual IMS Session resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29122_CommonData.yaml#/components/responses/409'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

ImsSession:

description: Represents the IMS Session Information.

type: object

properties:

afId:

type: string

imsSessionInfo:

$ref: 'TS29175_Nimsas_ImsSessionManagement.yaml#/components/schemas/ImsSessionInfo'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

notifCorrelationId:
          type: string

required:

- afId

- imsSessionInfo

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

# *** End of Changes ***