3GPP TSG CT WG3 Meeting #144	C3-255504

# Dallas, USA, 17th – 21st November, 2025

Source:	Nokia

Title:	General corrections across AF VFL APIs

Spec:	3GPP TS 29.530

Agenda item:	19.39 (AIML_CN)

Document for:	Agreement

1. Introduction

There are various things to be completed or corrected in the descriptions of the VFL-related APIs in TS 29.530, including interface names, small ENs, wrong NF names, and more.

2. Reason for Change

Correct and complete the descriptions of TS 29.530.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.530 V 1.1.0.

* * * * First change * * * *

#### 5.3.1.2	Service Architecture

The 5G System Architecture is defined in 3GPP TS 23.501 [2]. The Network Data Analytics Exposure architecture is defined in 3GPP TS 23.288 [14]. The VFL signalling flows are defined in 3GPP TS 29.552 [15].

The Naf_VFLInference service is part of the Naf service-based interface exhibited by the trusted Application Function (AF) or untrusted Application Function (AF).

Known consumers of the Naf_VFLInference service are:

-	Network Data Analytics Function (NWDAF) when the AF is trusted.

-	Network Exposure Function (NEF) when the AF is untrusted.

Figure 5.3.1.2-1: Reference Architecture for the Naf_VFLInference service; SBI representation

Figure 5.3.1.2-2: Reference Architecture for the Naf_VFLinference service: reference point representation

* * * * Next change * * * *

##### 5.3.1.3.1	Application Function (AF)

The Application Function (AF), provides VFL inference for different analytics events to NF service consumers.

The Application Function (AF) allows NF service consumers to subscribe to and unsubscribe from one-time, periodic notification or notification when a VFL inference event is detected.

* * * * Next change * * * *

##### 5.4.2.2.2	Training Subscription Creation

Figure 5.4.2.2.2-1 depicts a scenario where an NF service consumer sends a request to the AF to request the creation of a Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.2.2-1: Procedure for Training Subscription Creation

1.	In order to subscribe to Training, the NF service consumer shall send an HTTP POST request to the AF targeting the URI of the "Training Subscriptions" collection resource, with the request body including the TrainEventsSubsc data structure.

2a.	Upon success, the AF shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Training Subscription" resource within the TrainEventsSubsc data structure, and an HTTP "Location" header field containing the URI of the created resource.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

* * * * Next change * * * *

##### 5.4.2.2.3	Training Subscription Update

Figure 5.4.2.2.3-1 depicts a scenario where an NF service consumer sends a request to the AF to request the update of an existing Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.2.3-1: Procedure for Training Subscription Update

1.	In order to request the update of an existing Training Subscription, the NF service consumer shall send an HTTP PUT/PATCH request to the AF, targeting the URI of the corresponding "Individual Training Subscription" resource, with the request body including either:

-	the updated representation of the resource within the TrainEventsSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the TrainEventsSubscPatch data structure, in case the HTTP PATCH method is used.

2a.	Upon success, the AF shall update the targeted "Individual Training Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Training Subscription" resource within the TrainEventsSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.3.7.

* * * * Next change * * * *

##### 5.4.2.3.2	Training Subscription Deletion

Figure 5.4.2.3.2-1 depicts a scenario where an NF service consumer sends a request to the AF to delete an existing Training Subscription (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.3.2-1: Procedure for Training Subscription Deletion

1.	In order to request the deletion of an existing Training Subscription, the NF service consumer shall send an HTTP DELETE request to the AF targeting the URI of the corresponding "Individual Training Subscription" resource.

2a.	Upon success, the AF shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.3.7.

* * * * Next change * * * *

##### 5.4.2.4.2	Training Notification

Figure 5.4.2.4.2-1 depicts a scenario where the AF sends a request to notify a previously subscribed service consumer on Training report(s) (see also clause 6.2H of 3GPP°TS°23.288°[14]).

Figure 5.4.2.4.2-1: Procedure for Training Notification

1.	In order to notify a previously subscribed service consumer on Training report(s), the AF shall send an HTTP POST request to the NF service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the NF service consumer during the creation/update of the corresponding Training Subscription using the procedures defined in clauses 5.4.2.2, and the request body including the TrainEventsNotif data structure.

2a.	Upon success, the NF service consumer shall respond to the AF with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

* * * * Next change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Naf_VFLInference service-based interface protocol.

Table 6.2.6.1-1: Naf_VFLInference specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| VflInferSub | 6.2.6.2.2 | Represents a VFL inference subscription. |  |
| VflInferSubPatch | 6.2.6.2.3 | Represents parameters to request the modification of a VFL inference subscription. |  |

Table 6.2.6.1-2 specifies data types re-used by the Naf_VFLInference service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_VFLInference service-based interface.

Table 6.2.6.1-2: Naf_VFLInference re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ReportingInformation | 3GPP TS 29.523 [17] | Represents the type of reporting a subscription requires. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Contains redirection related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported features. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |
| VflInferAnaSub | 3GPP TS 29.520 [18] | Represents the VFL inference subscription information for the subscribed analytics ID. |  |
| VflInferNotif | 3GPP TS 29.520 [18] | Represents notification of a VFL inference subscription. |  |
| VflInferReq | 3GPP TS 29.520 [18] | Represents requirements for VFL inference. |  |
| VflInferResult | 3GPP TS 29.520 [18] | Represents intermediate VFL inference result per target. |  |

* * * * End of changes * * * *