3GPP TSG-CT WG3 Meeting #144	C3-255352

# Dallas, USA, 17 - 21 November, 2025													(was_C3-255167)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.552 | CR | 0172 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of VFL Training procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Ericsson, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 5.10.3.3, the following EN for training procedure remains. Editor's Note: The training procedures are FFS as the corresponding API(s) are not defined by stage-3 yet. It is proposed to resolve this EN. The Nnef_VFLTraining_Subscribe defined in clause 4.8.2.2 and 4.8.2.4 in TS 29.591 is proposed as TS 29.591 CR#0258 The Nnef_Training_Subscribe service defined in clause 4.11.2.2 and 4.11.2.4 in TS 29.591 is proposed as TS 29.591 CR#0260 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add VFL training procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.3.3, 5.10.3.3.1(new), 5.10.3.3.2(new), 5.10.3.3.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev1: -NF consumer -> 5GC NF -New NOTE: When the trusted AF or untrusted AF is acting as VFL server, the VFL client can only be the NWDAF. -Simplify step 4 without details. -typo correction: inference -> training -API name correction -correction arrow direction for step 5c-4 in the figure -add 29.510 for VFL discovery -additional supporting companies |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.10.3.3	Training procedures

5.10.3.3.1	General

The training procedures are triggered by analytics consumer, NWDAF containing AnLF, or NWDAF containing MTLF. The VFL server (i.e., NWDAF, trusted AF, or untrusted AF) is determined based on the clause 5.3.2.2 of 3GPP TS 29.510 [23] or the clause 4.4.52.2 of 3GPP TS 29.522 [10]. The VFL server repeats several rounds of VFL training process among multiple VFL clients until the VFL training process converges.

The training procedures in this clause include the following cases:

-	subclause 5.10.3.3.2 specifies the training procedure when NWDAF or trusted AF is acting as VFL server while VFL client(s) can be NWDAF, AF, and/or untrusted AF.

-	subclause 5.10.3.3.3 specifies the training procedure when untrusted AF is acting VFL server while VFL client(s) can be NWDAF.

5.10.3.3.2	Training procedure for vertical federated learning when NWDAF or trusted AF is acting as VFL server

The training procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF, NWDAF containing AnLF, or NWDAF containing MTLF. This procedure is used by the NWDAF acting as VFL server to trigger training procedure for Vertical Federated Learning among multiple NWDAF instances and/or AF(s) acting as VFL client.

Figure 5.10.3.3.2-1: Training procedure for Vertical Federated Learning when NWDAF or Trusted AF is acting as VFL server

1.	The NWDAF service consumer (e.g. 5GC NF, NWDAF containing AnLF, NWDAF containing MTLF) determines the NWDAF or trusted AF acting as an VFL server.

If trusted AF is selected as VFL server, the NWDAF service consumer invokes the Naf_Training_Subscribe service operation to VFL server NWDAF as described in clause 5.4.2.2 of 3GPP TS 29.530 [53].

If NWDAF is selected as VFL server and the NWDAF service consumer is 5GC NF, the 5GC NF invokes Nnwdaf_EventsSubscription_Subscribe service operation to VFL server NWDAF as described in clause 4.2.2.2 of 3GPP TS 29.520 [5]. If NWDAF is selected as VFL server and the NWDAF service consumer is NWDAF containing AnLF, the NWDAF containing AnLF invokes the Nnwdaf_MLModelProvision_Subscribe service operation to VFL server NWDAF as described in clause 4.5.2.2 of 3GPP TS 29.520 [5].

2.	The NWDAF or trusted AF acting as VFL server selects VFL client (s) as described in clause 5.3.2.2 of 3GPP TS 29.510 [23]. The NWDAF or trusted AF acting as VFL server performs the VFL preparation as described in clause 5.10.3.2.

3.	The NWDAF or trusted AF acting as VFL server sends the VFL training request to each VFL client.

NOTE 1:	When the trusted AF or untrusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3a. If the VFL client is another instance of NWDAF, the NWDAF or trusted AF acting as VFL server invokes Nnwdaf_VFLTraining_Subscribe service operation as described in clause 4.10.2.2.2 of 3GPP TS 29.520 [5]. If the Subscription is successfully processed and accepted, the VFL client sends Nnwdaf_VFLTraining_Subscribe Response message as described in clause 4.10.2.2.2 of 3GPP TS 29.520 [5].

3b.	If the VFL client is a trusted AF, the NWDAF acting as VFL server invokes Naf_VFLTraining_Subscribe service operation as described in clause 5.2.2.2.2 of 3GPP TS 29.530 [53]. If the Subscription is successfully processed and accepted, VFL client sends Naf_VFLTraining_Subscribe Response message as described in clause 5.2.2.2.2 of 3GPP TS 29.530 [53].

3c. If the VFL client is an untrusted AF, the NWDAF acting as VFL server invokes Nnef_VFLTraining_Subscribe service operation. The NEF invokes Naf_VFLTraining_Subscribe by sending an HTTP POST request as described in clause 5.2.2.2.2 of 3GPP TS 29.530 [53]. If the Subscription is successfully processed and accepted, VFL client sends Naf_VFLTraining_Subscribe response message as described in clause 5.2.2.2.2 of 3GPP TS 29.530 [53]. The NEF sends Nnef_VFLTraining_Subscribe response message to VFL server as described in clause 4.8.2.2 of 3GPP TS 29.591 [11].

4.	Each VFL client collects its local data and trains the corresponding local ML model.

5.	Each VFL client sends the ML model training information to the VFL server.

5a.	If the VFL client is an NWDAF, then it sends the response to the VFL server by invoking Nnwdaf_VFLTraining_Notify service operation as described in clause 4.10.2.4.2 of 3GPP TS 29.520 [5].

5b.	If the VFL client is a trusted AF, then it sends the response to the VFL server by invoking Naf_VFLTraining_Notify service operation as described in clause 5.2.2.4 of 3GPP TS 29.530 [53].

5c.	If the VFL client is an untrusted AF, then it sends the response to the VFL server to NEF by invoking Naf_VFLTraining_Notify service operation as described in clause 5.2.2.4 of 3GPP TS 29.530 [53]. For each untrusted AF VFL client, the NEF converts any external identifiers to internal identifiers and the NEF invoking Nnef_VFLTraining_Notify service operation as described in clause 4.8.2.4 of 3GPP TS 29.591 [11].

6.	The VFL server combines all the ML model training information to update the global ML model. The VFL server may also calculate and evaluate the global model metric.

7.	The VFL server sends the ML training information report to the consumer in step 1.

If the NWDAF is the VFL server in step 1 and the NWDAF service consumer is 5GC NF, the VFL server NWDAF invokes Nnwdaf_EventsSubscription_Notify service operation to 5GC NF as described in clause 4.2.2.4 of 3GPP TS 29.520 [5]. The 5GC NF responds to the Nnwdaf_EventsSubscription_Notify service operation as described in clause 4.2.2.4 of 3GPP TS 29.520 [5].

If the NWDAF is the VFL server in step 1 and the NWDAF service consumer is NWDAF containing AnLF, the VFL server NWDAF invokes Nnwdaf_MLModelProvision_Subscribe service operation to NWDAF containing AnLF as described in clause 4.5.2.4 of 3GPP TS 29.520 [5]. The 5GC NF responds to the Nnwdaf_MLModelProvision_Subscribe service operation as described in clause 4.5.2.4 of 3GPP TS 29.520 [5].

If the trusted AF is VFL server in step 1, the VFL server NWDAF invokes Naf_Training_Notify service operation to the NWDAF service consumer as described in clause 5.4.2.4 of 3GPP TS 29.530 [53]. The NWDAF service consumer responds to the Naf_Training_Notify service operation as described in clause 5.4.2.4 of 3GPP TS 29.530 [53].

8.	Same as step 1.

9.	The VFL Server NWDAF updates or terminates the current VFL training process.

NOTE 2:	The steps 3-9 can be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

10.	If the VFL server decides to terminate the VFL training process, the NWDAF or trusted AF acting as VFL server sends the VFL training unsubscription request to each VFL client.

10a. If the VFL client is an NWDAF, the NWDAF or trusted AF acting as VFL server invokes Nnwdaf_VFLTraining_Unsubscribe service operation as described in clause 4.10.2.3.2 of 3GPP TS 29.520 [5]. If the Unsubscription is successfully processed and accepted, the VFL client sends Nnwdaf_VFLTraining_Unsubscribe Response message as described in clause 4.10.2.3.2 of 3GPP TS 29.520 [5].

10b.	If the VFL client is a trusted AF, the NWDAF acting as VFL server invokes Naf_VFLTraining_Unsubscribe service operation as described in clause 5.2.2.3.2 of 3GPP TS 29.530 [53]. If the Unsubscription is successfully processed and accepted, VFL client sends Naf_VFLTraining_Unsubscribe Response message as described in clause 5.2.2.3.2 of 3GPP TS 29.530 [53].

10c. If NWDAF VFL client is an untrusted AF, the NWDAF acting as VFL server invokes Nnef_VFLTraining_Unsubscribe service operation. The NEF invokes Naf_VFLTraining_Unsubscribe by sending an HTTP POST request as described in clause 5.2.2.3.2 of 3GPP TS 29.530 [53]. If the Unsubscription is successfully processed and accepted, VFL client sends Naf_VFLTraining_Unsubscribe response message as described in clause 5.2.2.3.2 of 3GPP TS 29.530 [53]. The NEF sends Nnef_VFLTraining_Unsubscribe response message to VFL server as described in clause 4.10.2.3.2 of 3GPP TS 29.520 [5].

11.	Same as step 7.

5.10.3.3.3	Training procedure for vertical federated learning when untrusted AF is acting as VFL server

This procedure is used by the untrusted AF as VFL Server to trigger training procedure for Vertical Federated Learning among multiple NWDAF instances.

Figure 5.10.3.3.3-1: Training procedure for Vertical Federated Learning when untrusted AF is the VFL Server

1.	The NWDAF as service consumer determines the untrusted AF acting as an VFL server. The NWDAF may invoke Nnef_Training_Subscribe service operation to NEF as described in clause 4.11.2.2 of 3GPP TS 29.591 [11] and then NEF forwards the subscription to the VFL server untrusted AF using Naf_Training_Subscribe service operation to perform the VFL training as described in clause 5.4.2.2 of 3GPP TS 29.530 [53].

2.	The untrusted AF acting as VFL server selects VFL client (s) as described in clause 4.4.52.2 of 3GPP TS 29.522 [10]. The NWDAF or trusted AF acting as VFL server performs the VFL preparation as described in clause 5.10.3.2.

3.	The untrusted AF acting as VFL server sends a training request to VFL clients by invoking Nnef_VFLTraining_Subscribe service operation to all associated VFL clients i.e., NWDAF, as described in clause 4.11.2.2 of 3GPP TS 29.522 [10].

4.	The NEF sends the training request received from the untrusted AF acting as VFL server to VFL clients i.e., NWDAF instances. The NEF shall map the external NWDAF and GPSI(s) to the internal NWDAF and SUPI(s). The training request will be sent through NEF by invoking Nnwdaf_VFLTraining_Subscribe service operation as described in clause 4.4.50.2 of 3GPP TS 29.520 [5].

5.	Same as step 4 in clause 5.10.3.3.2.

6.	Each VFL client responses to NEF with the client ML model training information by invoking the Nnwdaf_VFLTraining_Notify service operation as described in clause 4.10.2.4.2 of 3GPP TS 29.520 [5].

7.	The NEF maps the internal NWDAF and SUPI(s) to external NWDAF and GPSI(s) by invoking Nnef_VFLTraining_Notify service operation to the untrusted AF acting as VFL server as described in clause 4.4.51.5 of 3GPP TS 29.522 [10].

The VFL server untrusted AF sends the notification of the ML Model training information to NWDAF by invoking Naf_Training_Notify service operation as described in clause 5.4.2.4 of 3GPP TS 29.530 [53].

8.	Same as step 6 in clause 5.10.3.3.2.

9.	Same as step 9 in clause 5.10.3.3.2.

NOTE:	The steps 3-9 can be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

10.	The VFL server untrusted AF invokes the Naf_Training_Notify service operation to NEF as described in clause 5.4.2.4 of 3GPP TS 29.530 [53]. Then the NEF invokes Nnef_Training_Notify service operation to NWDAF as described in clause 4.11.2.4 of 3GPP TS 29.591 [11].

* * * End of Change * * * *