3GPP TSG-CT WG3 Meeting #144	C3-255569

# Dallas, USA, 17-21 November 2025

Title:	LS on Signalling Storm Analytics

Release:	Release 19

Work Item:	AIML_CN

Source:	CT3

To:	SA2

Cc:	CT4

Contact person:	Shiramizu Takashi

takashi.shiramizu.gx@nttdocomo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

TS 23.288 clause 6.22.2 (input data of Signalling Storm analytics) included timer information for AMF, SMF as data source with below generic description:

| > timer information | AMF, SMF | Timer information which has been set for the UE, such as timer type, duration. |
| --- | --- | --- |

The Output analytics in 6.22.3 included Optional Timer List consists of Type of timer and Timer duration with below generic description:

| > OPTIONAL Timer List | The list of timer information per source UE(s) (NOTE 1). |
| --- | --- |
| >> Type of timer | The type of timer which has been set. |
| >> Timer duration | The timer duration that has be selected for the source UE(s). |
| NOTE 1: Only available when Cause ID of signalling storm is massive signalling from UEs, and there exists Source UE(s). |  |

The mitigation actions in clause 6.22.3 include the following entries related to timers:

| massive signalling from UE | AMF sets MM NAS related timer (e.g. back-off, T3512) for a selected set of UEs. |
| --- | --- |
| massive signalling from UE | SMF sets SM NAS related timer (e.g. back-off) for a selected set of Sessions. |

Finally, some companies in CT3 believe that the information related to timers need to include information about the slice to which the timer belongs, i.e. the DNN and S-NSSAI. CT3 would therefore like to ask the following questions:

Question 1: Which are the timer types that can be retrieved from the NFs (as input data) and/or exposed by the NWDAF (in the output analytics)?

Question 2: Are the DNN and/or S-NSSAI associated to these timers required?

CT3 kindly asks SA2 to provide answer for the above questions.

# 2	Actions

To SA2

ACTION: 	CT3 kindly asks SA2 to provide answer for the above questions and adjust the specs, if necessary.

# 3		Dates of next TSG CT WG 3 meetings

CT3#145	09-13 February 2026	India

CT3#146	13-17 April 2026 		Malta