3GPP TSG-CT WG4 Meeting #130	C4-253523

# Göteborg, Sweden; 25th – 29th August 2025									   was C4-253427, 3126

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0985 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provision of I-UPF ID over N4 Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For a PDU session involving I-SMF/I-UPF, when the SMF wants to collect the energy consumption information, it needs to collect the data volume in the gNB, I-UPF and Anchor UPF. To archieve this, the SMF needs to know the I-UPF ID. TS23.502 clause 5.51.2.2.2 specifies that the SMF requests the I-SMF to report the I-UPF ID and the subsequent changes: The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1. The SMF receives the data volume of the required granularities from the PSA UPF. If the I-SMF is involved in the PDU session, the SMF of PDU session requests I-SMF to report the current I-UPF ID and ULI and subsequent change of these entities. In existing TS29.244, it allows the SMF to learn the UPF instance ID from the PFCP Association Setup Request/Response. However, it states that the UPF may provide the UPF instance ID to the SMF. In the PDU session involving I-SMF, to allow the SMF (e.g. I-SMF) always retrieves the UPF (e.g. I-UPF) instance ID, it is proposed to change the UPF behaviour to always provide its NF Instance ID to the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the PFCP Association Setup Request/Response, the inclusion of UPF Instance ID is changed from "may" to "shall". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMF is not able to retrieve the UPF Instance ID from the PFCP Association Setup Request/Respose. If the I-UPF selection is not via NRF, the I-SMF is not possible to report the I-UPF Instance ID to the A-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2, 7.4.4.1.1, 7.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#2: Add the missing change to clause 6.2.6.2.2 of UPF behavior. Now the UPF shall always provide its NF Instance ID if the UPF Instance ID is available. Change the presence of UPF Instance ID from "O" to "C", in clause 7.4.4.1.1 and clause 7.4.4.2. Rev#1: Revert the original changes, and update the UPF behaviour of reporting its NF Instance ID to SMF during PFCP Association Setup procedure, i.e. from "may" to "shall". |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.6.2.2	UP Function behaviour

When receiving a PFCP Association Setup Request, the UP function:

-	if the request is accepted:

-	shall store the Node ID of the CP function as the identifier of the PFCP association;

-	shall send a PFCP Association Setup Response including:

-	a successful cause;

-	its Node ID;

-	information of all supported optional features in the UP function;

-	optionally one or more UE IP address Pool Information IE which contains a list of UE IP Address Pool Identities per Network Instance, S-NSSAI and IP version;

-	the NF Instance ID of the UPF if available;

-	optionally the supported Packet Inspection functionalities;

-	information about the supported Operator configurable UPF capabilities if any.

-	shall send a PFCP Version Not Supported Response if the PFCP header of the request indicates a PFCP protocol version that is not supported by the UP function;

-	otherwise, shall send a PFCP Association Setup Response with an appropriate error cause if the request is rejected.

If the PFCP Association Setup Request contains a Node ID for which a PFCP association was already established, the UP function shall:

-	proceed with establishing the new PFCP association (regardless of the Recovery Timestamp received in the request), overwriting the existing association;

-	retain the PFCP sessions that were established with the existing PFCP association and that are requested to be retained, if the PFCP Session Retention Information IE was received in the request; otherwise, delete the PFCP sessions that were established with the existing PFCP association;

-	set the PSREI (PFCP Session Retained Indication) flag to "1" in the PFCP Association Setup Response, if the PFCP Session Retention Information IE was received in the Request and the requested PFCP sessions have been retained.

If the UPF is configured to be used for IPUPS, the UPF shall set the UUPSI (UPF configured for IPUPS Indication) flag to "1" in the PFCP Association Setup Response message.

* * * Next Change * * * *

##### 7.4.4.1.1	General

Table 7.4.4.1-1: Information Elements in a PFCP Association Setup Request

| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Node ID | M | This IE shall contain the unique identifier of the sending Node. | X | X | X | X | X | Node ID |
| Recovery Time Stamp | M | This IE shall contain the time stamp when the CP or UP function was started, see clause 19A of 3GPP TS 23.007 [24]. (NOTE 1) | X | X | X | X | X | Recovery Time Stamp |
| UP Function Features | C | This IE shall be present if the UP function sends this message and the UP function supports at least one UP feature defined in this IE. When present, this IE shall indicate the features the UP function supports. | X | X | X | X | X | UP Function Features |
| CP Function Features | C | This IE shall be present if the CP function sends this message and the CP function supports at least one CP feature defined in this IE. When present, this IE shall indicate the features the CP function supports. | X | X | X | X | X | CP Function Features |
| Alternative SMF IP Address | O | This IE may be present if the (MB-)SMF advertises the support of the SSET and/or MPAS feature in the CP Function Features IE (see clause 8.2.58). When present, this IE shall contain an IPv4 and/or IPv6 address of an alternative (MB-)SMF or an alternative PFCP entity in the same (MB-)SMF when SSET feature is used, or an alternative PFCP entity in the same (MB-)SMF when MPAS feature is used. Several IEs with the same IE type may be present to represent multiple alternative (MB-)SMF IP addresses. | - | - | - | X | X | Alternative SMF IP Address |
| SMF Set ID | C | This IE shall be present if the (MB-)SMF advertises the support of the MPAS feature in the CP Function Features IE (see clause 5.22.3). When present, this IE shall contain an FQDN representing the (MB-)SMF set to which the (MB-)SMF belongs. | - | - | - | X | X | SMF Set ID |
| PFCP Session Retention Information | O | This IE may be present to request the UP function to keep all or part of the existing PFCP sessions upon receipt of a PFCP association setup request with a Node ID for which a PFCP association was already established. See clause 6.2.6.2.1. | X | X | X | X | X | PFCP Session Retention Information |
| UE IP address Pool Information | O | This IE may be present when the UP function sends this message, if UE IP Address Pools are configured in the UP function. Several IE with the same IE type may be present to represent multiple UE IP address Pool Information. | - | X | - | X | - | UE IP address Pool Information |
| GTP-U Path QoS Control Information | C | This IE may be present, if the CP function sends this message, to request the UPF to monitor the QoS on GTP-U paths (see clause 5.24.5). Several IEs with the same IE type may be present to represent multiple GTP-U paths (with different parameters) to monitor. | - | - | - | X | - | GTP-U Path QoS Control Information |
| Clock Drift Control Information | O | This IE may be present, if the CP function sends this message, to request the UPF to report clock drift between the external time and 5GS time for working domains (see clause 5.26.4). Several IEs with the same IE type may be present for multiple Time domains (with different parameters). | - | - | - | X | - | Clock Drift Control Information |
| UPF Instance ID | C | This IE shall be present if the UP function is a 5G UP function and if UPF Instance ID is available, and if the message is sent by the UP function. | - | - | - | X | X | NF Instance ID |
| PFCPASReq-Flags | O | This IE shall be included if at least one of the flags is set to "1": - UUPSI (UPF configured for IPUPS): when the message is sent by a UPF, the UP function shall set this flag to "1" if the UPF is configured to be used for IPUPS. See clause 5.27. | - | - | - | X | - | PFCPASReq-Flags |
| N6 Delay Measurement Control Information | C | This IE may be present, if the CP function sends this message, to request the UPF to measure the N6 delay between the UPF and one or more measurement endpoints (see clause 5.33.8). Several IEs with the same IE type may be present to represent multiple measurement endpoints to monitor. | - | - | - | X | - | N6 Delay Measurement Control Information |
| N6 Delay Measurement Protocols | C | This IE should be present if the UPF sends this message and if the UPF indicates support of the UN6DM feature. When present, it shall indicate the N6 delay measurement protocols that the UPF supports. | - | - | - | X | - | N6 Delay Measurement Protocols |
| Operator Configurable UPF Capability | O | This IE may be present if the UP function sends this message, and the UP function supports any of the Operator Configurable UPF Capabilities as described in clause 5.8.2.21 of 3GPP TS 23.501 [28]. Several IEs with the same IE type may be present to represent multiple Operator Configurable UPF Capabilities. | - | - | - | X | - | Operator Configurable UPF Capability |
| Packet Inspection functionality | O | This IE may be present, if the UP function sends this message, to indicate the supported packet inspection capabilties as described in clause 8.2.250. (NOTE 2) | - | - | - | X | - | Packet Inspection functionality |
| NOTE 1: A PFCP function shall ignore the Recovery Timestamp received in the PFCP Association Setup Request message. NOTE 2: A UPF not including the Packet Inspection functionality IE (e.g. UPF complying with an earlier release of the specification) may support one or more packet inspection capabilities. If the Packet Inspection functionality IE is not received by the CP function, the CP function may determine the packet inspection capabilities of the UPF based on local configuration, or by default, it shall assume that the UPF supports all the packet inspection capabilities required for all the network instances supported by the UPF. |  |  |  |  |  |  |  |  |

Table 7.4.4.1-2: PFCP Session Retention Information IE within PFCP Association Setup Request

| Octet 1 and 2 |  |  | PFCP Session Retention Information IE Type = 183 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| CP PFCP Entity IP Address | O | This IE may be present to indicate the IP address of a CP PFCP entity for which the UP function shall retain the existing PFCP sessions, upon receipt of a PFCP association setup request with a Node ID for which a PFCP association was already established. See clause 6.2.6.2.1 Several IEs with the same IE type may be present to represent multiple CP PFCP entities for which PFCP sessions shall be retained. If no CP PFCP Entity IP Address IE is present in the PFCP Session Retention Information IE, all existing PFCP sessions shall be kept upon receipt of a PFCP association setup request with a Node ID for which a PFCP association was already established. |  | X | X | X | X | X | CP PFCP Entity IP Address |

Table 7.4.4.1-3: UE IP address Pool Information IE within PFCP Association Setup Request

| Octet 1 and 2 |  |  | UE IP address Pool Information IE Type = 233 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| UE IP address Pool Identity | M | When present, this IE shall contain an UE IP address Pool Identity Several IEs with the same IE type may be present to represent multiple UE IP address Pool Identities. |  | - | X | - | X | - | UE IP address Pool Identity |
| Network Instance | O | The IE may be present to indicate for which DNN/APN the UE IP Address Pool Identifies are configured. |  | - | X | - | X | - | Network Instance |
| S-NSSAI | O | The IE may be present to indicate for which S-NSSAI the UE IP Address Pool Identities are configured. Several IEs with the same IE type may be present to represent multiple S-NSSAIs. |  | - | - | - | X | - | S-NSSAI |
| IP version | O | The IE may be present to indicate for which IP version the UE IP Address Pool Identities are configured. |  | - | - | - | X | - | IP version |

* * * Next Change * * * *

#### 7.4.4.2	PFCP Association Setup Response

Table 7.4.4.2-1: Information Elements in a PFCP Association Setup Response

| Information elements | P | Condition / Comment | Appl. |  |  |  |  | IE Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| Node ID | M | This IE shall contain the unique identifier of the sending Node. | X | X | X | X | X | Node ID |
| Cause | M | This IE shall indicate the acceptance or the rejection of the corresponding request message. | X | X | X | X | X | Cause |
| Recovery Time Stamp | M | This IE shall contain the time stamp when the CP or UP function was started, see clause 19A of 3GPP TS 23.007 [24]. (NOTE 1) | X | X | X | X | X | Recovery Time Stamp |
| UP Function Features | C | This IE shall be present if the UP function sends this message and the UP function supports at least one UP feature defined in this IE. When present, this IE shall indicate the features the UP function supports. | X | X | X | X | X | UP Function Features |
| CP Function Features | C | This IE shall be present if the CP function sends this message and the CP function supports at least one CP feature defined in this IE. When present, this IE indicates the features the CP function supports. | X | X | X | X | X | CP Function Features |
| Alternative SMF IP Address | O | This IE may be present if the (MB-)SMF advertises the support of the SSET and/or MPAS feature in the CP Function Features IE (see clause 8.2.58). When present, this IE shall contain an IPv4 and/or IPv6 address of an alternative (MB-)SMF or an alternative PFCP entity in the same (MB-)SMF when SSET feature is used, or an alternative PFCP entity in the same (MB-)SMF when MPAS feature is used. Several IEs with the same IE type may be present to represent multiple alternative (MB-)SMF IP addresses. | - | - | - | X | X | Alternative SMF IP Address |
| SMF Set ID | C | This IE shall be present if the CP function sends this message and (MB-)SMF advertises the support of the MPAS feature in the CP Function Features IE (see clause 5.22.3). When present, this IE shall contain an FQDN representing the (MB-)SMF set to which the (MB-)SMF belongs. | - | - | - | X | X | SMF Set ID |
| PFCPASRsp-Flags | O | This IE shall be included if at least one of the flags is set to "1": - PSREI (PFCP Session Retained Indication): the UP function shall set this flag to "1" if the PFCP Session Retention Information IE was received in the Request, an existing PFCP association was already established for the same Node ID and the requested PFCP sessions have been retained. See clause 6.2.6.2.2. - UUPSI (UPF configured for IPUPS): the UP function shall set this flag to "1" if the UPF is configured to be used for IPUPS. See clause 5.27. | X - | X - | X - | X X | X - | PFCPASRsp-Flags |
| Clock Drift Control Information | C | This IE may be present, if the CP function sends this message, to request the UPF to report clock drift between the external time and 5GS time for external working domains (see clause 5.26.4). Several IEs with the same IE type may be present to represent multiple external time domains (with different parameters). See Table 7.4.4.1.2-1. | - | - | - | X | - | Clock Drift Control Information |
| UE IP address Pool Information | O | This IE may be present when the UP function sends this message, if UE IP Address Pools are configured in the UP function. Several IE with the same IE type may be present to represent multiple UE IP address Pool Information. The IE shall be encoded as in Table 7.4.4.1-3. | - | X | - | X | - | UE IP address Pool Information |
| GTP-U Path QoS Control Information | C | This IE may be present, if the CP function sends this message, to request the UPF to monitor the QoS on GTP-U paths (see clause 5.24.5). Several IEs with the same IE type may be present to represent multiple GTP-U paths to monitor. See Table 7.4.4.1.3-1. | - | - | - | X | - | GTP-U Path QoS Control Information |
| UPF Instance ID | C | This IE shall be present if the UP function is a 5G UP function and if the UPF Instance ID is available, and if the message is sent by the UP function. | - | - | - | X | X | NF Instance ID |
| N6 Delay Measurement Control Information | C | This IE may be present, if the CP function sends this message, to request the UPF to measure the N6 delay between the UPF and one or more measurement endpoints (see clause 5.33.8). Several IEs with the same IE type may be present to represent multiple measurement endpoints (with different parameters) to monitor. See Table 7.4.4.1.4-1. | - | - | - | X | - | N6 Delay Measurement Control Information |
| N6 Delay Measurement Protocols | C | This IE should be present if the UPF sends this message and if the UPF indicates support of the UN6DM feature. When present, it shall indicate the N6 delay measurement protocols that the UPF supports. | - | - | - | X | - | N6 Delay Measurement Protocols |
| Operator Configurable UPF Capability | O | This IE may be present if the UP function sends this message, and the UP function supports any of the Operator Configurable UPF Capabilities as described in clause 5.8.2.21 of 3GPP TS 23.501 [28]. Several IEs with the same IE type may be present to represent multiple Operator Configurable UPF Capabilities. | - | - | - | X | - | Operator Configurable UPF Capability |
| Packet Inspection functionality | O | This IE may be present, if the UP function sends this message, to indicate the supported packet inspection capabilties as described in clause 8.2.250. (NOTE 2) | - | - | - | X | - | Packet Inspection functionality |
| NOTE: A PFCP function shall ignore the Recovery Timestamp received in PFCP Association Setup Response message. |  |  |  |  |  |  |  |  |

* * * End of Change * * * *