3GPP TSG-CT WG4 Meeting #130	C4-254256

Sophia Antipolis, France, 13th – 17th October 2025 	revision of C4-254124

Source:	KDDI, Nokia

Title:	Pseudo-CR on Evaluation of Solutions for Key Issue#1

Spec:	3GPP TR 29.867 v0.2.0

Agenda item:	21.4

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Evaluation of the Technical Report is missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867 v0.2.0.

Note that this evaluation is subject to revision in response to input from SA2, CT1 and CT3.

# * * * 1st Change (All new text) * * * *

7.X	Evaluation of Solutions for Key Issue#1

A group of Solutions have been proposed for Key Issue#1 to address to prevent UE SIP Registration attempts during IMS Failures.

Solution #1 proposes leveraging existing timeout mechanism during SIP Registration procedure to trigger a registration retry delay timer in the UE. Specifically, when Timer F expires for all P-CSCFs that are in the list of P-CSCFs for a UE, the retry delay timer is activated. This delay is computed using the algorithm specified in subclause 4.5 of RFC 5626 [9]. This solution builds upon existing procedures in 3GPP TS 24.229 [3]. Recovery detection similarly relies on the retry delay mechanism. The UE detects P-CSCF recovery when it sends a new SIP REGISTER after the expiry of the retry delay time and receives a positive SIP response.

Solution #2 proposes notifying P-CSCF failure status of registered UE with PCO/ePCO. To detect P-CSCF failure and recovery, several options are introduced, leveraging either existing standardized mechanisms or implementation-specific approaches. For P-CSCF failure notification, a new IE, Request Suppression Timer is introduced within PCO/ePCO. This IE indicates that the P-CSCF is unavailable and specifies the duration during which the UE should refrain from sending SIP Registration requests. During this period, the UE does not perform IMS PDN Detach or bearer re-establishment procedures such as those triggered by a Back-Off Timer or Retry-After header. Instead, the IMS PDN session is maintained. Additionally, the solution proposes defining a new IE during PDU Session Establishment to indicate whether the UE supports this feature. For P-CSCF recovery notification, the solution reuses the P-CSCF restoration mechanism as defined in 3GPP TS 23.380 [2].

Solution #3 proposes utilizing the network-requested PDU session release mechanism defined in 3GPP TS 24.501 [6] to release the PDU session used by the UE to send SIP signalling (e.g. PDU session to the IMS DNN) and thus preventing repeated SIP registration attempts by the UE. P-CSCF failure is detected e.g., by the SMF according to the features defined in the existing specifications such as P-CSCF Failure Detection at SMF/UPF defined in the clause 5.8.3 of 3GPP TS23.380 [2]. Upon detection, the SMF initiates the PDU session release used for SIP signalling and sets the 5GSM cause IE in the PDU SESSION RELEASE COMMAND to cause value #67. This prevents the UE from attempting SIP registration during the P-CSCF failure period. Additionally, the SMF includes a Back-off Timer IE in the PDU SESSION RELEASE COMMAND message, indicating the duration for which the UE shall refrain from re-establishing a PDU session for SIP signalling.

Solution #X proposes leveraging UAC with access category 9 (MO IMS registration) as defined in 3GPP TS 24.501 [6]. By applying access control to UEs using this category, the barring rate can be gradually relaxed to prevent traffic surges. Since the access category 9 is limited to IMS registration signaling, this approach enables targeted barring without affecting other communication services.

Table 7.X-1 shows a comparison of Solution#1, Solution#2, Solution#3 and Solution#X as follows.

Table 7.X-1 Comparison of Solution#1, Solution#2 and Solution#3

|  |  | Solution#1 | Solution#2 | Solution#3 | Solution#X |
| --- | --- | --- | --- | --- | --- |
| Impacts | Pros | No impacts to IMS Network. No impacts to the packet domain. | No impacts to IMS Network | No impact to IMS Network. Works with legacy UEs. | No impact to IMS Network and SIP Client of the UE. Works with legacy UEs. |
|  | Cons | The SIP Client of the UE needs to be modified, i.e. the solution relies on UE supporting the new functionality. | The SIP Client of the UE needs to be modified i.e. SIP registration suppression. Minimal functional enhancements to the 5GC (SMF, AMF) such as extending IEs in PCO/ePCO. Relying on a single detection method, such as Rx interface monitoring, might lead to inaccurate determination of P-CSCF failure. A bulk failure notification procedure implies sending individual notifications for all users currently registered in the P-CSCF, | None | - There is no restriction on the signalling that already establishes an RRC connection or the procedure for restoration. In case of roaming and network sharing including MVNO scenario, SLA or OAM level agreement may be needed. |
| Access dependency |  | Access independent, it can support 5G, 4G and all other accesses. | Covers 5G | Covers 5G | Covers 5G |

Editor's note:	This clause is subject to revision in response to input from SA2, CT1 and CT3.

# * * * End of Changes * * * *