3GPP TSG-CT WG4 Meeting #131	C4-254296

Sophia Antipolis, France; 13th – 17th October 2025	was C4-254136

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0378 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support muting of event notifications for LMF Data Exposure Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, TS 29.572 does not support event notification muting functionality for LMF data exposure service. To align with SA2 requirements in TS 23.288 and to support use cases where NWDAF requires controlled delivery of buffered data, it is necessary to enable notification muting, retrieval, and reactivation in the LMF Data Exposure service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new notifFlag IE in the LmfDataExposureSubscription data type to support event notification muting ("ACTIVATE", "DEACTIVATE", "RETRIEVAL"). Add support for optional mutingExcInstructions and mutingNotifSettings IEs. Update subscription creation and modification procedures to describe how notifications are muted, buffered, retrieved, and reactivated. define new application error MUTING_EXC_INSTR_NOT_ACCEPTED. Update Table 6.3.3.3.3.1-3 to support sending "UNSUPPORTED_ML_MODEL" to the NF service producer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support for notification muting would prevent NWDAF from controlling data exposure flow for ML training or monitoring. This would reduce flexibility, lead to inefficient data delivery. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.2.2, 5.4.2.3, 6.3.6.1, 6.3.3.2.3.1, 6.3.3.3.3.1, 6.3.6.2.2, 6.3.7.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29572_Nlmf_DataExposure.yaml TS29574_Ndccf_ContextManagement.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Editorial corrections - Update Table 6.3.3.3.3.1-3 to support sending " UNSUPPORTED_ML_MODEL " to the NF service producer. |  |  |  |  |  |  |  |  |

* * * First Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[15]	IETF RFC 9457: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 37.355: " LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[23]	3GPP TS 29.518: "Access and Mobility Management Services".

[24]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[25]	IETF RFC 4119: "A Presence-based GEOPRIV Location Object Format".

[26]	3GPP TS 33.256: "Security aspects of Uncrewed Aerial Systems (UAS)".

[27]	3GPP TS 29.515: "5G System; Gateway Mobile Location Services Stage 3".

[28]	Void

[29]	Void

[30]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[31]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[32]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[33]	IETF RFC 2387: "The MIME Multipart/Related Content-type".

[34]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[35]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[X]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

* * * Next Change * * *

#### 5.4.2.2	LMF Data Exposure Subscription

This procedure allows NF Service Consumer to subscribe for notifications of sampling data used for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning by using HTTP POST method with the URI of individual subscription resource.

Figure 5.4.2.2-1: Subscription of NF service consumer to LMF data exposure

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the subscription collection. The input parameters for the request shall include notification target address, notification correlation ID and AoI. The request may include other optional parameters for data collection (see clause 6.3.6.2.2).

If the subscription is for performance monitoring of LMF based AI/ML positioning, the request shall also include the ML model identifier to be monitored.

2a.	On success, "201 Created" shall be returned. The response body shall include a HTTP Location header including the subscription ID together with the status code 201 indicating the requested resource is created in the response message.

-	If the LmfDataExposureSubscription contains "recurTime" IE, then LMF shall generate a report at the end of every time interval provided in the "recurTime" IE.

-	If the LmfDataExposureSubscription contains "recurTime" IE and "numOfSamples" IE, then LMF shall generate a report when the requested number of samples are met or at the end of every time interval provided in the "recurTime" IE.

If the NF Service Consumer includes the notifFlag IE set to "DEACTIVATE", the LMF shall mute the event notification and buffer the event notifications. Additionally, if the NF service consumer includes event muting instructions in mutingExcInstructions, the LMF evaluates the received event muting instructions against local policies (if configured) and, if the subscription creation request is accepted, the LMF may indicate the following information to the NF service consumer in the response:

-	the maximum number of notifications that the LMF can buffer;

-	the maximum duration that the LMF can buffer the notifications.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.3.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

If the LMF cannot accept the received muting exception instructions, the AMF may reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

* * * Next Change * * *

#### 5.4.2.3	LMF Data Exposure Modification

This procedure allows NF Service Consumer to modify an existing subscription by using HTTP PATCH method with the URI of individual subscription resource.

Figure 5.4.2.3-1: Modification of subscription for LMF data exposure

1.	The NF Service Consumer shall send a PATCH request to modify a subscription resource in the LMF.

2a.	On success, ether "204 No Content" or "200 OK" shall be returned.

If the NF service consumer modifies the notifFlag IE:

-	If the notifFlag IE is set to "DEACTIVATE" in the request and the event notification is currently not muted, the LMF shall mute the event notification and buffer the available events, or

-	If the notifFlag IE is set to "RETRIEVAL" in the request and the event notification is currently muted, the LMF shall send the buffered events to the NF Service Consumer, mute the event notification again and buffer subsequent events, or

-	If the notifFlag IE is set to "ACTIVATE" in the request and the event notification is currently muted, the LMF shall send the buffered events to the NF Service Consumer and stop muting the event notification.

In addition, if the PATCH request modifies the notifFlag IE to "DEACTIVATE" and contains muting exception instructions, the LMF shall evaluate the received muting exception instructions against local policies (if configured). If the subscription modification request is accepted, the LMF may include in the response:

-	the maximum number of notifications that the LMF can buffer;

-	the maximum duration that the LMF can buffer the notifications.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.3.3.3.3.1-3 shall be returned. For a 4xx/5xx response, the message body contains a ProblemDetails indicating appropriate additional error information.

If the LMF cannot accept the received muting exception instructions, the AMF may reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

* * * Next Change * * *

###### 6.3.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LmfDataExposureSubscription | M | 1 | LMF Data Exposure Subscription to be created |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LmfDataExposureSubscription | M | 1 | 201 Created | Represents successful creation of an LMF Subscription |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application error: - UNSUPPORTED_ML_MODEL - MUTING_EXC_INSTR_NOT_ACCEPTED See table 6.3.7.3-1 for the description of these errors. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.3.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nlmf-dataexposure/<apiVersion>/subscriptions/{subscriptionId} |

Table 6.3.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.3.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * *

###### 6.3.3.3.3.1	PATCH

This method shall support the URI query parameters specified in table 6.3.3.3.3.1-1.

Table 6.3.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.1-2 and the response data structure and response codes specified in table 6.3.3.3.3.1-3.

Table 6.3.3.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | It contains the list of changes to be made to the subscription, according to the JSON PATCH format specified in IETF RFC 6902 [32]. |

Table 6.3.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Represents a successful update on LMF Data Exposure Subscription |
| LmfDataExposureSubscription | M | 1 | 200 OK | Represents a successful update on LMF Data Exposure Subscription with contents. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the subscription or the modification of subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - UNSUPPORTED_ML_MODEL - MUTING_EXC_INSTR_NOT_ACCEPTED See table 6.3.7.3-1 for the description of these errors. |
| NOTE 1: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP or SEPP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same LMF or LMF (service) set. For the case when a request is redirected to the same target resource via a different SCP or SEPP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

* * * Next Change * * *

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the Nlmf_DataExposure service-based interface protocol.

Table 6.3.6.1-1: Nlmf_DataExposure specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| LmfDataExposureSubscription | 6.3.6.2.2 | Represents an individual data collection subscription resource on LMF |
| LmfDataExposureNotification | 6.3.6.2.3 | Notify within an LMF exposure for data collection notification request. |
| LmfDataExposureReport | 6.3.6.2.4 | Represents the data collection exposure report. |
| LmfDataExposureSamplingData | 6.3.6.2.5 | LMF Data Exposure Sampling Data |
| LmfDataExposureCause | 6.3.6.3.3 | Cause code for LMF Data Exposure Service |
| DataSourceType | 6.3.6.3.4 | Data source type for measurement data collection |

Table 6.3.6.1-2 specifies data types re-used by the Nlmf_DataExposure service based interface protocol from other specifications or from other service APIs in current specification, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_DataExposure service based interface.

Table 6.3.6.1-2: Nlmf_DataExposure re-used Data Types

| Data type | Reference | Description |
| --- | --- | --- |
| PresenceInfo | 3GPP TS 29.571 [8] | Area of Interest |
| Uri | 3GPP TS 29.571 [8] | URI |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |
| DateTime | 3GPP TS 29.571 [8] | Date time |
| PatchItem | 3GPP TS 29.571 [8] | Patch item |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned Integer |
| NotificationFlag | 3GPP TS 29.571 [8] | Notification flag |
| MutingExceptionInstructions | 3GPP TS 29.571 [8] | Muting exception instructions |
| MutingNotificationsSettings | 3GPP TS 29.571 [8] | Muting notifications settings |
| RecurTime | 3GPP TS 29.503 [34] | Recurring Time window |
| LocationData | 6.1.6.2.3 | Location Data |
| LocationQoS | 6.1.6.2.13 | Location QoS |
| CorrelationId | 6.1.6.3.2 | Correlation Id |
| LocMeasurements | 6.1.6.2.47 | Location Measurements |

* * * Next Change * * *

##### 6.3.6.2.2	Type: LmfDataExposureSubscription

Table 6.3.6.2.2-1: Definition of type LmfDataExposureSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | M | 1 | This IE shall be included to identify the recipients of notifications sent by LMF for data exposure services. |  |
| notifyCorrelationId | CorrelationID | M | 1 | This IE shall be included to identify the notification correlation ID. The LMF shall include this ID in the notifications. The value of this IE shall be unique per subscription for a given NF service consumer. |  |
| aoi | PresenceInfo | M | 1 | The Area of Interest where the sampling data to be provided. |  |
| recurTime | RecurTime | O | 0..1 | When present, it shall indicate the list of time interval(s) during which LMF needs to initiate the collection of input data samples. |  |
| numOfSamples | Uinteger | O | 0..1 | When present, this IE shall indicate the required number of sampling data to be provided. If the recurTime IE is present, then this IE shall be applied per each time interval inside recurTime IE. |  |
| dataSources | array(DataSourceType) | O | 1..N | When present, this IE shall indicate the data source(s) from where the measurement data to be collected. |  |
| qualityThreshold | LocationQoS | O | 0..1 | When present, this IE shall indicate the quality threshold of the Ground Truth data to be provided. The LMF shall not report a sampling data in the notification, if the ground truth of the sampling data cannot meet the quality threshold. |  |
| mlModelId | Uinteger | C | 0..1 | This IE shall be present when the subscription is to collect data for performance monitoring for LMF based AI/ML positioning. When present, this IE shall indicate the unique identifier of the ML Model for performing monitoring of LMF based AI/ML positioning. |  |
| expiryTime | DateTime | O | 0..1 | When present, it shall indicate the time at which the subscription expires unless it is renewed. |  |
| notifFlag | NotificationFlag | O | 0..1 | This IE shall Indicate the notification flag. It shall be Used to mute/unmute notifications and to retrieve buffered events during a muted period. See clause 6.2.7 of 3GPP TS 23.288 [X]. |  |
| mutingExcInstructions | MutingExceptionInstructions | C | 0..1 | This IE may be included if notifFlag = "DEACTIVATE". It specifies instructions for handling buffered notifications and the subscription if an exception occurs (e.g. buffer full, exceeded number of buffered events). See clause 6.2.7 of 3GPP TS 23.288 [X]. Write-Only: true |  |
| mutingNotSettings | MutingNotificationsSettings | C | 0..1 | This IE shall Indicate the LMF’s applied muting notification settings (e.g. maximum number of buffered notifications, duration) if event notification muting is activated. See clause 6.2.7 of 3GPP TS 23.288 [X]. Read-Only: true |  |

* * * Next Change * * *

#### 6.3.7.3	Application Errors

The common application errors defined in Table 5.2.7.2-1 in 3GPP TS 29.500 [4] may be used for the Nlmf_DataExposure service. The application errors defined for the Nlmf_DataExposure service are listed in Table 6.3.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UNSUPPORTED_ML_MODEL | 403 Forbidden | The requested ML Model for data collection is not supported by the LMF. |
| MUTING_EXC_INSTR_NOT_ACCEPTED | 403 Forbidden | The LMF does not accept the received muting exception instructions. |

* * * Next Change * * *

# A.4	Nlmf_DataExposure API

openapi: 3.0.0

info:

version: 1.0.0-alpha.4

title: Nlmf_DataExposure

description: |

LMF Data Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- nlmf-dataexposure

[Skipped for clarity]

post:

summary: Nlmf_Data Exposure Subscribe Service Operation

tags:

- Subscriptions(Collection)

operationId: CreateSubscription

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureSubscription'

required: true

responses:

'201':

description: Subscription Created

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/nlmf-dataexposure/<apiVersion>/subscriptions/{subscriptionId}

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

LmfDataExposureNotification:

'{$request.body#/subscription/notificationUri}':

post:

summary: LMF Data Exposure Notification

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureNotification'

required: true

responses:

'204':

description: Successful acknowledgement

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

patch:

summary: Nlmf_DataExposure Subscribe Modify service Operation

tags:

- Individual subscription (Document)

operationId: ModifySubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be modified

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Expected response to a successful subscription modification

'200':

description: Subscription Modified

content:

application/json:

schema:

$ref: '#/components/schemas/LmfDataExposureSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

[Skipped for clarity]

#

# STRUCTURED TYPES

#

LmfDataExposureSubscription:

description: Describes subscription operation of LMF data exposure

type: object

properties:

notificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CorrelationID'

aoi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

recurTime:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/RecurTime'

numOfSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

dataSources:

type: array

items:

$ref: '#/components/schemas/DataSourceType'

minItems: 1

qualityThreshold:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

mlModelId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

expiryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

notifFlag:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationFlag'

mutingExcInstructions:

writeOnly: true

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/MutingExceptionInstructions'

mutingNotSettings:

readOnly: true

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/MutingNotificationsSettings'

required:

- notificationUri

- notifyCorrelationId

- aoi

[Skipped for clarity]

* * * End of Changes * * *