3GPP TSG CT WG4 Meeting #131	C4-254339

# Sophia-Antipolis, France, 13 – 17 October, 2025

3GPP TSG CT WG3 Meeting #143	C3-254338

# Sophia-Antipolis, France, 13 – 17 October, 2025

Source:	Samsung

Title:	Discussion paper on Unified Event Exposure in Core Network

Document for:	Discussion & Endorsement

# Agenda Item:	20.2.1

Abstract of the contribution:

# 1.	Introduction

As per the current core network architecture defined in 3GPP TS 23.501, multiple 5GC NFs provide EventExposure service in order to enable subscription/notifications of events by other NF consumers. Since each NF handles the event exposure functionality separately, it leads to duplication of functionality at NFs. In addition to that, managing subscriptions for large number of consumers is challenging for NFs. Therefore, this paper proposes to study the need for unification of management aspects of event exposure service to avoid redundancies and enable better functionalities.

# 2.	Discussion

3GPP TS 23.501 defines Event Exposure service as part of NFs like NEF, AMF, SMF, etc, that allows other network functions and applications to subscribe to and receive real-time notifications about specific events. Currently, each NF handles the event exposure functionality individually as shown below.

Figure 1. Decentralized Event Expsoure in 5GC

However, there are multiple challenges associated with the approach of handling management aspects (subscription, unsubscription, notify, delete, etc) of event exposure service at individual NF as given below.

- Redundant Implementation – The management aspect of event handling is same across multiple NFs but it needs to be implemented repeatedly for each NF.

- Specification Effort – Same operations like subscription, notification, unsubscription, etc are defined for each NF individually.

- Management Challenges - It becomes difficult for NFs to manage the subscriptions as the number of subscribers and events increase. For example, it becomes challenging to manage duplicate/expired subscriptions.

- Reliable delivery – Currently, the delivery mechanism follows best-effort. There is no mechanism to enable gurantees like exactly-once delivery of notifications.

- Lacks advanced features – Although, EventExposure service implements basic subscribe/notify functionality, it lacks the advanced features provided by modern platforms such as MQTT or Apache Kafka. For example, consumer can not replay events for analytics as the event are sent in real time only. Another important feature that current 5G event exposure functionality lacks is creating consumer groups. If multiple NFs want o subscribe to single event, they all subscribe individually.

Observation 1: As illustrated above, there are multiple challenges associated with the decentralized management of network events exposure at various network functions. 6G is an opportunity to address these challenges. The goal could be to unify the management aspects of multiple event exposure service APIs into one single service, to avoid redundancy, and enable better, streamlined management functionalities of various network events

# 3.	Protocols for centralized management of event exposure

Figure 2 illustrates the potential mechanism of centralized management of event exposure. Each NF of the core network can continue to be the entity generating the network events. However, the management of the events subscriptions can be centralized. For instance, an Event Broker Function can be single entity brokering between various consumer’s event subscriptions and producers of various events.

AMQP (Advanced Message Queuing Protocol) and MQTT (MQ Telemetry Transport) are some of the popular message brokering mechanisms.

Figure 2. Centralized Event Expsoure in 5GC

Observation 2: CT3 can study the potential protocols (like AMQP, MQTT) that can enable the centralized and streamlined management of 3GPP network events exposure.

# 4.	Proposal

Based on observation 1 and observation 2, Samsung proposes to study the following aspects as part of CT3 working group.

- Analyse current event exposure mechanisms in 5GC and develop the related key issues.

- Evaluate the need for unifying event exposure [CT3 study]

  - Centralized mangement of event exposure (e.g., using existing NFs with an SCP (Service Communication Proxy) based approach)

- Study the requirement of advanced features (e.g., events replay) as part of event exposure functionality [CT3 study]

- Study of various protocols (e.g., AMQP, MQTT) that are suitable for 3GPP event exposure [CT3 study]

- Study the impact on network architecture [CT3 study, potenital SA2 dependency]

  - With unified event exposure, individual NFs are responsible for reporting of events triggered at the NFs and unified event exposure functionality is responsible for event exposure (e.g. managing the subscriptions). The impact on the existing architecture with unified event exposure needs further evaluation.