3GPP TSG-CT WG4 Meeting #131	C4-254378

# Sophia Antipolis, France; 13rd – 17th October 2025					(Revision of C4-254344)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.329 | CR | 0256 | rev | 6 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add commands and AVPs to support HSS subscription to IMS AS and event notification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-07-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to 3GPP TS 23.228, the IMS AS receives subscription requests from HSS related to supported subscriber specific IMS DC events via the N71 reference point using the Nimsas_ImsEE_Subscribe service operation, or via the Sh reference point. So, this CR is proposed to add related commands and AVPs of HSS subscription to IMS AS and IMS AS notification via Sh interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Event-Subscribe-Request/Answer and Event-Notification-Request/Answer command. - Add related AVPs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HSS cannot subscribe to AS via Sh interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, 5, 8 (new), 8.1 (new), 8.1.1 (new), 8.1.2 (new), 8.1.3 (new), 8.1.4 (new), 8.2 (new), 8.2.1 (new), 8.2.2 (new), 8.2.3 (new), 8.3 (new), 8.3.1 (new), 8.3.2 (new), 8.3.3 (new), 8.3.4 (new), 8.3.5 (new), 8.3.6 (new), 8.3.7 (new), 8.3.8 (new), 8.3.9 (new), 8.3.10 (new), 8.3.11 (new), 8.3.12 (new), 8.3.13 (new), 8.3.n14 (new), 8.3.15 (new), 8.3.16 (new), 8.3.17 (new), 8.3.18 (new), 8.3.n19 (new), 8.3.20 (new) , 8.3.21 (new) , 8.3.22 (new) , 8.3.23 (new) , 8.3.24 (new) , 8.3.25 (new), 8.4 (new), 8.4.1 (new), 8.4.2 (new), 8.4.3 (new), 8.4.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add the event notification commands; Correct inconsistent descriptions; Delete event-information AVP. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions, which through reference in this text constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 29.328 "IP Multimedia (IM) Subsystem Sh interface; signalling flows and message contents".

[2]	3GPP TS 33.210 "3G Security; Network Domain Security; IP Network Layer Security".

[3]	IETF RFC 2960 "Stream Control Transmission Protocol".

[4]	Void.

[5]	IETF RFC 2234 "Augmented BNF for syntax specifications".

[6]	3GPP TS 29.229 "Cx and Dx Interfaces based on the Diameter protocol; protocol details".

[7]	IETF RFC 3589 "Diameter Command Codes for Third Generation Partnership Project (3GPP) Release 5".

[8]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[9]	3GPP TR 33.978 "Security aspects of early IP Multimedia Subsystem (IMS) (Release 6)".

[10]	3GPP TS 29.364 "IMS Application Server Service Data Descriptions for AS interoperability".

[11]	3GPP TS 29.002 "Mobile Application Part (MAP) specification".

[12]	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[13]	IETF RFC 7944: "Diameter Routing Message Priority".

[14]	IETF RFC 8583: "Diameter Load Information Conveyance".

[15]	IETF RFC 6733: "Diameter Base Protocol".

[16]]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[y]	IETF RFC 3986: "Uniform Resource Identifer (URI): Generic Syntax".

* * * Next Change * * * *

4	General

he Diameter base protocol as specified in IETF RFC 6733 [15] shall apply except as modified by the defined support of the methods and the defined support of the commands and AVPs, result and event codes specified in clause 6 and 8 of this specification. Unless otherwise specified, the procedures (including error handling and unrecognised information handling) are unmodified.

* * * Next Change * * * *

5	Use of the Diameter base protocol

The same clarifications of clause 5 of 3GPP TS 29.229 [6] shall apply to the Sh interface. An exception is that the application identifier for this application is defined in chapter 6 and 8.

* * * Next Change * * * *

8	Diameter application of Sh-ee for IMS events subscription/notification

This clause specifies a Diameter application named Sh-ee that allows:

-	An HSS can subscribe to receive notifications from the AS of IMS event.

-	The AS can notify an HSS of IMS events for which the HSS previously had subscribed

The Sh interface protocol is defined as an IETF vendor specific Diameter application, where the vendor is 3GPP. The vendor identifier assigned by IANA to 3GPP ( http://www.iana.org/assignments/enterprise-numbers) is 10415.

The Diameter application identifier assigned to Sh-ee application of the IMS events subscription/notification is xx1 (allocated by IANA).

8.1	Command-Code values

This clause defines Command-Code values for this Diameter application.

Every command is defined by means of the ABNF syntax (as defined in RFC 2234 [5]), according to the Command Code Format (CCF) specification defined in IETF RFC 6733 [15]. Whenever the definition and use of an AVP is not specified in this document, what is stated in 3GPP TS 29.229 [6] shall apply.

NOTE:	As the Diameter commands described in this specification have been defined based on the former specification of the Diameter base protocol, the Vendor-Specific-Application-Id AVP is still listed as a required AVP (an AVP indicated as {AVP}) in the command code format specifications defined in this specification to avoid backward compatibility issues, even if the use of this AVP has been deprecated in the new specification of the Diameter base protocol (IETF RFC 6733 [15]).

The command codes for the Sh-ee application of IMS events subscription/notification are taken from the range allocated by IANA in IETF RFC 3589 [7] as assigned in this specification. For these commands, the Application-ID field shall be set to xx1 (application identifier of the Sh-ee application, allocated by IANA).

The following Command Codes are defined in this specification:

Table 8.1.1: Command-Code values

| Command-Name | Abbreviation | Code | Clause |
| --- | --- | --- | --- |
| Event-Subscribe-Request | ESR | zz1 | 8.1.1 |
| Event-Subscribe-Answer | ESA | zz1 | 8.1.2 |
| Event-Notification-Request | ENR | zz2 | 8.1.3 |
| Event-Notification-Answer | ENA | zz2 | 8.1.4 |

8.1.1	Event-Subscribe-Request (ESR) Command

The Event-Subscribe-Request (ESR) command, indicated by the Command-Code field set to zz1 and the 'R' bit set in the Command Flags field, is sent by a Diameter Multimedia client to a Diameter Multimedia server in order to subscribe to notifications for when a specific IMS event for a specified IMS Public User Identity is updated.

Message Format

<Event-Subscribe-Request> ::=	< Diameter Header: zz1, REQ, PXY, xx1 >

< Session-Id >

[ DRMP ]

{ Vendor-Specific-Application-Id }

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ Destination-Host ]

{ Destination-Realm }

[ User-Name ]

[ OC-Supported-Features ]

*[ Supported-Features ]

[ Public-Identity ]

[ Any-User-Indication ]

{ Subs-Req-Type }

[ Send-Data-Indication ]

[ Expiry-Time ]

{ Event-Reporting-Options }

{ Event-List }

{ Notification-Address }

[ Notification-Correlation-Identity ]

*[ AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

8.1.2	Event-Subscribe-Answer (ESA) Command

The Event-Subscribe-Answer (ESA) command, indicated by the Command-Code field set to zz1 and the 'R' bit cleared in the Command Flags field, is sent by a server in response to the Event-Subscribe-Request command. The Experimental-Result AVP may contain one of the values defined in clause 8.2.

Message Format

<Event-Subscribe-Answer> ::=	< Diameter Header: zz1, PXY, xx1 >

< Session-Id >

[ DRMP ]

{ Vendor-Specific-Application-Id }

[ Result-Code ]

[ Experimental-Result ]

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ OC-Supported-Features ]

[ OC-OLR ]

*[ Load ]

*[ Supported-Features ]

[ Ims-Event-Report ]

*{ Event-List }

{ Subscription-Identity }

{ Expiry-Time }

*[ AVP ]

[ Failed-AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

8.1.3	Event-Notification-Request (ENR) Command

The Event-Notification-Request (ENR) command, indicated by the Command-Code field set to zz2 and the 'R' bit set in the Command Flags field, is sent by a Diameter Multimedia client to a Diameter Multimedia server in order to notify IMS event for which the HSS previously had subscribed.

Message Format

<Event-Notification-Request> ::=	< Diameter Header: zz2, PXY, xx1 >

< Session-Id >

[ DRMP ]

{ Vendor-Specific-Application-Id }

[ Result-Code ]

[ Experimental-Result ]

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ OC-Supported-Features ]

[ OC-OLR ]

*[ Load ]

*[ Supported-Features ]

{ Public-Identity }

{ Ims-Event }

[ Ims-Event-Report ]

{ Notification-Correction-Identity }

{ Timestamp }

*[ AVP ]

[ Failed-AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

8.1.4	Event-Notification-Answer (ENA) Command

The Event-Notification-Answer (ENA) command, indicated by the Command-Code field set to zz2 and the 'R' bit cleared in the Command Flags field, is sent by a server in response to the Event-Notification-Request command. The Experimental-Result AVP may contain one of the values defined in clause 8.2.

Message Format

<Event-Notification-Answer> ::=	< Diameter Header: zz2, PXY, xx1 >

< Session-Id >

[ DRMP ]

{ Vendor-Specific-Application-Id }

[ Result-Code ]

[ Experimental-Result ]

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ OC-Supported-Features ]

[ OC-OLR ]

*[ Load ]

*[ Supported-Features ]

*[ AVP ]

[ Failed-AVP ]

*[ Proxy-Info ]

*[ Route-Record ]

8.2	Result-Code AVP values

This clause defines new result code values that must be supported by all Diameter implementations that conform to this specification. The result codes defined in 3GPP TS 29.229 [6] are also applicable. When one of the result codes defined here is included in a response, it shall be inside an Experimental-Result AVP and Result-Code AVP shall be absent.

8.2.1	Success

Result codes that fall within the Success category are used to inform a peer that a request has been successfully completed.

No result codes within this category have been defined so far.

8.2.2	Permanent Failures

Errors that fall within the Permanent Failures category are used to inform the peer that the request failed, and should not be attempted again.

8.2.2.1	DIAMETER_INVALID_AVP_VALUE (5004)

The notification correlation identity received by the HSS is not supported or recognized.

8.2.2.2	DIAMETER_ERROR_OPERATION_NOT_ALLOWED (5101)

The requested operation is not allowed for the user.

8.2.3	Transient Failures

Errors that fall within the transient failures category are those used to inform a peer that the request could not be satisfied at the time that it was received. The request may be able to be satisfied in the future.

8.3	AVPs

The following table describes the Diameter application of Sh-ee for IMS events subscription/notification AVPs defined for the Sh interface protocol, their AVP Code values, types, possible flag values and whether the AVP may or not be encrypted.

Table 8.3.1: Diameter Application of IMS events subscription/notification AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encrypt |
| Public-Identity | 601 | 6.3.15 | UTF8String | M, V |  |  |  | No |
| Any-User-Indication | yy1 | 8.3.1 | Enumerated | M,V |  |  |  | No |
| Subscription-Identity | yy2 | 8.3.2 | UTF8String | M,V |  |  |  | No |
| Notification-Address | yy3 | 8.3.3 | UTF8String | M,V |  |  |  | No |
| Notification-Correlation-Identity | yy4 | 8.3.4 | UTF8String | M,V |  |  |  | No |
| Event-Reporting-Options | yy5 | 8.3.5 | Grouped | M,V |  |  |  | No |
| Report-Mode | yy6 | 8.3.6 | Enumerated | M,V |  |  |  | No |
| One-Time-Report | yy7 | 8.3.7 | Enumerated | M,V |  |  |  | No |
| Max-Number-of-Reports | yy8 | 8.3.8 | Unsigned32 | M,V |  |  |  | No |
| Report-Period | yy9 | 8.3.9 | Unsigned32 | M,V |  |  |  | No |
| Event-List | yy10 | 8.3.10 | Grouped | M,V |  |  |  | No |
| Ims-Event | y11 | 8.3.11 | Enumerated | M,V |  |  |  | No |
| Ims-Event-Filter | yy12 | 8.3.12 | Grouped | M,V |  |  |  | No |
| Event-Report | yy13 | 8.3.13 | Grouped | M,V |  |  |  | No |
| Ims-Event-Report-Information | yy14 | 8.3.14 | Grouped | M,V |  |  |  | No |
| Adc-Report | yy15 | 8.3.15 | Enumerated | M,V |  |  |  | No |
| Bdc-Report | yy16 | 8.3.16 | Enumerated | M,V |  |  |  | No |
| Session-Identity | yy17 | 8.3.17 | UTF8String | M,V |  |  |  | No |
| Timestamp | yy18 | 8.3.18 | Time | M,V |  |  |  | No |
| App-Bind-Information | yy19 | 8.3.19 | Grouped | M,V |  |  |  | No |
| App-Dc-Information | yy20 | 8.3.20 | Grouped | M,V |  |  |  | No |
| Application-Identity | yy21 | 8.3.21 | UTF8String | M,V |  |  |  | No |
| Stream-Identity | yy22 | 8.3.22 | UTF8String | M,V |  |  |  | No |
| Adc-Endpoint-Type | yy23 | 8.3.23 | Enumerated | M,V |  |  |  | No |
| Calling-Identity | yy24 | 8.3.24 | UTF8String | M,V |  |  |  | No |
| Called-Identity | yy25 | 8.3.25 | UTF8String | M,V |  |  |  | No |
| NOTE 1: The AVP header bit denoted as 'M', indicates whether support of the AVP is required. The AVP header bit denoted as 'V', indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see 3GPP TS 29.229 [6]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. NOTE 3: The value of these attributes is defined in IETF RFC 7683 [12]. NOTE 4: The value of this attribute is defined in IETF RFC 7944 [13]. NOTE 5: The value of this attribute is defined in IETF RFC 8583 [14]. |  |  |  |  |  |  |  |  |

8.3.1	Any-User-Indication AVP

The Any-User-Indication AVP is of type Enumerated and indicates whether the request applies to any UE. The following values are defined:

ANY_UE_NOT_APPLICABLE (0)

The request indicates that the request is not applicable for any UE.

ANY_UE_ APPLICABLE (1)

The request indicates that the request is applicable for any UE.

8.3.2	Subscription-Identity AVP

The Subscription-Identity AVP is of type UTF8String. This AVP contains the subscription ID.

8.3.3	Notification-Address AVP

The Notification-Address AVP is of type UTF8String. This AVP contains the recipient of notifications sent by the NF consumer. The syntax of this AVP corresponds with the format defined in IETF RFC 3986 [y].

8.3.4	Notification-Correlation-Identity AVP

The Notification-Correlation-Identity AVP is of type UTF8String. This AVP contains the notification correlation ID shall be present by NF consumer in subscription.

8.3.5	Event-Reporting-Options AVP

The Event-Reporting-Options AVP is of type Grouped. This AVP shall contain Report-Mode AVP, One-Time-Report AVP, One-Time-Notification AVP, Max-Number-of-Reports AVP, Report-Period AVP, and Expiry Time AVP.

AVP format

Event-Reporting-Options ::= <AVP header: yy5 10415>

[ Report-Mode ]

[ One-Time-Report ]

[ One-Time-Notification ]

[ Max-Number-of-Reports ]

[ Report-Period ]

[ Expiry Time ]

*[ AVP ]

8.3.6	Report-Mode AVP

The Report-Mode AVP is of type Enumerated and indicates the mode of report. The following values are defined:

ON_EVENT_DETECTION (0)

The request indicates that the report is sent based on event detection.

PERIODIC (1)

The request indicates that the report is periodically sent.

8.3.7	One-Time-Report AVP

The One-Time-Report AVP is of type Enumerated and indicates whether one time report is requested if immediate report is requested. The following values are defined:

ONE_TIME_REPORT_NOT_REQUESTED (0)

The request indicates that continuous notification is requested after the immediate report (if any) with the currently associated status is returned in the response.

ONE_TIME_REPORT_REQUESTED (1)

The request indicates that a single notification or status report is requested.

8.3.8	Max-Number-of-Reports AVP

The Max-Number-of-Reports AVP is of type Unsigned32. This AVP contains the maximum number of reports.

8.3.9	Report-Period AVP

The Report-Period AVP is of type Unsigned32. This AVP contains the interval time between which the event notification is reported, shall be present if reportMode is "PERIODIC".

8.3.10	Event-List AVP

The Event-List AVP is of type Grouped. This AVP shall contain Ims-Event AVP, Event Information AVP, Request Current Status AVP and Ims Event Filter AVP.

AVP format

Event-List ::= <AVP header: yy10 10415>

{ Ims-Event }

[ Request Current Status ]

*[ Ims Event Filter ]

*[ AVP ]

8.3.11	Ims-Event AVP

The Ims-Event AVP is of type Enumerated and indicates the IMS event. The following values are defined:

ADC_ESTABLISHMENT (0)

The request indicates the application data channel establishment event.

ADC_RELEASE (1)

The request indicates the application data channel release event.

BDC_ESTABLISHMENT (2)

The request indicates bootstrap data channel establishment event.

BDC_RELEASE (3)

The request indicates the bootstrap data channel release event.

ADC_IWK_ESTABLISHMENT (4)

The request indicates the application data channel requiring interworking establishment event.

8.3.12	Ims-Event-Filter AVP

The Ims-Event-Filter AVP is of type Grouped. This AVP shall contain Calling-Identity AVP, Called-Identity AVP, App-Bin-Information AVP, and Session-Identity AVP.

AVP format

Ims-Event-Filter ::= <AVP header: yy1310415>

[ Calling-Identity ]

[ Called-Identity ]

[ App-Bind-Information ]

[ Session-Identity ]

*[ AVP ]

8.3.13	Ims-Event-Report AVP

The Ims-Event-Report AVP is of type Grouped. This AVP shall contain Ims-Event AVP, Ims-Event-Report-Information AVP and Session-Identity AVP.

AVP format

Ims-Event-Report ::= <AVP header: yy14 10415>

{ Ims-Event }

[ Ims-Event-Report-Information ]

[ Session-Identity ]

*[ AVP ]

8.3.14	Ims-Event-Report-Information AVP

The Ims-Event-Report-Information AVP is of type Grouped. This AVP shall contain Adc-Report AVP and Bdc-Report AVP.

AVP format

Ims-Event-Report-Information ::= <AVP header: yy15 10415>

[ Adc-Report ]

[ Bdc-Report ]

*[ AVP ]

8.3.15	Adc-Report AVP

The Adc-Report AVP is of type Enumerated and indicates the ADC event status. The following values are defined:

ADC_ESTABLISHMENT (0)

The request indicates the application data channel establishment event.

ADC_RELEASE (1)

The request indicates the application data channel release event.

ADC_IWK_ESTABLISHMENT (2)

The request indicates the application data channel requiring interworking establishment event.

8.3.16	Bdc-Report AVP

The Bdc-Report AVP is of type Enumerated and indicates the BDC event status. The following values are defined:

BDC_ESTABLISHMENT (0)

The request indicates bootstrap data channel establishment event.

BDC_RELEASE (1)

The request indicates the bootstrap data channel release event.

8.3.17	Session-Identity AVP

The Session-Identity AVP is of type UTF8String. This AVP contains the IMS session ID.

8.3.18	Timestamp AVP

The Timestamp AVP is of type Time. This AVP contains the time at which the event is observed.

8.3.19	App-Bind-Information AVP

The App-Bind-Information AVP is of type Grouped. This AVP contains the data channel application binding information.

AVP format

App-Bind-Information ::= <AVP header: yy20 10415>

{ Application-Identity }

*[ App-Dc-Information ]

*[AVP]

8.3.20	App-Dc-Information AVP

The App-Dc-Information AVP is of type Grouped. This AVP represents information of the remote endpoint type of application data channels.

AVP format

App-Dc-Information ::= <AVP header: yy21 10415>

{ Stream-Identity }

[ Adc-Endpoint-Type ]

*[AVP]

8.3.21	Application-Identity AVP

The Application-Identity AVP is of type UTF8String. This AVP contains the application ID.

8.3.22	Stream-Identity AVP

The Stream-Identity AVP is of type UTF8String. This AVP contains the stream ID of the data channel.

8.3.23	Adc-Endpoint-Type AVP

The Adc-Endpoint-Type AVP is of type Enumerated and indicates the remote endpoint type of the ADC. The following values are defined:

UE (0)

The request indicates the remote endpoint type of the application data channel is the peer UE.

SERVER (1)

The request indicates the remote endpoint type of the application data channel is the application server.

8.3.24	Calling-Identity AVP

The Calling-Identity AVP is of type UTF8String. This AVP contains the public identity of the calling subscriber. The exact content and format of this AVP corresponds to the Public-Identity AVP described in 3GPP TS 29.229 [6].

8.3.25	Called-Identity AVP

The Called-Identity AVP is of type UTF8String. This AVP contains the public identity of the called subscriber. The exact content and format of this AVP corresponds to the Public-Identity AVP described in 3GPP TS 29.229 [6].

8.4	Use of namespaces

This clause contains the namespaces that have either been created in this specification, or the values assigned to existing namespaces managed by IANA.

8.4.1	AVP codes

This specification assigns the AVP values from the AVP Code namespace managed by 3GPP for its Diameter vendor-specific applications. See clause 6.3 for the assignment of the namespace in this specification.

8.4.2	Experimental-Result-Code AVP values

This specification has assigned Experimental-Result-Code AVP values 4100-4101 and 5100-5105. See clause 8.2.

8.4.3	Command Code values

This specification assigns the values xx2-xx3 from the range allocated by IANA to 3GPP in IETF RFC 3589 [7].

8.4.4	Application-ID value

IANA has allocated the value xx1 for the 3GPP Sh-ee application.

* * * End of Changes * * * *