3GPP TSG-CT WG4 Meeting #131	C4-254381

# Sophia Antipolis, France; 13th – 17th October 2025									   was C4-254366

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.272 | CR | 0882 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Disaster Roaming Ceased |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CATT, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In disaster roaming situation, the HSS may possibly know the disaster condition is terminated (e.g. by implementation, see S2-2202903 and C4-223237 for MINT phase 1 discussion). In this case, it is possible for the HSS to trigger Cancel Location procedure by indicating the reason of "DISASTER_CONDITION_TERMINATED". Furthermore, the MM cause mapping needs to be specified. According to TS24.301 clause 5.5.2.3.1, EMM cause value #13 shall be returned by the MME in this case: If UE supporting MINT-EPS is attached for disaster roaming services in EPS and the network de-registration is triggered due to the disaster condition no longer being applicable in the current location of the UE, the network shall set the EMM cause value to #13 "roaming not allowed in this tracking area" and may include a disaster return wait range in the Disaster return wait range IE in the DETACH REQUEST message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are made: - in clause 5.2.1.2.1, add trigger condition of CLR for supporting HSS cease of diaster roaming service; - in clause 5.2.1.2.3, address HSS behaviour of sending CLR triggering cease of disaster roaming service; - clause 7.3.24, define new Cancellation Type due to stopping of disater roaming. - Annex A, update the MM cause mapping (i.e. EMM cause #13) in disaster condition cease case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible for the HSS to trigger CLR procedure to stop disaster roaming servcie. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.2.1, 5.2.1.2.2, 5.2.1.2.3, 7.3.24, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#3: - Add MM cause mapping to Annex A. According to TS24.301, when NW deregisters UE due to disaster condition ceased, EMM cause value #13 is returned. Rev#2: - Correct the tdoc# in the coversheet. Rev#1: - Clarify that the HSS only sends Cancel Location Update message to the previous MME that has indicated the Disaster Roaming Indication in the ULR message; - Clarify the MME behaviour on receiving the Cancel Location Update with cancellation type set to "DISATER_CONDITION_TERMINATED", e.g. for statistics usage; - Change CR category to B. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.1.2.1	General

The Cancel Location Procedure shall be used between the HSS and the MME and between the HSS and the SGSN to delete a subscriber record from the MME or SGSN. The procedure shall be invoked by the HSS and is used:

-	to inform the MME or SGSN about a subscription withdrawal, or a change in the subscriber profile that does not allow PS services anymore (e.g., the Network Access Mode does not allow PS services), or a change in the subscriber profile that does not allow access to EPC anymore, or

-	to inform the MME or SGSN about an ongoing update procedure i.e. MME or SGSN change, or

-	to inform the MME or SGSN about an initial attach procedure, or

-	to inform the MME about the disaster condition terminated.

This procedure is mapped to the commands Cancel-Location-Request/Answer (CLR/CLA) in the Diameter application specified in clause 7.

Table 5.2.1.2.1/1 specifies the involved information elements for the request.

Table 5.2.1.2.1/2 specifies the involved information elements for the answer.

Table 5.2.1.2.1/1: Cancel Location Request

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| IMSI | User-Name (See IETF RFC 6733 [61]) | M | This information element shall contain the user IMSI, formatted according to 3GPP TS 23.003 [3], clause 2.2. |
| Supported Features (See 3GPP TS 29.229 [9]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |
| Cancellation Type (See 7.3.24) | Cancellation-Type | M | Defined values that can be used are: - MME-Update Procedure, - SGSN-Update Procedure, - Subscription Withdrawal, - Update Procedure_IWF, - Initial Attach Procedure. |
| CLR Flags (See 7.3.152) | CLR-Flags | O | This Information Element contains a bit mask. See 7.3.152 for the meaning of the bits and the condition for each bit to be set or not. |

Table 5.2.1.2.1/2: Cancel Location Answer

| Information element name | Mapping to Diameter AVP | Cat. | Description |
| --- | --- | --- | --- |
| Supported Features (See 3GPP TS 29.229 [9]) | Supported-Features | O | If present, this information element shall contain the list of features supported by the origin host. |
| Result (See 7.4) | Result-Code / Experimental-Result | M | The result of the operation. The Result-Code AVP shall be used to indicate success / errors as defined in the Diameter base protocol (see IETF RFC 6733 [61]). |

* * * Next Change * * * *

##### 5.2.1.2.2	Detailed behaviour of the MME and the SGSN

When receiving a Cancel Location request the MME or SGSN shall check whether the IMSI is known.

If it is not known, a result code of DIAMETER_SUCCESS is returned.

If it is known, the MME or SGSN shall check the Cancellation Type and act accordingly.

If the Cancellation Type is "Subscription Withdrawal", the MME or SGSN shall delete the subscription data and detach the UE; in addition, if the Reattach-Required flag is set, the MME or SGSN shall indicate to the UE to initiate an immediate re-attach procedure, as described in 3GPP TS 23.401 [2] and 3GPP TS 23.060 [12]. A result code of DIAMETER_SUCCESS shall be returned.

If a cancellation type of "Initial Attach Procedure" is received, the MME or SGSN shall not delete the subscription data. For details see 3GPP TS 23.401 [2] and 3GPP TS 23.060 [12]. If the MME receives this cancelation type, and it is registered for SMS, it shall consider itself as unregistered for SMS. Also in this case a result code of DIAMETER_SUCCESS shall be returned.

When a UE is served by a single combined MME/SGSN for both E-UTRAN and non-E-UTRAN access, the combined MME/SGSN shall check the Cancellation-Type. If it indicates Subscription Withdrawal or Update Procedure_IWF, the CLR is processed both in the MME part and in the SGSN part of the combined node. If it indicates Initial Attach Procedure, and if the CLR-Flags AVP is received and supported by the combined MME/SGSN, the CLR is processed only in the affected part of the combined node as indicated by the "S6a/S6d-Indicator" flag in the CLR-Flags AVP. Otherwise, the CLR is processed only in the affected part of the combined node and subscription data are kept for the not affected part.

If a cancellation type of "DISASTER_CONDITION_TERMINATED" is received, the MME shall delete the subscription data. The MME may record this cancellation type for network management usage (e.g. for statistics).

* * * Next Change * * * *

##### 5.2.1.2.3	Detailed behaviour of the HSS

The HSS shall make use of this procedure when the subscription is withdrawn by the HSS operator, and when the HSS detects that the UE has moved to a new MME or SGSN area, and when EPC access is not allowed due to Core Network Restrictions.

The HSS+UDM shall also make use of this procedure when the HSS+UDM detects that the UE has moved to a new AMF area, if the AMF indicates to the HSS+UDM to cancel MME and/or SGSN. The HSS+UDM shall include a cancellation type as specified in clause 5.4.2.2 of 3GPP TS 29.563 [70].

The HSS shall include a cancellation type of "Subscription Withdrawal" if the subscription is withdrawn by the operator, or if the subscriber profile does not allow PS services anymore, or if the Core Network Restrictions do not allow access to EPC anymore; the HSS may set the Reattach-Required flag in order to request the MME or the SGSN to trigger an immediate reattachment of the UE.

The HSS shall include a cancellation type of "MME Update Procedure" if the UE moved to a new MME area.

The HSS shall include a cancellation type of "SGSN Update Procedure" if the UE moved to a new SGSN area.

The HSS shall include a cancellation type of "Initial Attach Procedure" if the cancel location is initiated due to an Initial Attach from the UE.

The HSS shall include the CLR-Flags with the "S6a/S6d-Indicator" flag indicating the affected part of the combined node if the cancel location is to be sent to a combined MME/SGSN during initial attach procedure.

The HSS may make use of this procedure when the disaster condition is no longer being applicable while the UE is attached for disaster roaming service (i.e. the MME has previously indicated the Disaster Roaming Indication in the ULR message). The HSS shall include a cancellation type of "DISASTER_CONDITION_TERMINATION".

* * * Next Change * * * *

### 7.3.24	Cancellation-Type

The Cancellation-Type AVP is of type Enumerated and indicates the type of cancellation. The following values are defined:

MME_UPDATE_PROCEDURE (0)

This value is used when the Cancel Location is sent to the previous MME due to a received Update Location message from a new MME or due to the HSS+UDM receiving an Nudm_UEContextManagement service request from the AMF or due to the HSS receiving Nhss_UECM_SNDeregistration service operation from UDM (see clause 5.4.2.2 of 3GPP TS 29.563 [70]).

SGSN_UPDATE_PROCEDURE (1)

This value is used when the Cancel Location is sent to the previous SGSN due to a received Update Location message from a new SGSN or due to the HSS+UDM receiving an Nudm_UEContextManagement service request from the AMF or due to the HSS receiving Nhss_UECM_SNDeregistration service operation from UDM (see clause 5.4.2.2 of 3GPP TS 29.563 [70]).

SUBSCRIPTION_WITHDRAWAL (2)

This value is used:

-	when the Cancel Location is sent by the HSS to the current MME or SGSN due to withdrawal of the user's subscription by the HSS operator;

-	when the Cancel VCSG Location is sent by the CSS to the current MME or SGSN due to withdrawal of the user's VPLMN CSG subscription by the CSS operator.

UPDATE_PROCEDURE_IWF (3)

This value is used by an IWF when interworking with a pre-Rel-8 HSS.

INITIAL_ATTACH_PROCEDURE (4)

This value is used when the Cancel Location is sent to the MME or SGSN due to a received Update Location message during initial attach procedure from an SGSN or MME respectively.

DISASTER_CONDITION_TERMINATED (X)

This value is used when the Cancel Location is sent to the MME due to disaster condition is no longer being applicable while the UE is attached for disaster roaming service.

* * * Next Change * * * *

Annex A (normative):
MME mapping table for S6a and NAS Cause Code values

When the UE initiates Attach, Tracking Area Update or Service Request, there may be the need for the MME to communicate with the HSS via S6a to retrieve authentication data and/or subscription data. If this retrieval is rejected by the HSS, the received Diameter-Result-Code values or Experimental-Result values need to be mapped to appropriate cause codes over NAS to the UE.

This mapping shall be as shown in Table A.1.

If the retrieval is successful, not needed (e.g. because data are already available) or not possible (e.g. because HSS is unavailable or overloaded), detected error conditions need to be mapped to appropriate cause codes over NAS to the UE.

This mapping shall be as shown in Table A.2.

Table A.1: Mapping from S6a error code to NAS Cause Code values

| Reject indication received at MME over S6a | NAS Cause Code sent to UE |
| --- | --- |
| DIAMETER_ERROR_USER_UNKNOWN (5001) | #8 "EPS services and non-EPS services not allowed" |
| DIAMETER_ERROR_UNKNOWN_EPS_SUBSCRIPTION (5420) without Error Diagnostic, or with Error Diagnostic of GPRS_DATA_SUBSCRIBED | #15 "No suitable cells in tracking area" |
| DIAMETER_ERROR_UNKNOWN_EPS_SUBSCRIPTION (5420) with Error Diagnostic of NO_GPRS_DATA_SUBSCRIBED | #7 "EPS services not allowed" |
| DIAMETER_ERROR_RAT_NOT_ALLOWED (5421) | #15 "No suitable cells in tracking area", or #13 "Roaming not allowed in this tracking area", or #12 "Tracking area not allowed" (NOTE 1) |
| DIAMETER_ERROR_ROAMING_NOT_ALLOWED (5004) , without Error Diagnostic | #11 "PLMN not allowed" |
| DIAMETER_ERROR_ROAMING_NOT_ALLOWED (5004), with Error Diagnostic of ODB_HPLMN_APN or ODB_VPLMN_APN | #14 "EPS services not allowed in this PLMN" |
| DIAMETER_ERROR_ROAMING_NOT_ALLOWED (5004), with Error Diagnostic of ODB_ALL_APN | #15 "No suitable cells in tracking area" |
| DIAMETER_AUTHORIZATION_REJECTED (5003) DIAMETER_UNABLE_TO_DELIVER (3002) DIAMETER_REALM_NOT_SERVED (3003) | #15 "No suitable cells in tracking area", or #17 "Network failure", or #42 "Severe network failure" (NOTE 1) |
| DIAMETER_UNABLE_TO_COMPLY (5012), DIAMETER_INVALID_AVP_VALUE (5004) DIAMETER_AVP_UNSUPPORTED (5001) DIAMETER_MISSING_AVP (5005) DIAMETER_RESOURCES_EXCEEDED (5006) DIAMETER_AVP_OCCURS_TOO_MANY_TIMES (5009) DIAMETER_AUTHENTICATION_DATA_UNAVAILABLE (4181) (NOTE 2) | #17 "Network failure" or #42 "Severe network failure" (NOTE 1) |
| NOTE 1: Any of those NAS Cause Code values may be sent to the UE, depending on operator's choice. NOTE 2: Any other permanent errors from the Diameter base protocol as defined in IETF RFC 6733 [61], not listed here, should be mapped to NAS Cause Code #17 "Network failure". |  |

Table A.2: Mapping from detected error condition to NAS Cause Code values

| Condition | NAS cause code sent to UE |
| --- | --- |
| The MME receives a SGsAP-LOCATION-UPDATE-REJECT message from the VLR indicating in the reject cause "IMSI unknown in HLR" or if the UE has packet only subscription. Only used in the Combined Tracking and Location Area Update procedure. | #2 "IMSI Unknown in HSS" |
| The MME receives in Update-Location-Answer message an indication of Roaming restricted in MME due to unsupported feature | #14 "EPS services not allowed in this PLMN" |
| The MME cannot service an UE generated request because CS domain is not available and SMS in MME is not supported. | #18 "CS domain not available" |
| The value OPERATOR_DETERMINED_BARRING is received in the Subscriber-Status AVP | #15 "No suitable cells in tracking area" |
| The HSS indicates that due to subscription to a "regionally restricted service" the UE is not allowed to operate in the tracking area. | #12 "Tracking area not allowed" |
| The CSG ID of the cell from where the UE has sent the TRACKING AREA UPDATE REQUEST message is not contained in the Allowed CSG list. | #25 "Not authorized for this CSG" |
| The MME detects that it cannot communicate with the HSS in the HPLMN of the subscriber. How the MME detect this is implementation specific. | #15 "No suitable cells in tracking area" #14 "EPS services not allowed in this PLMN" #111 "Protocol error, unspecified" NOTE: Any of those NAS Cause Code values may be sent to the UE, depending on operator's choice / configuration, e.g. NAS Cause Code #14 is to be sent to the UE if the network is an LTE only network. |
| The MME detects by internal configuration that roaming is not allowed. | #11 "PLMN not allowed" |
| The MME detects that it cannot send a request to the HSS due to HSS overload (see Annex C). | #22 "Congestion" #42 "Severe network failure" NOTE 1: Cause #22 should be used. In addition, the MME may ask the UE not to retry before a backoff timer expires, based on an operator policy. The eventual timer value may take into account the value received in the corresponding active overload report and operator policy. NOTE 2: Cause #42 may be used, for attach requests, in case of severe overload, according to operator policy. |
| The MME receives in Cancel-Location-Request message an cancellation type set to "DISASTER_CONDITION_TERMINATED". | #13 "roaming not allowed in this tracking area" |

* * * End of Change * * * *