3GPP TSG-CT WG4 Meeting #132	C4-255292

Dallas, US; 17th – 21st November 2025	C4-255168

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 1005 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Allowed VLAN tags and VLAN Handling Information |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_VLANSUB |  |  |  |  |  | Date: |  |  |  | 2025-11-20 |  |
| Category: | F |  |  |  |  |  |  | Release: |  |  |  | Rel-19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |
| Reason for change: | It is specified how the VLAN Handling Information (vlanTagHandlingInfo) is handled in the SMF and the UPF as below: (in clause 5.13.1) The SMF shall also determine instructions on VLAN handling, i.e. using the Outer Header Creation IE to insert a C-TAG and/or S-TAG for the UL packets, and the Outer Header Removal to remove the C-TAG and/or S-TAG for the DL packets correspondingly, based on local configuration, or VLAN handling information in subscription data received from UDM, or VLAN handling information received from the DN-AAA, as specified in clause 5.6.10.2 of 3GPP TS 23.501 [28]. So, the DL Ethernet packets received over N6 are expected to contain the S-TAG and/or C-TAG which was inserted by the UPF when forwarding the Ethernet packets to the data network. It is not clear if such S-TAG and/or C-TAG should be included in the list of allowed VLAN tages, i.e. in the vlanTagAllowed received from the UDM. For example, a PDU session is allowed for Ethernet traffic with VLAN tage 1, 2 and 3 (vlanTagAllowed 1,2 and3), and the UPF is required to insert S-TAG 100 for uplink and remove the same for DL, then, should 100 be added also in the allowed list? Functional wise, there is no need to provide such S-TAG and/or C-TAG (included in the vlanTagHandlingInfo) ALSO in the vlanTagAllowed, more importantly, if 100 is added into vlanTagAllowed, this means for the uplink direction, before the UPF inserts 100, Ethernet packet with VLAN 100 is allowed, this is against the original intention to allow Ethernet traffic with VLAN tag set to 1, 2 and 3. So, it is proposed to clarify that the VLAN Tag value included in the vlanTagHandlingInfo should not be included in the vlanTagAllowed. The C-TAG and/or S-TAG included in the vlanTagHandlingInfo are considered allowed when the UPF does packet matching for DL packets received over N6. |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: | Clarify that the VLAN Tag value included in the vlanTagHandlingInfo should not be included in the vlanTagAllowed. The C-TAG and/or S-TAG included in the vlanTagHandlingInfo are considered allowed when the UPF does packet matching for DL packets received over N6. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: | Ambiguous requirements leads to interworking problem. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: | 5.13.7 |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | N |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: | Rev1: add description for the scenario when an Ethernet PDU session having both a list of allowed VLAN tags and VLAN handling information provisioned and some wording improvement on the VLAN handling information. |  |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

### 5.13.7	Handling of allowed VLAN tags and VLAN Handling Information

The stage 2 requirements related to the handling of allowed VLAN tags and VLAN Handling Information are specified in clauses 5.6.6 and 5.6.10.2 of 3GPP TS 23.501 [28] and clause 5.2.3.3.1 of 3GPP TS 23.502 [29].

The SMF may be provisioned with a list of allowed VLAN tags (for a maximum of 16 VLAN tags) from the UDM or the DN-AAA or may be locally configured for a PDU Session. If allowed VLAN tags are available for a PDU Session, the Ethernet packets (of the PDU session) shall be only allowed when the outer VLAN tag (which is the one closest to the Ethernet header) is set to one of the allowed VLAN tags. Based on local configuration for the DNN/S-NSSAI, e.g. depending on whether the DL Ethernet packets contains one tag (which is either a C-TAG or a S-TAG) or two tags (i.e. a S-TAG and a C-TAG), the SMF shall decide whether the allowed VLAN tags correspond to a C-TAG or S-TAG and the SMF shall provision them accordingly in the UPF in the C-TAG and/or S-TAG IEs in the Ethernet Packet Filter IE (see Table 7.5.2.2-3). Ethernet packets which are not matching any Ethernet Packet Filter shall be dropped by the UPF.

The SMF shall also determine instructions on VLAN handling, based on local configuration, or VLAN handling information in subscription data received from UDM, or VLAN handling information received from the DN-AAA, as specified in clause 5.6.10.2 of 3GPP TS 23.501 [28] and it shall then provision:

-	an UL FAR containing the Outer Header Creation IE to insert a C-TAG and/or S-TAG for the UL packets; and

-	a DL PDR with the Ethernet Packet Filter IE including the C-TAG and/or S-TAG which was inserted in the UL direction to match the DL packets and the Outer Header Removal IE to remove the C-TAG and/or S-TAG.

NOTE:	The C-TAG and/or S-TAG included in the vlanTagHandlingInfo are considered allowed when the UPF performs packet matching for DL packets received over N6, therefore, these tags need not be included in the vlanTagAllowed.

For an Ethernet PDU session having both a list of allowed VLAN tags and VLAN handling information provisioned, the SMF shall proceed as specified above with the difference that the SMF shall provision a DL PDR comprising the VLAN tag (S-TAG) from the VLAN handling instructions and the allowed VLAN tag (C-TAG).

# *** End of Changes ***