3GPP TSG-CT WG4 Meeting #132	C4-255065

# Dallas, United States, 17 - 21 November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1273 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Signalling Storm Analytics data collection from AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The stage2 for Signalling Storm Analytics details the input of timer information (e.g., back-off timer) as a necessary part of the signalling storm mitigation strategy. According to TS23.288, Table 6.22.2-1: UE related Context Data collection Information; Source; Description. timer information; AMF, SMF; Timer information which has been set for the UE, such as timer type, duration.. Observation1: Stage2 defines timer informaiton which has been set by AMF or SMF to be collected as part of UE related Context Data collection. According to TS29.518, AmfTimerInfo in AmfEventReport can report amfTimerType and timerDuration, but with respect to back-off timer, it can not indicate to which DNN and/or S-NSSAI the property is associated. Observation2: TS29.518 lacks the property definiton for timer infomraiton collection. According to TS24.501 (5.4.5.2.4), e.g., Upon reception of an UL NAS TRANSPORT message, […] DNN based congestion control is activated for the DNN included in the UL NAS TRANSPORT message, or DNN based congestion control is activated for the selected DNN in case of no DNN included in the UL NAS TRANSPORT message, e.g. configured by operation and maintenance, the AMF shall send back to the UE the 5GSM message which was not forwarded, a back-off timer value and 5GMM cause #22 "congestion". #same for S-NSSAI and DNN based congestion control and S-NSSAI only based congestion control Observation3: When the AMF can not forward the SM message to the SMF, it is the AMF(not the SMF) sets the back-off timer for the UE. Considering the case where the AMF sets following back-off timer to UE A and UE A is running the timers. - back-off timer#1 associated to [DNN#1] - back-off timer#2 associated to [DNN#1, SNSSAI#1] - back-off timer#3 associated to [SNSSAI#1] For the NWDAF, it requires the ability to receive the back-off timer's context (the DNN and/or S-NSSAI association) to fully analyze the congestion patterns. Without this context, the NWDAF cannot report necessary information for the signalling storm mitigation. Observation4: As part of input data, the back-off timer's context information (the DNN and/or S-NSSAI association) needs to be collected by NWDAF. |
| Summary of change: |  | Adding a propery to convey back-off timer's contextual information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the back-off timer's critical contextual information. NWDAF will not be able to perform effective analysis for signalling storm mitigation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.1, 6.2.6.2.45, 6.2.6.2.x(new), 6.2.6.3.x(new), A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible feature in the OpenAPI file of TS29518_Namf_EventExposure |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Change * * * *

6.2.6.2.45	Type: AmfTimerInfo

Table 6.2.6.2.45-1: Definition of type AmfTimerInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfTimerType | string | M | 1 | Indicates the AMF timer type as described in Table 10.2.1 of 3GPP TS 24.501 [11] and the back-off timer type as described in Table 10.3.1 of 3GPP TS 24.501 [11]. |
| timerDuration | DurationSec | M | 1 | Indicates the AMF timer duration |
| dnn | Dnn | O | 1 | Represents the dnn associated to the timer. |
| snssai | Snssai | O | 1 | Represents the snssai associated to the timer. |

* * * Next Change * * * *

A.3	Namf_EventExposure API

openapi: 3.0.0

info:

version: 1.4.0-alpha.4

title: Namf_EventExposure

description: |

AMF Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- namf-evts

externalDocs:

description: 3GPP TS 29.518 V19.4.0; 5G System; Access and Mobility Management Services

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.518/'

servers:

- url: '{apiRoot}/namf-evts/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/subscriptions:

post:

summary: Namf_EventExposure Subscribe service Operation

tags:

- Subscriptions collection (Collection)

operationId: CreateSubscription

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AmfCreateEventSubscription'

required: true

responses:

'201':

description: Subsription Created

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/namf-evts/<apiVersion>/subscriptions/{subscriptionId}'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/AmfCreatedEventSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

onEventReport:

'{$request.body#/subscription/eventNotifyUri}':

post:

summary: Event Notificaiton Delivery

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AmfEventNotification'

required: true

responses:

'204':

description: Successful acknowledgement

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

onSubscriptionIdChangeEvtReport:

'{$request.body#/subscription/subsChangeNotifyUri}':

post:

summary: Event Notificaiton Delivery For Subscription Id Change

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AmfEventNotification'

required: true

responses:

'204':

description: Successful acknowledgement

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/subscriptions/{subscriptionId}:

patch:

summary: Namf_EventExposure Subscribe Modify service Operation

tags:

- Individual subscription (Document)

operationId: ModifySubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be modified

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

oneOf:

- type: array

items:

$ref: '#/components/schemas/AmfUpdateEventSubscriptionItem'

minItems: 1

- type: array

items:

$ref: '#/components/schemas/AmfUpdateEventOptionItem'

minItems: 1

maxItems: 1

required: true

responses:

'200':

description: Subsription modified successfully

content:

application/json:

schema:

$ref: '#/components/schemas/AmfUpdatedEventSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: Namf_EventExposure Unsubscribe service Operation

tags:

- Individual subscription (Document)

operationId: DeleteSubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be deleted

schema:

type: string

responses:

'204':

description: Subsription deleted successfully

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/set-subscriptions:

post:

summary: Namf_EventExposure Subscribe service Operation

tags:

- AMF Set level Bulk Subscriptions collection (Collection)

operationId: CreateAMFSetLevelBulkSubscription

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AmfCreateEventSubscription'

required: true

responses:

'201':

description: Subsription Created

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/namf-evts/<apiVersion>/set-subscriptions/{subscriptionId}'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/AmfCreatedEventSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

onEventReport:

'{$request.body#/subscription/eventNotifyUri}':

post:

summary: Event Notification Delivery

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AmfEventNotification'

required: true

responses:

'204':

description: Successful acknowledgement

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

onSubscriptionIdChangeEvtReport:

'{$request.body#/subscription/subsChangeNotifyUri}':

post:

summary: Event Notificaiton Delivery For Subscription Id Change

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AmfEventNotification'

required: true

responses:

'204':

description: Successful acknowledgement

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/set-subscriptions/{subscriptionId}:

patch:

summary: Namf_EventExposure Subscribe Modify service Operation

tags:

- Individual subscription (Document)

operationId: ModifyAMFSetLevelBulkSubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be modified

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

oneOf:

- type: array

items:

$ref: '#/components/schemas/AmfUpdateEventSubscriptionItem'

minItems: 1

- type: array

items:

$ref: '#/components/schemas/AmfUpdateEventOptionItem'

minItems: 1

maxItems: 1

required: true

responses:

'200':

description: Subsription modified successfully

content:

application/json:

schema:

$ref: '#/components/schemas/AmfUpdatedEventSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: Namf_EventExposure Unsubscribe service Operation

tags:

- Individual subscription (Document)

operationId: DeleteAMFSetLevelBulkSubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be deleted

schema:

type: string

responses:

'204':

description: Subsription deleted successfully

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

namf-evts: Access to the Namf_EventExposure API

schemas:

AmfEventSubscription:

description: Represents an individual event subscription resource on AMF

type: object

properties:

eventList:

type: array

items:

$ref: '#/components/schemas/AmfEvent'

minItems: 1

eventNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

type: string

nfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

subsChangeNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

subsChangeNotifyCorrelationId:

type: string

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

excludeSupiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

excludeGpsiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

includeSupiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

includeGpsiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

anyUE:

type: boolean

options:

$ref: '#/components/schemas/AmfEventMode'

sourceNfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

termNotifyInd:

type: boolean

required:

- eventList

- eventNotifyUri

- notifyCorrelationId

- nfId

AmfEvent:

description: Describes an event to be subscribed

type: object

properties:

type:

$ref: '#/components/schemas/AmfEventType'

immediateFlag:

type: boolean

default: false

areaList:

type: array

items:

$ref: '#/components/schemas/AmfEventArea'

minItems: 1

locationFilterList:

type: array

items:

$ref: '#/components/schemas/LocationFilter'

minItems: 1

locationTrendsFilterList:

type: array

items:

$ref: '#/components/schemas/LocationFilter'

minItems: 1

refId:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/ReferenceId'

trafficDescriptorList:

type: array

items:

$ref: '#/components/schemas/TrafficDescriptor'

minItems: 1

reportUeReachable:

type: boolean

default: false

reachabilityFilter:

$ref: '#/components/schemas/ReachabilityFilter'

udmDetectInd:

type: boolean

default: false

maxReports:

type: integer

presenceInfoList:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 1

description: A map(list of key-value pairs) where praId serves as key.

maxResponseTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

targetArea:

$ref: '#/components/schemas/TargetArea'

snssaiFilter:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

minItems: 1

ueInAreaFilter:

$ref: '#/components/schemas/UeInAreaFilter'

minInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

nextReport:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

idleStatusInd:

type: boolean

default: false

dispersionArea:

$ref: '#/components/schemas/DispersionArea'

nextPeriodicReportTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

adjustAoIOnRa:

type: boolean

default: false

ranTimingSynchroStatusChange:

type: boolean

default: false

notifyForSupiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

notifyForGroupList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

notifyForSnssaiDnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SnssaiDnnItem'

minItems: 1

reportingThreshold:

type: integer

assignTrajectory:

$ref: '#/components/schemas/Trajectory'

smCommFailureFilter:

$ref: '#/components/schemas/SmCommFailureFilter'

uePosCapRequestedInd:

type: boolean

enum:

- true

twList:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

uavAltitudeReportingConfig:

$ref: '#/components/schemas/UavAltitudeReportingConfig'

required:

- type

AmfEventNotification:

description: Data within a AMF Event Notification request

type: object

properties:

notifyCorrelationId:

type: string

subsChangeNotifyCorrelationId:

type: string

reportList:

type: array

items:

$ref: '#/components/schemas/AmfEventReport'

minItems: 1

eventSubsSyncInfo:

$ref: '#/components/schemas/AmfEventSubsSyncInfo'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

AmfEventReport:

description: Represents a report triggered by a subscribed event type

type: object

properties:

type:

$ref: '#/components/schemas/AmfEventType'

state:

$ref: '#/components/schemas/AmfEventState'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

subscriptionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

anyUe:

type: boolean

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

areaList:

type: array

items:

$ref: '#/components/schemas/AmfEventArea'

minItems: 1

refId:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/ReferenceId'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

location:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

additionalLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

timezone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

accessTypeList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

rmInfoList:

type: array

items:

$ref: '#/components/schemas/RmInfo'

minItems: 1

cmInfoList:

type: array

items:

$ref: '#/components/schemas/CmInfo'

minItems: 1

reachability:

$ref: '#/components/schemas/UeReachability'

commFailure:

$ref: '#/components/schemas/CommunicationFailure'

lossOfConnectReason:

$ref: '#/components/schemas/LossOfConnectivityReason'

numberOfUes:

type: integer

5gsUserStateList:

type: array

items:

$ref: '#/components/schemas/5GsUserStateInfo'

minItems: 1

typeCode:

type: string

pattern: '^imeitac-[0-9]{8}$'

registrationNumber:

type: integer

maxAvailabilityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ueIdExt:

type: array

items:

$ref: '#/components/schemas/UEIdExt'

minItems: 1

snssaiTaiList:

type: array

items:

$ref: '#/components/schemas/SnssaiTaiMapping'

minItems: 1

idleStatusIndication:

$ref: '#/components/schemas/IdleStatusIndication'

ueAccessBehaviorTrends:

type: array

items:

$ref: '#/components/schemas/UeAccessBehaviorReportItem'

minItems: 1

ueLocationTrends:

type: array

items:

$ref: '#/components/schemas/UeLocationTrendsReportItem'

minItems: 1

mmTransLocationReportList:

type: array

items:

$ref: '#/components/schemas/MmTransactionLocationReportItem'

minItems: 1

mmTransSliceReportList:

type: array

items:

$ref: '#/components/schemas/MmTransactionSliceReportItem'

minItems: 1

termReason:

$ref: '#/components/schemas/SubTerminationReason'

unavailabilityPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

trajRepReason:

$ref: '#/components/schemas/TrajectoryReportReason'

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

uePosCap:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/UePositioningCapabilities'

ueUpPosCap:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/UeUpPositioningCapabilities'

minItems: 1

signallingMeasurementInfo:

$ref: '#/components/schemas/SignallingMeasurementInfo'

uavAltRptFailureInd:

$ref: '#/components/schemas/UavAltRptFailureCause'

signallingMeasurementUeRelatedInfo:

$ref: '#/components/schemas/SignallingMeasurementUeRelatedInfo'

required:

- type

- state

- timeStamp

AmfEventMode:

description: Describes how the reports shall be generated by a subscribed event

type: object

properties:

trigger:

$ref: '#/components/schemas/AmfEventTrigger'

maxReports:

type: integer

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

sampRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

partitioningCriteria:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PartitioningCriteria'

minItems: 1

notifFlag:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationFlag'

mutingExcInstructions:

writeOnly: true

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/MutingExceptionInstructions'

mutingNotSettings:

readOnly: true

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/MutingNotificationsSettings'

varRepPeriodInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarRepPeriod'

minItems: 1

required:

- trigger

AmfEventState:

description: Represents the state of a subscribed event

type: object

properties:

active:

type: boolean

remainReports:

type: integer

remainDuration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- active

RmInfo:

description: Represents the registration state of a UE for an access type

type: object

properties:

rmState:

$ref: '#/components/schemas/RmState'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

required:

- rmState

- accessType

CmInfo:

description: Represents the connection management state of a UE for an access type

type: object

properties:

cmState:

$ref: '#/components/schemas/CmState'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

required:

- cmState

- accessType

CommunicationFailure:

description: Describes a communication failure detected by AMF

type: object

properties:

nasReleaseCode:

type: string

ranReleaseCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

AmfCreateEventSubscription:

description: Data within a create AMF event subscription request

type: object

properties:

subscription:

$ref: '#/components/schemas/AmfEventSubscription'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

oldGuami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

udrRestartInd:

type: boolean

default: false

required:

- subscription

AmfCreatedEventSubscription:

description: Data within a create AMF event subscription response

type: object

properties:

subscription:

$ref: '#/components/schemas/AmfEventSubscription'

subscriptionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

reportList:

type: array

items:

$ref: '#/components/schemas/AmfEventReport'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- subscription

- subscriptionId

AmfUpdateEventSubscriptionItem:

description: Document describing the modification(s) to an AMF Event Subscription

type: object

properties:

op:

type: string

enum:

- add

- remove

- replace

path:

type: string

pattern: '^\/eventList\/-|(\/eventList\/0|\/eventList\/[1-9][0-9]*){1}(\/presenceInfoList\/0|\/presenceInfoList\/[1-9][0-9]* |\/notifyForSupiList|\/notifyForGroupList|\/notifyForSnssaiDnnList)?|\/excludeSupiList|\/excludeGpsiList|\/includeSupiList|\/includeGpsiList$'

value: {}

required:

- op

- path

AmfUpdateEventOptionItem:

description: Document describing the modifications to AMF event subscription options

type: object

properties:

op:

type: string

enum:

- replace

path:

type: string

pattern: '^(\/options\/expiry|\/options\/notifFlag|\/options\/mutingExcInstructions)$'

value: {}

required:

- op

- path

- value

AmfUpdatedEventSubscription:

description: Represents a successful update on an AMF Event Subscription

type: object

properties:

subscription:

$ref: '#/components/schemas/AmfEventSubscription'

reportList:

type: array

items:

$ref: '#/components/schemas/AmfEventReport'

minItems: 1

required:

- subscription

AmfEventArea:

description: Represents an area to be monitored by an AMF event

type: object

properties:

presenceInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

ladnInfo:

$ref: '#/components/schemas/LadnInfo'

sliceAreaRestrictionInfo:

$ref: '#/components/schemas/SliceAreaRestrictionInfo'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nsiId:

$ref: 'TS29531_Nnssf_NSSelection.yaml#/components/schemas/NsiId'

LadnInfo:

description: LADN Information

type: object

properties:

ladn:

type: string

presence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

required:

- ladn

5GsUserStateInfo:

description: Represents the 5GS User state of the UE for an access type

type: object

properties:

5gsUserState:

$ref: '#/components/schemas/5GsUserState'

accessType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

required:

- 5gsUserState

- accessType

TrafficDescriptor:

description: Represents the Traffic Descriptor

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dddTrafficDescriptorList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DddTrafficDescriptor'

minItems: 1

UEIdExt:

description: UE Identity

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

uePosCap:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/UePositioningCapabilities'

ueUpPosCap:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/UeUpPositioningCapabilities'

minItems: 1

AmfEventSubsSyncInfo:

description: AMF Event Subscriptions Information for synchronization

type: object

properties:

subscriptionList:

type: array

items:

$ref: '#/components/schemas/AmfEventSubscriptionInfo'

minItems: 1

required:

- subscriptionList

AmfEventSubscriptionInfo:

description: Individual AMF Event Subscription Information

type: object

properties:

subId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

type: string

refIdList:

type: array

items:

$ref: 'TS29503_Nudm_EE.yaml#/components/schemas/ReferenceId'

minItems: 1

oldSubId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- subId

- refIdList

TargetArea:

description: TA list or TAI range list or any TA

type: object

properties:

taList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

taiRangeList:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/TaiRange'

minItems: 1

anyTa:

type: boolean

default: false

SnssaiTaiMapping:

description: List of restricted or unrestricted S-NSSAIs per TAI(s)

type: object

properties:

reportingArea:

$ref: '#/components/schemas/TargetArea'

accessTypeList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

supportedSnssaiList:

type: array

items:

$ref: '#/components/schemas/SupportedSnssai'

minItems: 1

required:

- reportingArea

UeAccessBehaviorReportItem:

description: Report Item for UE Access Behavior Trends event.

type: object

properties:

stateTransitionType:

$ref: '#/components/schemas/AccessStateTransitionType'

spacing:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

spacingVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

duration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

durationVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

occurrences:

type: integer

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- stateTransitionType

- spacing

- duration

IdleStatusIndication:

description: Represents the idle status indication.

type: object

properties:

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

activeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

subsRegTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

edrxCycleLength:

type: integer

suggestedNumOfDlPackets:

type: integer

UeInAreaFilter:

description: Additional filters for UE in Area Report event

type: object

properties:

ueType:

$ref: '#/components/schemas/UeType'

aerialSrvDnnInd:

type: boolean

default: false

ueIdOmitInd:

type: boolean

default: false

lcsBroadcastingAssistanceDataType:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/LcsBroadcastAssistanceTypesData'

SupportedSnssai:

description: Supported S-NSSAIs

type: object

properties:

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

restrictionInd:

type: boolean

default: false

required:

- sNssai

UeLocationTrendsReportItem:

description: Report Item for UE Location Trends event.

type: object

properties:

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

ranNodeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

n3gaLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/N3gaLocation'

spacing:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

spacingVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

duration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

durationVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

occurrences:

type: integer

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- spacing

- duration

- timestamp

DispersionArea:

description: Dispersion Area

type: object

properties:

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

ranNodeIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

minItems: 1

ncgiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

minItems: 1

ecgiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

minItems: 1

n3gaInd:

type: boolean

default: false

MmTransactionLocationReportItem:

description: UE MM Transaction Report Item per Location

type: object

properties:

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

n3gaLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/N3gaLocation'

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

transactions:

type: integer

required:

- timestamp

- transactions

MmTransactionSliceReportItem:

description: UE MM Transaction Report Item per Slice

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

transactions:

type: integer

required:

- timestamp

- transactions

SliceAreaRestrictionInfo:

description: Slice Area Restriction Information

type: object

properties:

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

presence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

required:

- sNssai

ScheduledArea:

description: A scheduled area in the trajectory

type: object

properties:

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 1

enterTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

leaveTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

altitudeLow:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

altitudeHigh:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

ranNodeIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

minItems: 1

ncgiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

minItems: 1

ecgiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

minItems: 1

anyOf:

- required: [ taiList ]

- required: [ ranNodeIdList ]

- required: [ ncgiList ]

- required: [ ecgiList ]

TrajectorySegment:

description: Trajectory segment

type: object

properties:

scheduledAreaList:

type: array

items:

$ref: '#/components/schemas/ScheduledArea'

minItems: 1

required:

- scheduledAreaList

Trajectory:

description: Trajectory

type: object

properties:

scheduledAreaList:

type: array

items:

$ref: '#/components/schemas/ScheduledArea'

minItems: 1

segmentList:

type: array

items:

$ref: '#/components/schemas/TrajectorySegment'

minItems: 1

acceptableDeviationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

SmCommFailureFilter:

description: Filter for Session Management Communication Failure

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- dnn

- snssai

SignallingMeasurementInfo:

description: AMF Signalling Measurement Information

type: object

properties:

nfSignallingInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSignallingInfo'

SignallingMeasurementUeRelatedInfo:

description: AMF Signalling Measurement for UE-related information

type: object

properties:

ueRelatedInfoList:

type: array

items:

$ref: '#/components/schemas/UeRelatedInfo'

minItems: 1

UeRelatedInfo:

description: UE related signalling information

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

ueSignallingInfoList:

type: array

items:

$ref: '#/components/schemas/UeSignallingInfo'

minItems: 1

ueStateInfoList:

type: array

items:

$ref: '#/components/schemas/UeStateInfo'

minItems: 1

timerInfoList:

type: array

items:

$ref: '#/components/schemas/AmfTimerInfo'

minItems: 1

UeSignallingInfo:

description: statistics related to signalling messages received from the UE or RAN

type: object

properties:

ueSignallingType:

$ref: '#/components/schemas/UeSignallingType'

numOfReq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgRespTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

numOfReqUnresp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

numOfReqReject:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

numOfPosteriorReq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

reasonOfFailure:

$ref: '#/components/schemas/FailureReason'

UeStateInfo:

description: statistics related to UE state information.

type: object

properties:

stateChangeType:

$ref: '#/components/schemas/StateChangeType'

count:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

AmfTimerInfo:

description: UE timer information.

type: object

properties:

amfTimerType:

type: string

timerDuration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- amfTimerType

- timerDuration

UavAltitudeReportingConfig:

description: altitude reporting

type: object

properties:

rptThresholdMin:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

rptThresholdMax:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

altReportPeriod:

type: integer

minimum: 120

maximum: 1800000

altReportEventTrigger:

type: array

items:

$ref: '#/components/schemas/AltReportEventTrigger'

minItems: 1

required:

- rptThresholdMin

- rptThresholdMax

AmfEventType:

description: Describes the supported event types of Namf_EventExposure Service

anyOf:

- type: string

enum:

- LOCATION_REPORT

- PRESENCE_IN_AOI_REPORT

- TIMEZONE_REPORT

- ACCESS_TYPE_REPORT

- REGISTRATION_STATE_REPORT

- CONNECTIVITY_STATE_REPORT

- REACHABILITY_REPORT

- COMMUNICATION_FAILURE_REPORT

- UES_IN_AREA_REPORT

- SUBSCRIPTION_ID_CHANGE

- SUBSCRIPTION_ID_ADDITION

- SUBSCRIPTION_TERMINATION

- LOSS_OF_CONNECTIVITY

- 5GS_USER_STATE_REPORT

- AVAILABILITY_AFTER_DDN_FAILURE

- TYPE_ALLOCATION_CODE_REPORT

- FREQUENT_MOBILITY_REGISTRATION_REPORT

- SNSSAI_TA_MAPPING_REPORT

- UE_LOCATION_TRENDS

- UE_ACCESS_BEHAVIOR_TRENDS

- UE_MM_TRANSACTION_REPORT

- TRAJECTORY_TRACKING_REPORT

- SIGNALLING_MEASUREMENT_REPORT

- SIGNALLING_MEASUREMENT_UE_RELATED_REPORT

- ALTITUDE_REPORT

- type: string

AmfEventTrigger:

description: Describes how AMF should generate the report for the event

anyOf:

- type: string

enum:

- ONE_TIME

- CONTINUOUS

- PERIODIC

- type: string

LocationFilter :

description: Describes the supported filters of LOCATION_REPORT event type

anyOf:

- type: string

enum:

- TAI

- CELL_ID

- RAN_NODE

- N3IWF

- UE_IP

- UDP_PORT

- TNAP_ID

- GLI

- TWAP_ID

- type: string

UeReachability:

description: Describes the reachability of the UE

anyOf:

- type: string

enum:

- UNREACHABLE

- REACHABLE

- REGULATORY_ONLY

- type: string

RmState:

description: Describes the registration management state of a UE

anyOf:

- type: string

enum:

- REGISTERED

- DEREGISTERED

- type: string

CmState:

description: Describes the connection management state of a UE

anyOf:

- type: string

enum:

- IDLE

- CONNECTED

- type: string

5GsUserState:

description: Describes the 5GS User State of a UE

anyOf:

- type: string

enum:

- DEREGISTERED

- CONNECTED_NOT_REACHABLE_FOR_PAGING

- CONNECTED_REACHABLE_FOR_PAGING

- NOT_PROVIDED_FROM_AMF

- type: string

LossOfConnectivityReason:

description: Describes the reason for loss of connectivity

anyOf:

- type: string

enum:

- DEREGISTERED

- MAX_DETECTION_TIME_EXPIRED

- PURGED

- UNAVAILABLE_PERIOD

- type: string

ReachabilityFilter:

description: Event filter for REACHABILITY_REPORT event type

anyOf:

- type: string

enum:

- UE_REACHABILITY_STATUS_CHANGE

- UE_REACHABLE_DL_TRAFFIC

- type: string

UeType:

description: Describes the type of UEs

anyOf:

- type: string

enum:

- AERIAL_UE

- type: string

AccessStateTransitionType:

description: Access State Transition Type.

anyOf:

- type: string

enum:

- ACCESS_TYPE_CHANGE_3GPP

- ACCESS_TYPE_CHANGE_N3GPP

- RM_STATE_CHANGE_DEREGISTERED

- RM_STATE_CHANGE_REGISTERED

- CM_STATE_CHANGE_IDLE

- CM_STATE_CHANGE_CONNECTED

- HANDOVER

- MOBILITY_REGISTRATION_UPDATE

- type: string

SubTerminationReason:

description: Subscription Termination Reason.

anyOf:

- type: string

enum:

- INVALID_SUBSCRIPTION

- SUBSCRIPTION_NOT_AUTHORIZED

- type: string

TrajectoryReportReason:

description: Trajectory Tracking Event Report Reason.

anyOf:

- type: string

enum:

- TRAJECTORY_DEVIATED

- TRAJECTORY_SEGMENT_CHANGE

- type: string

AltReportEventTrigger:

description: the event-based altitude reporting requirement reason.

anyOf:

- type: string

enum:

- TA_CHANGE

- RAN_NODE

- type: string

UavAltRptFailureCause:

description: Indicates when the UAV Flight information reporting request failed.

anyOf:

- type: string

enum:

- AERIAL_UE_FLIGHT_INFORMATION_REPORTING_FAILED

- AERIAL_UE_FLIGHT_INFORMATION_REPORTING_STOPPED

- type: string

FailureReason:

description: Reason of failed responses of UE/RAN.

anyOf:

- type: string

enum:

- SUBSCRIPTION_REJECT

- AUTHENTICATION_FAILURE

- SECURITY_MODE_REJECT"

- MISSING_OR_INVALID_INFORMATION

- CONGESTION_REJECT

- POLICY_REJECT

- RESOURCES_EXHAUSTED

- INTERNAL_ERROR

- UNSPECIFIED

- type: string

UeSignallingType:

description: >

The UE signalling type(e.g. Registration Request, Mobility and Periodic Registration

Request, Service Request).

anyOf:

- type: string

enum:

- INITIAL_REGISTRATION

- PERIODIC_REGISTRATION

- MOBILITY_REGISTRATION

- SERVICE_REQUEST

- UL_NAS_TRANSPORT

- type: string

StateChangeType:

description: The state change type.

anyOf:

- type: string

enum:

- ACCESS_TYPE_CHANGE_TO_3GPP_ACCESS

- ACCESS_TYPE_CHANGE_TO_NON3GPP_ACCESS

- RM_STATE_CHANGE_TO_RM_DEREGISTERED

- RM_STATE_CHANGE_TO_RM_REGISTERED

- CM_STATE_CHANGE_TO_CM_IDLE

- CM_STATE_CHANGE_TO_CM_CONNECTED

- HANDOVER

- MOBILITY_REGISTRATION_UPDATE

- type: string

* * * End Change * * * *