3GPP TSG RAN Meeting #109					RP-251945

Beijing, China, Sep 15-18, 2025

Agenda Item:	8.2.1.2

Source:	Ericsson

Title:		Technical Performance Requirements (TPR) for the IMT-2030 Satellite Component

Document for:	Approval

# 1	Introduction

At TSG RAN#106, a study was initiated for 6G Scenarios and requirements (RP-243327), with the objective is to investigate a candidate set of items for minimum TPRs for the terrestrial component of IMT-2030, based on the ITU-R Recommendation M.2160 [1], and to share the outcome with ITU-R WP5D as suitable. In parallel, ITU-R WP4B has started its work on the satellite component of IMT-2030. In its #56 meeting in May 2025, WP4B agreed on the timeline for the development of the IMT-2030 satellite component [2]. For the next meeting #57 in October, input on the potential set of TPRs is expected.

In this contribution, we provide our view on the potential set of requirements for satellite IMT-2030.

# 2	Discussion

3GPP submitted NR NTN and IoT NTN as the IMT-2020 satellite component in 2023 [3]. ITU-R accepted the 3GPP submission as the IMT-2020 satellite component in 2025 [4]. The requirements for the IMT-2020 satellite component closely followed the terrestrial component with regards to their structure and evaluation procedure. The values of the TPRs were, however, adapted to satellite conditions. This procedure ensures consistency and coherence between the terrestrial and satellite components and facilitates comparison of their respective performance. In our view, the same example should also be followed for the IMT-2030 satellite component, and its TPRs should be based on those for the terrestrial component. As for IMT-2020, the TPR values will need to be adapted to the satellite conditions, where long delays, large path loss, and limited bandwidth restrict the achievable performance.

The definition of the TPRs for satellite IMT-2030 should be based on those for the terrestrial component of IMT-2030, with the values adapted to satellite conditions, following the example from IMT-2020.

The requirements for the IMT-2020 satellite component can be found in ITU-R Report M.2514 [5]. The requirements for the terrestrial component of IMT-2030 are discussed in RP-251952.

### 2.1	Requirements inherited from IMT-2020

The following TPRs were part of the set of IMT-2020 requirements (both terrestrial and satellite) and are now also considered for terrestrial IMT-2030 with unchanged evaluation methodologies:

- Peak data rate

- User experienced data rate

- Peak spectral efficiency

- Average spectral efficiency

- 5th percentile spectral efficiency

- Area traffic capacity

- CP and UP latency

- Connection density

- Mobility

- Mobility interruption time

- Bandwidth (by inspection only)

We support including all these TPRs as requirements for the IMT-2030 satellite component, even considering that the evaluation methodology for the UL requirement on User experienced data rate might be changed slightly compared to IMT-2020. The values should be adapted to satellite conditions, as for IMT-2020.

TPRs inherited from IMT-2020 with unchanged evaluation methodologies should also become TPRs for satellite IMT-2030.

### 2.2	Reliability

Reliability as part of the terrestrial IMT-2020 TPRs relates to successful transmission of a certain amount of data within a certain time. For the satellite variant, it was agreed to remove the time requirement, due to the long propagation delay in satellite networks. Instead, the proponent has to report the time required to achieve the reliability target. Since the long propagation delay is an inherent property of satellite communications, the same argument applies also for IMT-2030. In our view the reliability TPR for satellite IMT-2030 should follow the example from IMT-2020 and consist of the success probability of a data transmission without a simultaneous requirement on transmission time.

The reliability TPR for satellite IMT-2030 should consist of the success probability of a data transmission without a simultaneous requirement on transmission time, as in the IMT-2020 satellite component.

### 2.3	Energy efficiency

Energy efficiency has been part of the IMT-2020 requirements, both for terrestrial and satellite, where it was to be evaluated by inspection. For IMT-2030, it is proposed that energy efficiency is to be evaluated analytically. While the details have yet to be settled, we believe that the same approach should be followed for the satellite version of this requirement.

Energy efficiency should be part of the satellite IMT-2030 TPRs, and should be evaluated analytically, following the model of the terrestrial requirement. Details are FFS.

### 2.4	New requirements

For terrestrial IMT-2030, a number of new TPRs is also being discussed, in particular the following:

- Resilience

- Positioning

- Sensing-related capabilities

- Applicable AI-related capabilities

- Interoperability

- “Joint requirement”

Out of these, we believe that Sensing-related capabilities and the “Joint requirement” are targeting use cases (sensing and XR, respectively) which are not applicable to the satellite component. In our view, these should therefore not become TPRs for satellite IMT-2030.

Sensing-related capabilities and the “Joint requirement” should not become TPRs for the IMT-2030 satellite component.

In a terrestrial context, the Positioning TPR relates mainly to indoor and dense urban usage scenarios. Again, these scenarios are not relevant for the satellite component. On the other hand, the ability to perform positioning as such could be relevant for satellite scenarios, so a corresponding TPR might be motivated. In our view, the inclusion of a positioning requirement for satellite IMT-2030 should be discussed further.

The inclusion of Positioning as a TPR for the IMT-2030 satellite component needs further discussion.

When it comes to Resilience, Interoperability, and AI-related capabilities, we believe that these relate to basic features and capabilities of the radio interface. Since we envision the same radio interface for the terrestrial and satellite components of IMT-2030, we believe that these TPRs should be evaluated by inspection and would be suitable requirements for the satellite component. The details need further discussion, however.

We support the inclusion of Resilience, Interoperability, and AI-related capabilities as TPRs for satellite IMT-2030, to be evaluated by inspection. The details are FFS.

# 4	Conclusion

In this contribution we provide our view on the potential set of requirements for satellite IMT-2030, we propose the following:

- The definition of the TPRs for satellite IMT-2030 should be based on those for the terrestrial component of IMT-2030, with the values adapted to satellite conditions, following the example from IMT-2020.

TPRs inherited from IMT-2020 with unchanged evaluation methodologies should also become TPRs for satellite IMT-2030.

The reliability TPR for satellite IMT-2030 should consist of the success probability of a data transmission without a simultaneous requirement on transmission time, as in the IMT-2020 satellite component.

Energy efficiency should be part of the satellite IMT-2030 TPRs, and should be evaluated analytically, following the model of the terrestrial requirement. Details are FFS.

Sensing-related capabilities and the “Joint requirement” should not become TPRs for the IMT-2030 satellite component.

The inclusion of Positioning as a TPR for the IMT-2030 satellite component needs further discussion.

We support the inclusion of Resilience, Interoperability, and AI-related capabilities as TPRs for satellite IMT-2030, to be evaluated by inspection. The details are FFS.

# References

- ITU-R Recommendation M.2160-0 (11/2023), “Framework and overall objectives of the future development of IMT for 2030 and beyond”.

- Annex 3 to the Chair’s Report on the fifty-sixth meeting of Working Party 4B (Document 4B/139 Annex 3), “Proposed Work Plan for the Future Development of Satellite IMT-2030”.

- 3GPP TR 37.911, “Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology”.

- ITU-R Report M.2543-0 (11/2024), “Outcome of the evaluation, consensus building and decision of the IMT-2020 satellite process (Steps 4 to 7), including characteristics of IMT-2020 satellite radio interfaces”.

- ITU-R Report M.2514-0 (09/2022), “Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020”.