3GPP TSG RAN meeting #109                                                 RP-252038

Beijing, China, Sep 15-18, 2025

Title: 	Views on Rel-20 NTN topics for 5G-A

Source: 	ZTE Corporation,Sanechips

Agenda item:	9.1.4.6

Document for:	Discussion

Introduction

In the RAN#108 meeting, there were lots of discussions on Rel-20 RAN4 package and the discussion outcomes are endorsed in the way forward on RAN4 R20 package [1]. In this contribution, we would like to share some further views on Rel-20 NTN topics for 5G-A.

Discussion

- 2.1. NTN enhancement

- 2.1.1 ZTE’s view

The high level views on the companies’ proposals on Rel-20 NTN enhancement so far are summarized in the following table.

High level scope for WI on NTN enhancement (the exact scope and WID draft will be discussed and decided in RAN#109): - NTN HPUE - Finish remaining PC1.5 (~29 dBm) work for hand-held devices for both NR NTN and IoT NTN (several studies and discussions already done in R19) - NR NTN with 2TX and/or FFS 1TX - NTN HD-FDD VSAT for Ka/Ku bands - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm). - Once the mobile VSAT with small antennas work is done, - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable - Leverage RedCap over NTN HD-FDD solution to NTN to minimize RAN1/2 impacts. - NTN DL CA - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026 - RAN1/2 impact is expected and will be further discussed in RAN#109 - Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3 - Take one example band with minimum work with PC3 - The rest cases can be considered as part of spectrum work once the requirements on the example band is completed - Target 1TU in Q4’25 and 1.5 TU allocation including RF, RRM and demod from Q1’26

In the last RAN#108 meeting, there were still open issues for NTN HPUE part, here we provide some further detailed objectives as following:

Proposal 1: for NTN HPUE objective, we propose to consider the following objective:

- NTN HPUE for handheld device in FR1 FDD band

    - NR NTN with 2TX (for FDD 2TX with 26dBm per PA)

    - IoT NTN with 1TX with 29dBm per PA

    - The feasibility study NR NTN with 1TX 29dBm PA and its duplexer

In the last RAN#108 meeting, some notes for HD-FDD and DL CA indicate that RAN1/RAN2 impacts is expected.  From our understanding, since NTN Redcap has already been supported for FR1 FDD bands, the mechanism for handling DL-UL collision for NTN Redcap UE could be reused for HD-FDD VSAT design, therefore we don’t expect the additional RAN1/RAN2 spec impact.

For NTN DL CA, in the current specification, from our understanding, there are the specific optimization for NTN CA, in other words, the TN CA framework could be reused for NTN CA without spec impacts or dedicated optimization for it.

Observation 1: for HD-FDD VSAT, it is not expected to have spec impacts in addition to NTN Redcap UE defined in Rel-19.

- 2.1.3 WID objective

- Core part:

- NTN HPUE for handheld device in FR1 FDD band

    - NR NTN with 2TX (for FDD 2TX with 26dBm per PA)

    - IoT NTN with 1TX with 29dBm per PA

    - The feasibility study NR NTN with 1TX 29dBm PA and its duplexer

- NTN HD-FDD VSAT for Ka/Ku bands

  - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm).

  - Once the mobile VSAT with small antennas work is done,

    - Once the mobile VSAT with small antennas work is done,

      - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable

      - reuse RedCap over NTN HD-FDD solution to avoid RAN1/RAN2 impacts

      - RF requirement for small VSAT and HD-FDD operation;

      - The necessary RRM requirement for HD-FDD VSAT

- NTN DL CA

    - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC

      - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands

      - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026

    - RAN1/2 impact is not expected for NTN CA in Rel-20.

- Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

  - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3

    - Take one example band with minimum work with PC3

- Perf part:

- NTN HD-FDD VSAT for Ka/Ku bands

      - The necessary demod requirement for HD-FDD VSAT

      - The necessary RRM test case for HD-FDD VSAT

Conclusions

In this contribution, we shared some further views on Rel-20 RRM enhancement based on RAN#108 moderator’s way forward.

Proposal 1: for NTN HPUE objective, we propose to consider the following objective:

- NTN HPUE for handheld device in FR1 FDD band

    - NR NTN with 2TX (for FDD 2TX with 26dBm per PA)

    - IoT NTN with 1TX with 29dBm per PA

    - The feasibility study NR NTN with 1TX 29dBm PA and its duplexer

Observation 1: for HD-FDD VSAT, it is not expected to have spec impacts in addition to NTN Redcap UE defined in Rel-19.

Proposal 2: for Rel-20 NTN enhancement, we propose to rephrase in the following way:

- Core part:

- NTN HPUE for handheld device in FR1 FDD band

    - NR NTN with 2TX (for FDD 2TX with 26dBm per PA)

    - IoT NTN with 1TX with 29dBm per PA

    - The feasibility study NR NTN with 1TX 29dBm PA and its duplexer

- NTN HD-FDD VSAT for Ka/Ku bands

  - RAN4 will start with HD-FDD mobile VSAT with small antennas (e.g 20x20cm).

  - Once the mobile VSAT with small antennas work is done,

    - Once the mobile VSAT with small antennas work is done,

      - NTN HD-FDD for fixed VSAT and the mobile VSAT with large/normal antenna will be discussed, including how the agreements for the mobile VSAT with small antenna can be leveraged as applicable

      - reuse RedCap over NTN HD-FDD solution to avoid RAN1/RAN2 impacts

      - RF requirement for small VSAT and HD-FDD operation;

      - The necessary RRM requirement for HD-FDD VSAT

- NTN DL CA

    - Intra-SAN CA limited to DL intra-band contiguous and non-contiguous with 2CC

      - Example bands will be further decided within L/S-band and Ku/Ka-band as example bands

      - Depending on the available band combinations, interested companies can propose to extend this work to intra-band with more than 2CC in the check-point in June 2026

    - RAN1/2 impact is not expected for NTN CA in Rel-20.

- Spec clean-up/fixing: Missing Channel BW Common Core and Perf Requirements

  - 30MHz (DL/UL), 40MHz (priority DL), 25MHz with PC3

    - Take one example band with minimum work with PC3

- Perf part:

- NTN HD-FDD VSAT for Ka/Ku bands

      - The necessary demod requirement for HD-FDD VSAT

      - The necessary RRM test case for HD-FDD VSAT

References

[1] RP-251879,Way forward on RAN4 R20 package, RAN4 Chair (Apple), Approved.