3GPP TSG RAN Meeting #109				RP-252061

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.2

Title:		Discussions on 6G KPI

Source:		SK Telecom

Document for:	Discussion

Introduction

The study item on 6G Scenarios and Requirements (FS_6G_RAN_Scen_Req) includes KPI definition as a key step to align 3GPP with ITU-R IMT-2030 Technical Performance Requirements (TPRs).

- ITU-R WP5D has preliminarily agreed on a set of candidate TPRs including throughput, latency, efficiency, positioning, sensing, and AI.

- Korea has submitted contribution 5D/658 proposing updated definitions and target values with pragmatic operator considerations.

- In parallel, 3GPP RAN has begun documenting KPI definitions in TR 38.914 (RP-251880), focusing on evaluation methodology and mapping to IMT-2030.

This contribution reflects SK Telecom’s view, with emphasis on:

- Alignment between ITU-R TPRs and 3GPP KPIs,

- Realism in target setting considering deployment feasibility,

- Expansion of KPI scope to capture 6G characteristics (composite performance, sensing, AI, sustainability).

Operator’s View on KPIs

Throughput-related KPIs

Peak data rate and peak spectral efficiency remain important indicators of technical potential. However, from the operator perspective, these should not be overemphasized as commercial services rarely demand extreme peak values. A conservative uplift of approximately 1.5× over IMT-2020 is considered sufficient. User experienced data rate should target a four-fold improvement in the downlink, while uplink values must be set with caution, recognizing the fundamental constraint of UE transmit power. The 5th percentile user spectral efficiency should also be improved by 3–4× in downlink, with uplink values defined more conservatively.

Capacity-related KPIs

Average spectral efficiency should aim at a three-fold increase compared to IMT-2020, reflecting antenna scaling and higher TRxP density. Area traffic capacity is expected to improve by 3–5×, which is particularly relevant in dense urban immersive communication scenarios. The composite requirement, combining data rate, latency, reliability, and the number of users per TRxP, is strongly supported. This KPI better represents immersive and XR use cases than any single metric.

Latency and Reliability KPIs

For user plane latency, the IMT-2020 levels of 4 ms for immersive communication and 1 ms for HRLLC are sufficient. Control plane latency should be further reduced from 20 ms to 10 ms, achievable with techniques such as RACH-less or two-step access. Reliability and connection density requirements are adequately covered by IMT-2020 targets of 10⁻⁵ success probability and 10⁶ devices per km²; further tightening would not be cost-effective from an operator’s perspective.

Mobility-related KPIs

Mobility classes defined in IMT-2020 should be maintained. Additional classes such as airplane mobility at 1000+ km/h are not a priority for commercial operators and should remain optional. Mobility interruption time should remain at zero, consistent with IMT-2020 requirements, to ensure seamless service continuity.

Spectrum and Bandwidth KPIs

A system bandwidth capability of at least 400 MHz should be required. This is essential to leverage FR3 (7–8 GHz) and sub-THz spectrum, which will be critical for 6G capacity and ISAC service enablement.

Positioning KPIs

For positioning, ambitious targets are desirable, such as 1 m horizontal / 3 m vertical accuracy for indoor, and 3 m / 3 m for outdoor. However, it should be recognized that such accuracies may not be achievable across all scenarios. Operators see this KPI as important for UAV, IoT, and location-based monetization, but deployment feasibility must be considered.

Energy Efficiency and Sustainability KPIs

Energy efficiency should be defined as the relative energy saving gain under unloaded conditions, evaluated analytically. This avoids disputes around traffic model assumptions. Both network and device energy efficiency should be reported, and the KPI should explicitly be positioned as “Energy efficiency for sustainability.”

New KPIs for 6G: Sensing and AI

Sensing capabilities should be captured as new KPIs, including detectability, localization accuracy, velocity accuracy, and resolution. A staged approach is recommended: detectability and localization accuracy as primary metrics, with resolution optional. This aligns with operator interests in Sensing-as-a-Service. For AI-related KPIs, only qualitative definitions are appropriate. Support for distributed learning, inference, and AI-native RAN functionalities should be ensured, but no quantitative targets should be defined, as performance heavily depends on implementation and hardware.

KPI Comparison Table (ITU-R, Korea, 3GPP, and SKT’s view)

| KPI Category | WP5D WD (5D/792 Annex 5.14) [1] | ITU-R WP5D Korea Contribution (5D/658) [2] | 3GPP TP (RP-251880) [3] | SKT ‘s View |
| --- | --- | --- | --- | --- |
| Peak Data Rate | Defined as Rp = W×SEp; DL/UL TBD Gbps | 1.5× | Formula defined, TBD | Conservative uplift, demo metric only |
| Peak Spectral Efficiency | Defined as max bit/s/Hz under ideal; DL/UL TBD | 1.5× with 1024-QAM, 8/4 layers | Defined, values TBD | Support modest gain, aligned with Korea |
| User Exp. Data Rate | Defined via 5% CDF; formula Ruser = W×SEuser; TBD values | 4× DL, UL TBD (no scaling) | Defined as 5% CDF throughput | Support modest gain, aligned with Korea |
| 5th Percentile SE | Defined as 5% CDF normalized throughput; TBD per env. | 3× DL, 2× UL | Values per environment TBD | Support DL uplift, UL constrained |
| Average SE | Defined as eq. (4), per env. TBD | 3× DL/UL | Formula given, TBD | Accept 3× if deployment density realistic |
| Area Traffic Capacity | Defined as eq. (6), DL TBD in Hotspot/Urban | 3–5× | Defined as Carea = ρ·W·SEavg | Support modest gain, aligned with Korea |
| User Plane Latency | Defined as one-way radio latency; values TBD | Same as IMT-2020 | Same, small-packet assumption | Retain IMT-2020 levels |
| Control Plane Latency | Defined as Idle→Active transition; TBD ms | 10 ms | 20 ms baseline, 10 ms option | Support 10 ms |
| Connection Density | Defined as devices/km² with QoS; TBD | Same as IMT-2020 | Same as IMT-2020 | Retain IMT-2020 values |
| Reliability | Defined as success probability; TBD (aligned with IMT-2020) | Same as IMT-2020 | 10⁻⁵ defined | Retain IMT-2020 values |
| Mobility | Classes defined (stationary, pedestrian, vehicular, high-speed vehicular); TBD values | Same as IMT-2020 | Extended incl. airplane | Retain IMT-2020 values |
| Mobility Interruption | Defined as shortest time with no packets; TBD ms | Same as IMT-2020 | TBD definition | Maintain 0 |
| Bandwidth | Requirement: at least TBD MHz | ≥400 MHz | ≥400 MHz | Support ≥400 MHz |
| Positioning | Defined as accuracy @90% CDF; TBD per env. | Indoor 1/3 m, Outdoor 3/3 m | - | Support Korea proposal |
| Energy Efficiency | Relative gain vs full load; unloaded analytical evaluation | TBD | TBD, aligned with power model | Support unloaded case, both network/device gains |
| Composite KPI | Defined as joint metric (rate, latency, reliability, users); Table X TBD | DL/UL 30/10 Mbps, latency 10/30 ms, 10 users/TRxP | Editor’s note only | Support as key 6G KPI |
| Sensing KPIs | Defined as detectability, localization, velocity; resolution TBD | Same, resolution optional | - | Support Korea proposal |
| AI Capabilities | Defined as “Network for AI” and “AI for Network”; qualitative | Qualitative, no numeric target | Editor’s note only | Support qualitative; distributed AI, no numeric |

Conclusion

SK Telecom supports the definition of 6G KPIs in alignment with ITU-R IMT-2030 TPRs, emphasizing realistic deployment feasibility and operator-centric service enablement.
Key proposals are as follows:

- Keep peak throughput and spectral efficiency targets conservative, while prioritizing user-experienced KPIs.

- Introduce a Composite KPI as a central metric for immersive and XR services.

- Adopt Energy Efficiency, Sensing, and AI-related capabilities as new KPIs with pragmatic evaluation methods.

- Ensure ≥400 MHz system bandwidth and realistic positioning accuracy targets aligned with ISAC and UAV use cases.

- The table includes our operator view alongside the ITU-R, Korean, and 3GPP perspectives for each KPI.

References

- ITU-R WP5D, Document 5D/792-E, Annex 5.14, “Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ] – Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, 11 July 2025

- Republic of Korea, Contribution 5D/658-E, “Proposal for Update to a Working Document Towards a Preliminary Draft New Report ITU-R M.[IMT-2030.TECH PERF REQ],” ITU-R WP5D #49, 12 June 2025.

- CMCC, RP-251880, “Text proposal for TR 38.914 on KPIs,” 3GPP TSG RAN #108, Prague, Czech Republic, June 9–13, 2025.

- RAN Chair (Samsung), RP-251890, “Agenda for RAN #109,” Beijing, China, September 15–18, 2025.RP-250819, Moderator's summary for IMT-2030 TPRs, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 12-14, 2025.