3GPP TSG RAN Meeting #109			RP-252062

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.3

Title:	Views on Requirements for Migration

Source:	SK Telecom

Document for:	Discussion

1. Introduction

At RAN#107, the revised SID on 6G requirements has been approved with updating the objectives as follows [1]:

This study item aims to - investigate a candidate set of items for minimum TPRs based on the Recommendation ITU-R M.2160, and, where applicable, the associated target values and key assumptions for the identified minimum TPRs. - The outcome is expected to be shared by LS with ITU-R WP5D, as suitable, and used as a baseline for the subsequent study 6G in RAN - Identify typical and practical deployment scenarios defined by attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, and other relevant factors. - Develop 3GPP requirements for 6G Radio for improvement of existing services and for new services. - Determine the applicability of legacy services to 6G Radio, and define radio requirements for these, as appropriate. - Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least: - Ensure appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting - Energy efficiency and energy saving: both for network and device - Enhanced spectral efficiency - Enhanced overall coverage, focus on cell-edge performance and UL coverage - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band - Target scalable and forward compatible design for diverse device types - Improved spectrum utilization and operations taking into account diverse spectrum allocations - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs - Aim at a harmonized 6G Radio design for TN and NTN, including their integration - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. - Define a time plan and steer work as appropriate for the RAN WGs during the 6G WG SI to deliver high-level decisions at least on the following areas: - Fundamental 6G radio design aspects: waveform, numerology, channel coding, etc… - Overall high-level aspects of 5G to 6G migration - RAN architecture and interfaces, including RAN-Core interface - Coordination of 6G AI/ML framework Note: The detailed time plan for the high-level decisions above will be decided at RAN#108 at the approval of the 6G RAN WG SI and is to be aligned with the RAN SI progress.

In this contribution, we aim at providing our view on requirements regarding 6G migration.

2. Discussion

Recollecting the migration to 5G, it has posed not only technical challenges for operators, but also architectural choices that critically shape the long-term scalability and innovation of mobile networks. According to 3GPP discussion, seven main deployment options were defined to support various migration paths between 4G LTE and 5G NR. These range from non-standalone (NSA) configurations like Option 3 (LTE anchor + 5G NR) or Option 4 (5G NR anchor + LTE) to standalone (SA) modes like Option 2 (pure 5G NR with 5G Core).

In fact, 5G was the first try to standardize the deployment option that utilizes the legacy network for migration, and actually option 3x allowed fast 5G deployment with EPC/eNB. However, it entailed huge standardization efforts by considering all possible candidates for deployment options and the introduction of ‘real’ 5G has been indispensably delayed due to the commercialization of ‘early drop’, which resulted in market fragmentation and a slow uptake of 5G standalone deployments.

Accordingly, operator faced several difficulties such as (1) architectural complexity such as selection among various options, impact due to UE capability, and inefficiency in economic scale, (2) challenges in migration such as operational complexity for transition from NSA to SA, (3) slow adoption of key capabilities for new generation. From these, it becomes obvious how important the design/standardization related to migration for new generation would be.

From lessons learned from 5G, although it sems common understanding, simple architecture option should be strived for by avoiding too many migration options. And, several aspects for migration should be considered: QoS/performance, service scalability, network/UE impact, deployment situation (e.g., NSA or nation-wide SA), etc.

- Simple architecture option should be pursued by avoiding too many migration options.

- Several aspects should be considered for migration: QoS/performance, service scalability, network/UE impact, deployment situation, etc

5G-6G multi-RAT spectrum sharing (MRSS) has been considered as one of promising migration options, and it has been already included as a scope of 6G study. For assessing the MRSS option, several aspects need to be taken into account, e,g., throughput, latency, inter-RAT interruption, etc. For 4G-5G DSS (dynamic spectrum sharing), it has been mainly focused on FDD deployment. Given the different spectrum situation per operator/region, it would be undesirable to consider MRSS only in FDD just like DSS case, and the support of MRSS in TDD as well as FDD also needs to be taken into account.

One essential aspect would be 6G PHY layer design to enable MRSS. Considering MRSS allows resource pooling between 5G and 6G, at least frame structure, numerology, and waveform needs to be inter-operable. For instance, in case different subcarrier spacing between 5G and 6G are considered for MRSS operation, scheduling coordination would be much more complicated which might result in inefficiency for spectrum sharing. Moreover, as discussed during RAN#108, from MNO perspective, minimizing hardware refresh is preferred and software upgrade driven 6G deployment with MRSS needs to be pursued. In this context, how to enable 6G PHY layer design inter-operable with 5G needs to be studied.

Another consideration point would be how to advance data aggregation gain for MRSS. As known, there was quite considerable overhead due to LTE CRS for 4G-5G DSS case, which induced overall throughput loss compared to the case without spectrum sharing. 5G design tried to avoid always-on signal, and thus 5G-6G MRSS would provide performance with relatively lower loss. But, still MRSS only can offer spectrum utilization efficiency on a single carrier. To realize the benefit of data aggregation for MRSS, 6G-6G carrier aggregation needs to be ready from specification point of view.

- For 5G-6G MRSS, supporting TDD also needs to be considered.

- For 5G-6G MRSS, how to enable 6G PHY layer design inter-operable with 5G needs to be studied.

- 6G-6G carrier aggregation needs to be supported from 6G day-1 specification.

It looks no common understanding in 3GPP on whether to study further migration option other than MRSS. Although MRSS is considered by majority of companies, 5G-6G aggregation may also need to be carefully checked. Basically, such 5G-6G aggregation would allow throughput gain over standalone option via data aggregation. Also, less interruption time might be expected with support of inter-RAT mobility.

One option could be core-level 5G-6G aggregation. It would enable 6G service via 6GC without limitation from legacy core network, and less complexity is expected without interaction between 5G and 6G RAN compared with RAN-level aggregation. However, it would bring about less throughput gain than RAN-level aggregation as data aggregation needs to be conducted without knowing situation for traffic load or channel state of each RAN side. Furthermore, there was no previous commercialization experience on this option, which might impose uncertainty to operators.

The other option would be RAN-level 5G-6G aggregation, which would obviously allow throughput gain over standalone option. However, from past experience, we could observe that deploying RAN-level aggregation such as 4G-5G dual connectivity would induce high complexity for network from both implementation and maintenance perspectives. Meanwhile, it should be noted that there could be two different ways for RAN-level aggregation depending on which RAT is a primary cell group, i.e., 5G option 3-like (which would be 5G-6G dual connectivity) and 5G option 4-like (which would be 6G-5G dual connectivity). The former would enable rapid 6G deployment with using legacy 5G network (and without rollout of 6G core) at the expense of being unable to support 6G-specific feature. The latter would benefit from 6G-specific features as well as overall throughput gain via introducing 6G core.

Regarding all the above options, it would be good to identify options to be further studied as early as possible so that RAN WGs can study those options sufficiently.

- It is essential to identify migration option(s) to be further studied as early as possible to provide sufficient time to be studied in RAN WGs.

Regarding mobility/interworking between generations, mobility between 6G and 5G would be definitely needed, and on top of that, whether to support mobility between 6G and 4G needs to be further studied considering the discrepancy of coverage between 4G and 5G in the commercial network. At the early stage of 6G commercialization, we expect still most 4G network would be maintained, and 6G coverage could be smaller than 4G or 5G. In case that UE moves from 6G service area to out-of-coverage, this would cause handover from 6G to 5G or even to 4G. If there is no connected mode mobility mechanism from 6G to 4G, the overall service quality would be degraded. In this context, handover from 6G TN to 4G TN needs to be studied.

- Whether to support mobility between 6G and 4G needs to be further studied.

3. Conclusion

In this contribution, the following proposals are provided.

- Simple architecture option should be pursued by avoiding too many migration options.

- Several aspects should be considered for migration: QoS/performance, service scalability, network/UE impact, deployment situation, etc

- For 5G-6G MRSS, supporting TDD also needs to be considered.

- For 5G-6G MRSS, how to enable 6G PHY layer design inter-operable with 5G needs to be studied.

- 6G-6G carrier aggregation needs to be supported from 6G day-1 specification.

- It is essential to identify migration option(s) to be further studied as early as possible to provide sufficient time to be studied in RAN WGs.

- Whether to support mobility between 6G and 4G needs to be further studied.

References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, RAN#107.