3GPP TSG RAN Meeting #109					    	                    RP-252131

Beijing, China, Sep 15-18, 2025

Agenda item:	8.2.1.4

Source:	Huawei, HiSilicon

Title:	Consideration on supporting immersive communication services for 6G

Document for:	Discussion

Typical use cases

Immersive services are supposed to be the mainstream for future wireless communication networks. Recently, advanced achievements in AI have further broadened the scope of immersive applications, from 5G-era services like XR to innovative mobile AI services. These immersive experiences are expected to be seamlessly integrated into our daily life, from traveling and entertainment to working, education, and eldercare.

The typical use cases of immersive communication services may include:

- Higher resolution XR cloud gaming: Immersive gaming with 8K resolution display has  been defined in Section 9.1 of TR 22.870 [1]. However, for XR display devices with angular resolution below around 60 PPD (Pixels Per Degree), a so-called “Screen Door Effect” is expected, whereas users can perceive individual display pixels (i.e. pixel boundaries), creating a mesh-like visual obstruction [2]. Therefore, higher solution display such as 12K/16K is preferred for immersive user experience.

- Intelligent video conferencing on AI phones: As defined in Section 8.3 of TR 22.870 [1], the intelligent video conferencing on AI phone offers real-time functions such as facial enhancement and meeting summary, using gesture and speech analysis. Users no longer need to divide their attention by taking notes or repeatedly reviewing meeting recordings to find important discussion points. Instead, these AI solutions can automatically transcribe speech from active speakers, extract key information from discussion, and compile them into well-organized text files.

- Contextual understanding on AI wearable: As shown in Section 8.3 of TR 22.870 [1], contextual understanding on AI wearable may change the way we interact with our world. Until to July 2025, lots of companies, such as Google, Xiaomi, and Ali, have released their newest AI glasses, which can provide advanced awareness of the real-time environment [3]. These AI glasses can realize a variety of real-time tasks, such as instant translation, navigation guidance, and blind assistance, make your life more convenient..

- Healthcare for aging population on embodied AI robots:  Healthcare for aging population has been defined in Section 9.9 of TR 22.870 [1]. Powered by large language models, embodied AI robots can sense elderly people needs and emotions via cameras, and provide health guidance, emotional support, and entertainment. Recent achievement in physical AI will further help these robots work well in open environments. For example, Meta’s research shows that physical AI models can learn physical rules from videos and respond in real time to events like cup falling [4]. This prompts the service of embodied AI robots from indoor to outdoor in the future.

Different level of composite requirement

The aforementioned typical use cases for immersive communication brings new requirements for future network. The service flow of these use cases is firstly discussed in the following Section 2.1 and then the composite requirement with different level of data rate and latency is discussed in Section 2.2.

2.1 Service flow

For the aforementioned mobile AI cases, the service workflow is illustrated in Figure 2.1.1, which consists of the following steps.

Figure 2.1.1 Service flow of mobile AI services

- Step 1: Mobile AI device collects necessary data unit for AI inference.

- Step 2: Mobile AI device uploads the data unit to cloud server through 6G network.

- Step 3: Cloud server performs AI inference based on received data unit.

- Step 4: Cloud server sends the inference results to the mobile AI device through 6G network.

For the high resolution XR cloud gaming, the service flow and traffic model can be referred to TR 38.838 [5]. This is similar to Figure 2.1.1, where the upload data includes pose/control information collected by XR devices, while the download data includes the high-resolution rendering video from the cloud server.

2.2 Composite Requirement for different use cases

2.2.1 DL/UL Service Data Rate Requirement

Based on the aforementioned service flow, it can be expected that the XR and mobile AI services are downlink-heavy and uplink heavy, respectively.

For XR services, the downlink service data rate can be from 100Mbps to 500Mbps. Taking cloud gaming for example, the refresh rate should be 90 fps, or even 120 fps to provide smoother user experience as defined in TR 22.870 [1]. In the meanwhile, the resolution should be 8K or even 12K/16K to avoid the “Screen Door Effect”. The corresponding downlink service data rate can be computed as shown in Table 2.1.1.

For mobile AI services, the uplink service data rates could scale from 20Mbps to 100Mbps, depending on different AI workloads. Taking the embodied AI robots as an example, there are several options for robots: 1) 20m gesture recognition, where 6*1080p cameras are equipped in the intelligent terminal; 2) 30m 180° micro expression capture capability, where the front view cameras (dual camera similar to “human eyes”) are replaced with 4K resolution and 30 fps refresh rate; 3) 30m 360° micro expression capture capability, where two additional cameras are upgraded to 4K.  The corresponding uplink service data rate can be computed as shown in Table 2.1.2.

2.2.2 DL/UL Latency Requirement

For XR services, TR 38.838 has already preserved a low 50-70ms motion-to-render-to-photon (M2R2P) latency [5], which is often treated as the end-to-end (E2E) latency. Deducting the processing time at the XR devices, the round trip time at transport network, and the rendering time at cloud server, a minimum downlink RAN latency of 10ms is considered for DL cloud gaming frame with the packet success probability of 99%, as described in TR 38.838 [5].

For mobile AI services, a typical E2E latency of 200ms for human involved services should be supported to maintain comfortable and smooth human interaction [6]. This value can be decomposed as follows: 1) 160ms cloud inference latency according to the latest Gemini 2.0 capability [7]; 2) 10ms UE processing latency; 3) 10ms transport network latency; 4) 20ms uplink and downlink RAN latency. Considering that the uplink traffic is heavier for mobile AI, a minimum RAN latency should be 15ms with the packet success probability of 99%.

2.2.3 Summary of different level of composite requirement

In conclusion, different level of composite requirement for XR and mobile AI services is summarized in Table 2.1.1 and Table 2.1.2, respectively. Note that the number of satisfied users supported for these services also matter, as the network needs to support multiple users at the same time to meet the composite requirements, the specific values need to be further investigated.

Table 2.1.1: Downlink composite requirements for XR services

| Use case | Refresh rate | Service data rate[NOTE 1] | DL RAN latency | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- | --- |
| 8K cloud gaming | 90fps | 100Mbps | 10ms | 99% | TBD |
| 16K cloud gaming | 120fps | 500Mbps | 10ms | 99% | TBD |
| NOTE 1: Data Rate = Resolution × Bit Per Pixel × Refresh Rate / Compression Ratio, where compression ratio of 400 and 12 bits per pixel (YUV4:1:1 compression encoding) are considered [8]. |  |  |  |  |  |

Table 2.1.2: Uplink composite requirements for mobile AI services

| Use case | Refresh rate | Service data rate[NOTE 1] | UL RAN latency | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- | --- |
| 20m gesture recognition | 15fps@1080p | ~ 20 Mbps | 15ms | 99% | TBD |
| 30m 180° Micro Expression Capture Capability | 15fps@1080p，30fps@4K | ~ 60 Mbps | 15ms | 99% | TBD |
| 30m 360° Micro Expression Capture Capability | 15fps@1080p，30fps@4K | ~100 Mbps | 15ms | 99% | TBD |
| NOTE 1: Data Rate = Video resolution * (Bits per colour*3) * Refresh rate / Compress ratio. The data rate is estimated assuming: compression ratio of 240 and 8 bits per colour. |  |  |  |  |  |

Proposal 1:  The 6G RAN design shall support different levels of the composite requirement for different use cases as follows:

| Level of composite requirement | Service data rate (Mbps) | RAN Latency (ms) | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- |
| UL |  |  |  |  |
| Level 1 | 20 | 15 | 99% | TBD |
| Level 2 | 60 | 15 | 99% | TBD |
| Level 3 | 100 | 15 | 99% | TBD |
| DL |  |  |  |  |
| Level 1 | 100 | 10 | 99% | TBD |
| Level 2 | 500 | 10 | 99% | TBD |

Requirement for consistent user experience

Immersive services impose stringent network performance requirements, including high data rate, low latency, and high reliability simultaneously. Such user experience should be consistent no matter whether the user is at cell center or cell edge. It is important for satisfying user experience, e.g. robust control of embodied AI robot, and smooth reaction of AI phones/wearables when the devices are moving round the cell.

However, due to multi-cell interference, satisfying user experience becomes more challenging at cell-edge. At cell edge, users are closer to neighbouring cells and therefore more likely to suffer from strong interference from other cells, which may cause sudden degradation of transmission quality. In 5G, multiple TRP has been introduced, which is a good direction to improve cell edge performance. In 6G, multiple TRP direction shall continue to be considered to satisfy new services requirements and provide consistent user experience, by better collaboration to reduce multi-cell interference

Proposal 2: 6G RAN design shall support consistent user experience for new services, e.g., immersive communication, by studying means to improve cell edge performance like multiple TRP and interference management.

For immersive interactive services as introduced above, a smooth user experience also relies on whether the end-to-end (E2E) latency is met. The latency would vary even for the same application. Take embodied AI robots as an example. When the AI robot executes actions such as obstacle avoidance, a short E2E latency is required to guarantee the safety, especially in the moderate or high mobility scenario. When the AI robot executes the actions such as route planning, the E2E latency can be longer, since the complex and precise inference is required. Therefore, the latency requirement can be changed from time to time and how to satisfy such changeable E2E latency needs to be addressed to provide consistent user experience.

As shown in Figure 2.1.1, the 3GPP E2E latency mainly includes air interface (Uu) latency and transport layer (N3) latency. As the E2E latency may change, the delay budget for air interface and transport layer may need adjustment as well. If this is not adjusted timely, the latency over air interface has to always set to satisfy the minimized latency requirement, which could be too stringent. A better way is to jointly negotiate the delay budget between different interfaces, which can give more flexibility for the network to schedule resources in an efficient way as shown in Figure 3.1.1.

Figure 3.1.1. Delay budget collaboration among different layers.

The delay budget collaboration needs accurate and timely delay measurement for each interface, and subsequently RAN and Core network can have coordination on how much delay budget can be allocated for each interface. In 5G L2 delay over air interface is measured per DRB as an average value, which cannot differentiate packets with different QoS requirements. Therefore in 6G, more timely latency measurement and QoS coordination between different interfaces shall be supported, to adapt to the varying latency requirement from applications.

Proposal 3: 6G RAN design shall support consistent user experience for new services, e.g. immersive communication, by studying means to provide real-time E2E latency measurement for delay budget adjustment among interfaces.

Summary

According to the above discussion, it is proposed to adopt proposal 1 into TR 38.914 “key performance indicators” section, and adopt proposal 2 and 3 into TR 38.914 “Requirements of New and Existing Services” section.

Proposal 1:  The 6G RAN design shall support different levels of the composite requirement for different use cases as follows:

| Level of composite requirement | Service data rate (Mbps) | RAN Latency (ms) | Packet success probability | Number of users ([/km2 or /TRxP]) |
| --- | --- | --- | --- | --- |
| UL |  |  |  |  |
| Level 1 | 20 | 15 | 99% | TBD |
| Level 2 | 60 | 15 | 99% | TBD |
| Level 3 | 100 | 15 | 99% | TBD |
| DL |  |  |  |  |
| Level 1 | 100 | 10 | 99% | TBD |
| Level 2 | 500 | 10 | 99% | TBD |

Proposal 2: 6G RAN design shall support consistent user experience for new services, e.g. immersive communication, by studying means to improve cell edge performance like multiple TRP and interference management.

Proposal 3: 6G RAN design shall support consistent user experience for new services, e.g. immersive communication, by studying means to provide real-time E2E latency measurement for delay budget adjustment among interfaces.

References

- 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements.

- https://www.displaymodule.com/blogs/knowledge/exploring-the-screen-door-effect-in-virtual-reality-phenomenon-impact-and-solutions.

- Baig M S A, Gillani S A, Shah S M, et al. AI-based Wearable Vision Assistance System for the Visually Impaired: Integrating Real-Time Object Recognition and Contextual Understanding Using Large Vision-Language Models https://arxiv.org/abs/2412.20059, Dec, 2024.

- M. Assran, A. Bardes, D. Fan, et al.. “V-JEPA 2: Self-supervised video models enable understanding, prediction and planning,” arXiv preprint arXiv:2506.09985, 2025.

- 3GPP TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

- https://humanbenchmark.com/tests/reactiontime.

- G. R. Team, S. Abeyruwan, J. Ainslie, et al., “Gemini robotics: Bringing AI into the physical world,” arXiv preprint arXiv:2503.20020, 2025.

- https://std.samr.gov.cn/hb/search/stdHBDetailed?id=2001448F0D0D32F5E06397BE0A0AAD9C.