3GPP TSG RAN #109	RP-252170

Beijing, China, 15th – 18th September, 2025

Agenda item:		8.2.1.2

Source:	MediaTek Inc.

Title:	Energy efficiency requirements

Document for:					Discussion and Decision

# Introduction

This contribution discusses Energy Efficiency for 6G and makes corresponding proposals.

# Discussion

## Energy efficiency requirements framework considerations

The following objectives are captured within the RAN plenary SID [1] in relation to Energy Efficiency:

- Energy efficiency and energy saving: both for network and device

Energy Efficiency metrics

Within TSG RAN plenary there have been inputs on how to define the Energy Efficiency metric from the perspective of input to ITU-R on the corresponding IMT-2030 requirement. However, formal discussion has not taken place yet due to remaining discussions on conventional TPR values. Therefore, there is no text agreed yet to TR 38.914 [2].

At RAN1#122, the following was agreed:

Agreement

- Study metric(s) for UE energy efficiency.

- Study metric(s) for BS energy efficiency.

At RAN#109, 3GPP should aim to provide feedback to ITU-R on Energy Efficiency metrics and whether it recommends any target values for those metrics. Such a recommendation towards ITU-R does NOT need to be identical to Energy Efficiency metrics defined for internal 3GPP requirements.

For internal 3GPP RAN requirements/targets for Energy Efficiency, there is more time to develop a solid framework and we think the overall requirements setting should be the responsibility of RAN plenary, with technical support (given the quite complex nature of this area) needed from ongoing studies in RAN1.

Additionally, energy efficiency must be balanced with performance considerations such as service coverage and quality (e.g., maintaining average data rates while meeting service latency requirements).

Regarding the metrics, energy efficiency is typically measured as "bits per energy unit" and can be equivalently expressed as the ratio of average data rate (bits/sec) to average power consumption (energy unit/sec). However, when the average data rate remains constant relative to the baseline or under empty cell load, energy efficiency can be simplified to focus on power/energy consumption. In this case, power/energy saving gain compared to a baseline becomes a practical energy efficiency metric for both BS and UE.

Proposal 1: For 3GPP internal Energy Efficiency (EE) requirements, allow for a different metric than the metric recommended to ITU-R for IMT-2030.

Proposal 2: Within the EE requirements framework, ensure that improved energy savings does not cause noticeable degradation to user experience.

Proposal 3: For the 3GPP internal EE requirements framework, adopt 6G power/energy saving gain vs a baseline with same loading as the primary energy efficiency metric for BS and UE evaluations, and where the average data rate served by 6G and baseline are similar/equal.

Reference baseline for comparison

At RAN1#122, the following was agreed:

Agreement

- Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s)

- Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

Many Network Energy Saving (NES) features in 5G NR remain undeployed due to legacy UE capability limitations. As we move toward higher numbers of antenna ports (e.g., 128 or 256 for FR3), the complexity of NES CSI enhancements becomes overwhelming for UE implementation. For meaningful 6G energy efficiency evaluations, the baseline should incorporate UE/network power/energy saving features with proven deployments and verified UE support. NR Release 17 features offer an appropriate starting point for this baseline.

Proposal 4: For the 3GPP internal EE requirements framework, include NR features with real-world deployments as the baseline for 6G energy efficiency comparison. NR Release 17 UE/network power/energy saving features can serve as the initial reference.

Power model assumptions

At RAN1#122, the following was agreed:

Agreement

Study how to reuse and update reference configurations in TR 38.864 for 6G BS.

Agreement

Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS.

Agreement

Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following:

- TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations

- TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models

RAN plenary will need input on the power model before defining any clear requirements. We recommend to give RAN1 some time to progress on the power models first.

Proposal 5: Give time for RAN1 to provide feedback on the BS and UE power consumption models to support practical requirements, taking into account considerations on practical deployment and realistic device implementation.

## Requirements framework definitions

Sustainability TPR for IMT-2030 (also provided in [3]):

The following is proposed for the definition of the requirements framework for the IMT-2020 Sustainability TPR:

Network EE: Quantify the relative improvement in energy consumption for a given system loading (unloaded, partially loaded) vs the energy consumption in fully loaded scenario.

Device EE: Quantify the relative improvement in energy consumption for a given traffic loading (zero load, partial load) vs energy consumption in fully active scenario.

Proponents to report the power model used for the evaluation, as well as impacts on communication performance associated with the evaluated energy saving operation.

No target value for ITU-R, just report what is achieved.

Proposal 6: Consider the suggested ITU-R requirements framework for Sustainability in this document as the recommendation towards ITU-R.

3GPP internal EE requirements framework:
We provide the following as a potential starting point for the definition of the “3GPP-internal” Energy Efficiency requirements framework:

Energy Efficiency is the improvement is the ratio of reduction in energy consumption for Network and Device compared to a baseline with same loading and same [average data rate]. The following formula shall be used to determine the energy efficiency improvement for a given network/device traffic loading:

100 * ( EnergyConsumption_Baseline - EnergyConsumption_6G ) / EnergyConsumption_Baseline

where EnergyConsumption_Baseline is

-		for BS, a Rel-17 network with NES features

-	for UE, a Rel-17 UE with Power Saving features

The recommended power model assumptions are to TBD.

For energy efficiency improvement vs. EnergyConsumption_Baseline, the following targets apply:

-		Partial load – BS: [TBD %]; UE: [TBD %]

-	Zero load – BS: [TBD %]; UE: [TBD %]

Realistic traffic models shall be used for the evaluation and are TBD.

Improved energy efficiency shall not cause noticeable degradation to end user experience. Further metrics to evaluate that impact are TBD.

TBD aspects will be finalized following guidance from RAN1.

Proposal 7: Consider the 3GPP internal requirements framework for EE in this document as a starting point for further elaboration.

# 3	Proposal

Proposal 1: For 3GPP internal Energy Efficiency (EE) requirements, allow for a different metric than the metric recommended to ITU-R for IMT-2030.

Proposal 2: Within the EE requirements framework, ensure that improved energy savings does not cause noticeable degradation to user experience.

Proposal 3: For the 3GPP internal EE requirements framework, adopt 6G power/energy saving gain vs a baseline with same loading as the primary energy efficiency metric for BS and UE evaluations, and where the average data rate served by 6G and baseline are similar/equal.

Proposal 4: For the 3GPP internal EE requirements framework, include NR features with real-world deployments as the baseline for 6G energy efficiency comparison. NR Release 17 UE/network power/energy saving features can serve as the initial reference.

Proposal 5: Give time for RAN1 to provide feedback on the BS and UE power consumption models to support final requirements, taking into account considerations on practical deployment and realistic device implementation.

Proposal 6: Consider the suggested ITU-R requirements framework for Sustainability in this document as the recommendation towards ITU-R.

Proposal 7: Consider the 3GPP internal requirements framework for EE in this document as a starting point for further elaboration.

# 4	References

[1]	RP-250810, “Revised SID: Study on 6G Scenarios and requirements”; CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom.

[2]	3GPP TR 38.914 v0.1.0 “Study on 6G Scenarios and Requirements”.

[3]	RP-252172, IMT2030 TPRs and revisions to TR 38.914; MediaTek