3GPP TSG RAN    Meeting #109	RP-252180

Beijing, China, September 15-18, 2025

Agenda item:	8.2.1.1

Source:	Nokia

Title:	Antenna Configurations for 6G Deployment Scenarios

Document for:	Approval

# 1	Introduction

This document discusses the antenna configurations (number of base station and UE antenna elements) for various 6G deployment scenarios listed in TR 38.914 v0.0.1, which is the main output of the SI “Study on 6G Scenarios and requirements”, agreed in [1].

# 2	Discussion

TR 38.914 v0.0.1. lists currently 12 deployment scenarios for 6G radio, two of which are in square brackets. For each deployment scenario, a set of attributes is/will be provided, such as carrier frequency, aggregated system bandwidth, BS and UE antenna elements, etc. However, it is apparent that the set of attributes is not as such extensive enough to perform necessary evaluations of 6G radio; instead RAN1 and other RAN WGs will need to agree upon further details that are necessary to facilitate comparisons of different technology proposals. It is therefore proposed that the TR shall focus on only high-level aspects of deployment scenarios and leaves detailed assumptions to respective RAN Working Groups to decide.

Proposal #1: TR 38.914 shall consider only high-level assumptions related to deployment scenarios. Details of the attributes are discussed in RAN WGs.

Considering the attributes for deployment scenarios, TR 38.914 v0.0.1. already lists most of the key parameters and their corresponding values and assumptions. The main open points are the assumptions for the number of BS and UE antenna elements, which are indicated to be “TBD”. In our view the TR 38.914 should follow the same approach as the corresponding NR TR 38.913, which simply indicates up to how many antenna elements are assumed in evaluations, while leaving the exact antenna modelling to RAN1 and other RAN WGs.

As for the number of antenna elements, the LS to ITU-R in RP-251825 [2] already indicated that 1024 base station and 4/6/8 UE antenna elements should be assumed when setting the requirements for spectral efficiency. The same assumption should naturally be included into TR 38.914 as well. It may be further discussed if and how the number of antenna elements varies in different deployment scenarios (e.g. indoor vs. outdoor environments), with different carrier frequencies, and different UE types.

Furthermore, NR TR 38.913 has the following not associated with the definition of various deployment scenarios: “The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.”. We see that a similar not should be included into TR 38.914 as well, to account for necessary adjustments to the assumptions in RAN WGs, and to also consider the requirements set later in ITU-R WP 5D.

Proposal #2: The assumption for the number of base station antenna elements in Sections 6.1.X of TR 38.914 (Deployment scenarios) is up to 1024. The number may be lower for certain deployment scenarios and carrier frequencies.

Proposal #3: The assumption for the number of UE antenna elements in Sections 6.1.X of TR 38.914 (Deployment scenarios) is up to 8. The number may be lower for certain deployment scenarios, carrier frequencies, and UE types.

Proposal #4: Add the following note in sections 6.1.X under the tables describing the attributes of deployment scenarios:  “NOTE:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations”.

# 3	Conclusion

In this contribution we discuss aspects related to evaluation assumption for antenna configurations captured in TR 38.914. We make the following proposals

Proposal #1: TR 38.914 shall consider only high-level assumptions related to deployment scenarios. Details of the attributes are discussed in RAN WGs.

Proposal #2: The assumption for the number of base station antenna elements in Sections 6.1.X of TR 38.914 (Deployment scenarios) is up to 1024. The number may be lower for certain deployment scenarios and carrier frequencies.

Proposal #3: The assumption for the number of UE antenna elements in Sections 6.1.X of TR 38.914 (Deployment scenarios) is up to 8. The number may be lower for certain deployment scenarios, carrier frequencies, and UE types.

Proposal #4: Add the following note in sections 6.1.X under the tables describing the attributes of deployment scenarios:  NOTE:	The maximum number of antenna elements is a working assumption. 3GPP needs to strive to meet the target with typical antenna configurations.

# References

[1] RP-251395 Study on 6G Scenarios and requirements, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom

[2] RP-251814 Draft LS on IMT-2030 Technical Performance Requirements