3GPP TSG-RAN Meeting #109	RP-252203

Beijing, China, 15-18 September, 2025

Agenda item:		8.2.1.4

Source:	Xiaomi, Panasonic

Title:	Requirements of Integrated Sensing And Communication (ISAC)

Document for:		Discussion and Decision

Introduction

In RAN#106 meeting, RAN plenary study item on “Study on 6G scenarios and requirements” [1] was approved. In RAN#108 meeting, RAN WG study item on “Study on 6G Radio” [2] was approved.

In this contribution, our views on 6G Requirements for Integrated Sensing And Communication (ISAC) study are provided.

Discussion

Integrated Sensing and Communication (ISAC) is envisioned to become a key application for 6G. It extends and complements the communication oriented 3GPP system with new sensing capabilities for broader business opportunities with new use cases. Additionally, it can also be used to enhance the achievable air-interface performance for communication.

In SA1 TR 22.837 [3] and TS 22.137 [4] 32 use cases are proposed for sensing, which are categorized into three groups, i.e., object detection and tracking, environment monitoring and motion monitoring (see Annex A). They cover a wide field of new applications and services. Additionally, 20 advanced use cases are further identified in the current SA1 TR 22.870 [5], e.g., environment reconstruction, etc.

The initial high-level requirements for sensing performance in [4] are outlined and expressed as “Accuracy of positioning estimate”, “Accuracy of velocity estimate”, “Confidence level”, “Sensing Resolution”, “Missed detection probability”, “False alarm probability”, “Max sensing service latency” and “Refreshing rate.” Depending on the given use case the requirements vary largely and it may be hard to fulfil all of them in the first release of 6G studies. Therefore, before defining details on requirements, RAN should decide on the most attractive and feasible sensing use cases for the start of 6G.

Proposal 1: To facilitate requirement definition for sensing as a new service in 6G, RAN selects a subset of use cases from TR 22.837 with focus on the most feasible and promising applications for the near future.

The selection of use cases for requirement definition should be based on two criteria:

- Near future commercial needs

- Already existing work in RAN that can be re-used

For the second point, the RAN1 study in Rel-19 has already developed the ISAC channel model for object detection/tracking [6]. Therein, the characteristics of the following sensing target types have been defined: UAV, automotive, human, AGV, and objects creating hazard on road/railways. For the use case selection, it is therefore more practical to focus on object detection/tracking.

Proposal 2: It is preferred to focus on use cases for object detection/tracking for ISAC in 6G studies.

Regarding the commercial needs, we see the following services to be attractive for the near future:

Low-altitude economy: This is an emerging business area driven by UAVs, and their detection/tracking are an essential part of the service. Different UAV target types should be supported. The sensing transmitter/receiver can be TRPs and/or UEs. We assume that the UAV itself is not a sensing node, and just treated as a target.

Human intruder detection: This is another promising category, offering safety while keeping privacy (e.g. because no cameras are needed). Also here, the sensing transmitter/receiver can be TRPs and/or UEs. We assume that the human itself is just treated as a target.

Automotive: For better support of automatic driving, early accident indication and intruder detection on the road, sensing should be supported. The sensing transmitter/receiver can be TRPs and/or UEs. In details, the UE can be vehicle UE, pedestrian UE, and RSU type UE.

Proposal 3: To facilitate requirement definition for sensing as a new service in 6G, RAN prioritizes the following use cases

- UAV related use cases

- Human related use cases

- Automotive related use cases

In 5GA, only the performance of gNB monostatic sensing is evaluated. For 6G, this scope should be extended largely. All sensing modes can be considered. Especially involving the UE in the sensing procedure (e.g. TRP-UE bistatic sensing, UE monostatic sensing, UE bistatic sensing) can increase the quality of the sensing service. Further, cooperative sensing and/or multi-static approaches can be considered to further enhance the achievable performance and efficiency.

UEs can play an important role to enable better sensing performance in various use cases. In the following some exemplary benefits are outlined for the above selected use case:

For UAV detection and tracking, for TRP based sensing (monostatic or bistatic), the antenna configurations used for communication are expected to be re-used. The TRP antennas would typically be located at a height of 25m and the targets to be sensed can fly as high as 300m. When communication networks are operating with an antenna downtilt, it may be difficult to transmit/receive a sufficiently strong signal to/from the UAV to ensure the sensing performance. This problem can be overcome by involving a UE in the sensing procedure since there is more freedom to adjust direction of UE antennas. The UE antennas can be directed upwards and act as a receiver in case of bistatic TRP-UE sensing.

Figure 1 – UAV detection/tracking with gNB-UE bistatic sensing mode

Observation 1: Involving UEs as sensing nodes for UAV detection and tracking can increase the sensing performance compared to pure TRP based sensing.

For Human intruder detection, for indoor environment, only one gNB may provide indoor coverage, thus at least gNB bistatic sensing may be difficult to use. Also, in the indoor scenario, it is often the case that there is no LOS ray between gNB and a target, e.g. due to block of walls. On the other hand, there can exist multiple UEs in the same room which provides more opportunities for better sensing results. For example, for UE monostatic sensing, a UE in the room transmits/receives its own signals to sense movement. Or for gNB-UE bistatic sensing, the UE leverages gNB downlink as an illuminator and receives the reflected signals.

Observation 2: For human intruder detection and tracking, involving a UE in the sensing procedure (e.g. gNB-UE bistatic, UE-UE bistatic or UE monostatic) can overcome indoor related coverage issues that may arise for pure TRP based solutions.

For Automotive related sensing services, the targets are on the ground. For sensing to work accurately, LOS is typically required. This can be ensured by placing UEs at strategic locations. Furthermore, by employing a UE, e.g. for gNB-UE bistatic sensing, the distance between sensing transmitter and receiver can be kept smaller than for gNB-gNB bistatic sensing, resulting in less sensitivity to e.g. angle estimation errors, when obtaining locations of sensing targets.

Figure 1 – Vehicle detection/tracking with gNB-UE bistatic, enabling LOS conditions

Observation 3: For automotive, using UEs as sensing transmitter and/or receiver can increase the LOS probability and optimize the range between sensing transmitter/receiver resulting in accurate location estimates.

Proposal 4: For RAN requirement definition to support sensing services in 6G, all sensing modes, i.e., TRP monostatic, TRP-TRP bistatic, UE monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic (Uu based) can be considered.

- Involving the UE in the sensing procedure should be studied.

- Cooperative sensing and/or multi-static sensing can be included in 6G studies.

Regarding the requirements for the above described new sensing services, the following high level design requirements can be given, enabling the efficient support of sensing, while minimizing the negative impact on communication:

- Sensing Performance

  - The sensing operation should be guaranteed for certain KPIs (e.g. accuracy, coverage, latency), so that the application knows what sensing performance it can expect.

- Impact on communication

  - The impact of ISAC on communication should be minimized, to not cause an unacceptable degradation of existing communication (e.g. with respect to throughput, latency, reliability).

  - It should be possible to schedule sensing operation so that they can be harmonized with communication.

- Efficient use of radio resources

  - Sensing resources should not be over-provisioned relative to their actual need.

  - Reuse of existing communication signals (SSB, CSI-RS, DMRS) for sensing where possible

- Scalability/Flexibility

  - ISAC should scale with target density and sensing demand, without requiring dedicated infrastructure everywhere.

  - Support for distributed transmitters and receivers without overwhelming coordination overhead

- Interoperability

  - At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

Proposal 5: For supporting 6G sensing services, the following requirements can be considered as a guidance,

- Sensing Performance (e.g. accuracy, coverage, latency) should be guaranteed

- Impact on communication shall be minimized.

  - Meeting given sensing requirements should not cause unacceptable degradation of communication KPIs (e.g. throughput, latency, reliability)

- Radio resources should be used efficiently

  - Sensing resources should not be over-provisioned.

  - Reuse of existing communication signals (SSB, CSI-RS, DMRS) for sensing where possible

- Scalability/Flexibility

  - ISAC should scale with target density and sensing demand, without requiring dedicated infrastructure everywhere.

  - Support for distributed transmitters and receivers without overwhelming coordination overhead

- Interoperability

  - At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

Conclusion

In this contribution, we discuss requirements for architecture and migration and make the following observations and proposals:

Observation 1: Involving UEs as sensing nodes for UAV detection and tracking can increase the sensing performance compared to pure TRP based sensing.

Observation 2: For human intruder detection and tracking, involving a UE in the sensing procedure (e.g. gNB-UE bistatic, UE-UE bistatic or UE monostatic) can overcome indoor related coverage issues that may arise for pure TRP based solutions.

Observation 3: For automotive, using UEs as sensing transmitter and/or receiver can increase the LOS probability and optimize the range between sensing transmitter/receiver resulting in accurate location estimates.

Proposal 1: To facilitate requirement definition for sensing as a new service in 6G, RAN selects a subset of use cases from TR 22.837 with focus on the most feasible and promising applications for the near future.

Proposal 2: It is preferred to focus on use cases for object detection/tracking for ISAC in 6G studies.

Proposal 3: To facilitate requirement definition for sensing as a new service in 6G, RAN prioritizes the following use cases

- UAV related use cases

- Human related use cases

- Automotive related use cases

Proposal 4: For RAN requirement definition to support sensing services in 6G, all sensing modes, i.e., TRP monostatic, TRP-TRP bistatic, UE monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic (Uu based) can be considered.

- Involving the UE in the sensing procedure should be studied.

- Cooperative sensing and/or multi-static sensing can be included in 6G studies.

Proposal 5: For supporting 6G sensing services, the following requirements can be considered as a guidance,

- Sensing Performance (e.g. accuracy, coverage, latency) should be guaranteed

- Impact on communication shall be minimized.

  - Meeting given sensing requirements should not cause unacceptable degradation of communication KPIs (e.g. throughput, latency, reliability)

- Radio resources should be used efficiently

  - Sensing resources should not be over-provisioned.

  - Reuse of existing communication signals (SSB, CSI-RS, DMRS) for sensing where possible

- Scalability/Flexibility

  - ISAC should scale with target density and sensing demand, without requiring dedicated infrastructure everywhere.

  - Support for distributed transmitters and receivers without overwhelming coordination overhead

- Interoperability

  - At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

  - Support for distributed transmitters and receivers without overwhelming coordination overhead

- Interoperability

  - At least bistatic sensing operations should support multi-vendor environments. Hence, standardized sensing data formats and signaling procedures should be developed.

- References

[1] RP-243327, CMCC, Verizon, NTT DOCOMO, INC., Deutsche Telekom, “New SID: Study on 6G Scenarios and requirements”

[2] RP-251881, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[3] 3GPP TSG SA, “Feasibility Study on Integrated Sensing and Communication”, TR 22.837, V19.4.0

[4] 3GPP TSG SA, “Service requirements for Integrated Sensing and Communication; Stage 1”, TS 22.137, V19.4.0

[5] 3GPP TSG SA, “Study on Use Cases and Service Requirements; Stage 1”, TR 22.870, V0.3.1

[6] 3GPP TSG RAN, “Study on channel model for frequencies from 0.5 to 100 GHz”, TR 38.901, V19.0.0

- Annex A: Use case grouping defined in TS 22.137

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 | 95 | 10 | 10 | N/A | N/A | 10 [3] | 5 [3] | 1000 | 1 | 5 | 2 | Indoor/outdoor (e.g., detection of human, UAV) |
|  | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |
|  | 4 | 99 for public safety, otherwise, 95 | 0.5 | 0.5 | 1.5 for pedestrian, 15 for vehicle, otherwise, 0.1 | 1.5 for pedestrian | 0.5 | 0.5 x 0.5 for factories | 100 to 5000 | 0.1 | 1 | 3 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV, AGV, vehicle) |
| Environment monitoring | 5 | 95 | 10 | 0.2 (NOTE 4) | N/A | N/A | N/A | N/A | 60000 | 60 to 600 | 0.1 to 5 | 3 | Nature of environments monitored by sensing (e.g. rainfall, flooding monitoring) |
| Motion monitoring | 6 | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60000 | 60 | 5 | 5 | Human motions and activities obtained by sensing (NOTE 5) |
|  | 7 | 95 | 0.2 | 0.2 | 0.1 | 0.1 | 0.375 | 0.3 | 5 to 50 | 0.1 | 5 | 5 | Human hand gestures obtained by sensing (NOTE 6) |
| NOTE 1: For sensing service categories to which UAV, human or vehicle is a sensing target, the typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of human is 0.5m x 0.5m x 1.75m, and the typical size of vehicle is 7.5m x 2.5m x 3.5 m. NOTE 2: The safe distance between pedestrian/vehicle and power transmission station/line is 0.7m/0.95m. NOTE 3: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s. NOTE 4: This value is derived from the water level where people feel difficulty in walking. NOTE 5: To achieve human motion monitoring, different accuracy KPI is needed to measure different human motions. E.g., respiration rate accuracy (2 times/min) is a KPI used to measure the accuracy of sleep monitoring, sit-up rate accuracy (3 times/min) is a KPI used to measure the accuracy of sports monitoring. NOTE 6: Category 7 has more stringent requirements (e.g., for KPIs such as positioning accuracy and sensing resolution) compared to other categories and typically requires more radio resources. |  |  |  |  |  |  |  |  |  |  |  |  |  |