3GPP TSG-RAN Meeting #109	RP-252316

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source:	Intel Corporation

Title:	Views on 6G requirements for new and existing services and support of diverse device types

Document for:	Discussion and decision

# Introduction

In this document we provide our views on 1) 6G requirements for new and existing services, and 2) support of diverse device types to handle various 6G services.

# 6G requirements for new and existing services

## General principles

5G technology supports a broad range of key services, including eMBB, URLLC, and mMTC, as well as many supplementary services such as MBMS, positioning, NTN and V2X as summarized in [2]. This breadth of supported services has been critical in extending the reach of cellular systems beyond traditional mobile broadband into areas such as industrial automation, critical communications, vehicular connectivity and satellite communications.

Looking ahead, 6G is expected to build on this foundation by not only sustaining existing use cases but also enabling new applications that were not possible with earlier generations. At the same time, to ensure delivery of high-quality specifications and support successful 6G commercialization, it is important that 3GPP focuses on a well-defined set of essential services in the initial release of 6G specification. The initial 6G service requirements should therefore aim to establish the baseline capabilities of the system, creating a stable platform for the first deployments. Additional use cases and advanced features can then be introduced incrementally in subsequent releases as part of the natural evolution of the technology.

Proposal #1:	6G day-1 requirements shall focus on key services to ensure delivery of high-quality specifications and support successful 6G commercialization.

Below we provide views on selected existing and new services and their application for 6G technology.

## eMBB evolving to Immersive Communication

Immersive communication is expected to represent the natural evolution of the eMBB service category. It will remain a key element of 3GPP systems and is anticipated to support a wide range of applications, including extended reality (XR/VR), holographic communication, and extremely high-rate broadband access. To meet the requirements of such applications, the service shall provide:

- High data rates to accommodate extremely large volumes of information aiming notable improvement over 5G in both cell average and cell edge performance.

- Low latency to enable real-time interactivity and seamless user experiences in immersive environments aiming same level as of 5G.

- High reliability to ensure service continuity for critical applications aiming same level of reliability as of 5G.

Proposal #2:	6G shall support features required to enable eMBB evolving to Immersive Communication services including

    - High data rates aiming notable improvement over 5G in both cell average and cell edge performance.

    - Low latency aiming same level of latency as of 5G.

    - High reliability aiming same level of reliability as of 5G.

## Computing and AI

With the increasing coupling between communication network infrastructure and cloud computing infrastructure, a natural direction for 6G is to support computing and AI services in the network. This would also allow network operators to benefit from the rapid growth of computing and AI services and develop new revenue streams beyond communication services.

In general, there are two types of services that the 6G system can provide on computing and AI: 1) 6G system provides optimized handling for computing and AI traffic in the air interface and network (e.g., service-aware scheduling); 2) 6G system provides computing and AI services to applications and mobile devices (e.g., computing workload offloading, computing resource discovery, data curation and management).

Computing and AI services impose new requirements on both 6G system design and air interface design. From 6G air interface perspective, computing and AI services may have specific traffic models and new requirements on throughput, latency and reliability. 5G work on supporting XR services is a starting point, but the 6G SI shall aim to continue the work and identify AI traffic characteristics.

Proposal #3:	6G shall support two types of Computing and AI services 1) optimized handling for computing and AI traffic, and 2) computing and AI services for applications and mobile devices. 6GR shall support use case dependent computing/AI requirements, which generally involves new traffic models and new requirements on throughput, latency and reliability. RAN architecture and RAN-CN interface definition shall take into account requirements on computing and AI services.

## Massive Communication for IoT

6G should support IoT devices from day one, ensuring coexistence with legacy 4G/5G IoT technologies such as NB-IoT, eMTC, Cat-1/1bis, RedCap, and eRedCap, while eventually enabling full migration to 6G for all significant IoT use cases. The details of the proposed requirements are described in details in the co-sourced paper [3]. Specifically, low-end IoT UEs should retain simple, cost- and energy-efficient designs, including single-antenna operation, modest peak rates of approximately 5-10 Mbps, limited bandwidth of 3-5 MHz, and HD-FDD support. These features help maintain small form factors, low complexity, and long battery life. In addition to complexity considerations, 6G IoT support should account for performance metrics including end-to-end latency (~100 ms for typical small-packet or conversational voice traffic), extended coverage through repetition mechanisms, and energy-saving techniques such as eDRX and wake-up receivers. High connection density requirements should also be considered to ensure scalable deployments.

Proposal #4:	6G shall support Massive Communication IoT services with detailed requirements outlined in RP-251951

## Integrated Sensing and Communication (ISAC)

Integrated Sensing and Communication (ISAC) is anticipated to be one of the enablers for new type of services in 6G, allowing the radio interface to simultaneously deliver connectivity and environmental awareness. By embedding sensing capabilities into the communication system, 6G can support new classes of applications such as advanced positioning, environmental monitoring, intelligent transportation, and enhanced awareness for safety-critical scenarios.

To ensure feasibility for early deployments, ISAC in 6G should target requirements that are practical and broadly deployable, while leaving room for enhancement in later releases. The requirements can be defined on a use case basis (i.e., object detection and tracking, environment monitoring, motion monitoring) and initial 6G targets can be based the requirements outlined in TS 22.137 such as accuracy of positioning estimate, accuracy of velocity estimate, confidence level, sensing resolution, missed detection and false alarm probabilities, max sensing service latency and refreshing rate. Additional 6GR design requirements may include:

- Coverage: operation across both wide-area and local deployments, leveraging existing communication infrastructure.

- Complexity and cost: solutions should be designed for efficient integration into existing radio architectures, with minimal additional overhead in signaling and processing.

- Power efficiency: sensing operations should avoid significant impact on network and device power consumption.

Proposal #5:		6G shall support ISAC services with the following requirements

    - ISAC KPIs/requirements can be defined on a use case basis (i.e., object detection and tracking, environment monitoring, motion monitoring) and initial 6G targets can be based the requirements outlined in TS 22.137

    - 6G ISAC shall provide coverage and operation across both wide-area and local deployments, leveraging existing communication infrastructure.

    - 6G ISAC solutions should be designed for efficient integration into existing radio architectures, with minimal additional overhead in signaling and processing.

    - 6G ISAC operations should avoid significant impact on network and device power consumption.

## Fixed Wireless Access (FWA)

As highlighted in TR 28.870 [5], Fixed Wireless Access (FWA) in 5G has transformed the home internet business and become an important emerging revenue driver for mobile network operators (MNOs). With 6G, FWA adoption is expected to continue expanding, serving not only residential broadband but also enterprise and private network deployments. To support this growth, 6G systems should be designed to address the specific requirements of FWA use cases, which differ in several respects from traditional mobile broadband - distinct mobility and traffic profiles, the need for robust in-building coverage, and availability of devices with larger form factors, which can take advantage of higher transmit power and the integration of larger antenna arrays, thereby improving link performance and service quality.

To maximize the commercial potential of FWA in 6G, the following target requirements are proposed:

- High throughput: support for multi-gigabit per second peak and sustained data rates to match fibre-like broadband performance.

- Coverage: enhanced support for urban, suburban, rural scenarios, ensuring service availability even in challenging radio in-building coverage environments.

- Latency: sufficiently low latency to enable not only broadband access but also time-sensitive applications (e.g., cloud gaming, remote work, industrial access).

- Device capabilities: leverage higher transmit power and multi-antenna configurations in FWA/CPE devices to extend coverage and improve reliability.

- Cost and deployment efficiency: solutions should minimize total cost of ownership for operators and consumers.

Proposal #6:	6G shall support FWA services aiming for high throughput, coverage, capacity and latency FWA-specific RAN specific optimizations and optimized support for FWA/CPE type of devices

## Aerials

3GPP 4G/5G technologies support various aerial use cases including unmanned aerial vehicles (UAVs) and air-to-ground (ATG) scenarios. For 6G, corresponding requirements should be met from the very beginning to allow the technology to provide respective services. The details of the proposed requirements are proposed in the co-sourced paper [6]. In particular, 6G should support both UAVs (from small drones to larger uncrewed aircraft) and crewed aerial vehicles (e.g., air taxis, helicopters, commercial aircrafts) across low, medium, and high altitudes. It should accommodate diverse mobility profiles, from a few km/h for drones up to and beyond 1000 km/h for commercial and high-performance aircraft.

Proposal #7:		6G shall support aerial services with detailed requirements outlined in RP-251948

## MBMS

5G requirements [2] include support for multicast and broadcast services (MBMS) such as content download, media streaming, group communication, and TV delivery. The existing 4G/5G solutions are expected to be sufficient to address these use cases in the near term and there is no strong urgency to introduce MBMS support in the initial release of 6G. The support of MBMS functionality may add significant burden to both the study and normative work without clear immediate benefits. Instead, we recommend that MBMS services could be considered for later 6G releases if justified by commercial or regulatory demand. Meantime, 6G requirements for multicast and broadcast could still be defined, similar to 5G, with the RAN realization continuing to rely on LTE-based solutions in line with the approach adopted for 5G.

Proposal #8:	MBMS functionality is not supported by 6GR. 6G could support requirements for multicast and broadcast, similar to 5G, in which case their realization would continue to rely on LTE-based solutions in line with the 5G approach.

## Location/Positioning

In 5G, positioning requirements were defined with the objective of enabling RAN-embedded (Cell-ID, E-Cell ID, OTDOA, UTDOA, etc.) and RAN-external (GNSS, Bluetooth, WLAN, Terrestrial Beacon Systems (TBS), sensors, etc.) positioning techniques [2]. In addition, device-to-device (D2D) based positioning techniques were considered. The 5G NR positioning framework further takes advantage of enabling features such as wide bandwidths, sidelink operation, enhanced positioning techniques, etc. These capabilities allow NR positioning to support both indoor and outdoor use cases with improved accuracy, latency, and reliability compared to previous generations.

For the initial release of 6G the primary focus of positioning should be to meet regulatory requirements. This ensures that location services can be reliably deployed where required without adding unnecessary complexity to the initial version of the standard. Addressing more demanding requirements, such as high-accuracy positioning for industrial or indoor applications, may be considered at a later stage if justified by strong commercial drivers. In line with this prioritization, 6G design targets for positioning should aim to maximize the likelihood of large-scale deployment, emphasizing solutions that are simple to deploy, cost-effective, low in implementation complexity, power-efficient, and with minimal signaling or resource overhead.

Proposal #9:	6G shall support the minimum set of positioning requirements and techniques required to meet regulatory requirements and aim to maximize likelihood of large-scale deployment, emphasizing solutions that are simple to deploy, cost-effective, low in implementation complexity, power-efficient, and with minimal signaling or resource overhead.

## Critical Communications

5G provides support for the following critical communications services: 1) public safety communications, 2) emergency communications, and 3) public warning/emergency alert systems [2]. To streamline the development of 6G the support of sidelink operation shall not be considered for the initial release. Therefore, 6G public safety services should be supported over the Uu interface without introducing specific optimizations in the RAN design and the services should remain agnostic to whether they operate over 4G, 5G, or 6G. Public safety use cases requiring sidelink or sidelink relaying should not be redefined in 6G; instead, such services should continue to rely on existing 5G capabilities. This would allow the industry to avoid fragmentation and prevent the introduction of another sidelink variant. For emergency communications and public warning/emergency alert systems, similar to the approach proposed for positioning, it is proposed that 6G should support the set of services mandated by regulatory requirements.

Proposal #10:	6G should support emergency communications and public warning/emergency alert services required to meet regulatory requirements. Public Safety services should be supported over the Uu interface without introducing specific optimizations in the RAN, while use cases requiring sidelink should not be redefined in 6G and can rely on 5G solutions.

## Vehicle-to-Everything (V2X)

In 5G, V2X services are supported through two mechanisms: 1) Uu-based communication, including MBMS, and 2) sidelink communication. For 6G, V2X services should be supported on the Uu interface without introducing V2X-specific RAN or radio optimizations. Services should remain agnostic to whether they operate over 4G, 5G, or 6G. At the same time, it is expected that the 6G Uu system design should, from day-1, efficiently accommodate vehicular-type devices characterized by high mobility, stringent latency and reliability requirements, and operation in high-density scenarios. V2X sidelink and MBMS services should not be redefined in 6G, at least for the initial release. Services requiring these mechanisms should continue to rely on existing 4G/5G capabilities, thereby avoiding fragmentation and preventing multiple sidelink or MBMS variants across generations.

Proposal #11:	6G should support V2X services.

- 6GR Uu system design should, from day-1, efficiently accommodate V2X type of services characterized by high mobility, stringent latency and reliability requirements.

- V2X sidelink and MBMS should not be redefined in 6GR, at least for the initial release of the technology.

# 6G support for diverse device types

6G is expected to support a wide range of device types for various services and use cases, continuing and extending the diversity already introduced in 5G. In addition to baseline handheld devices, 5G has enabled support for a variety of device categories, including FWA/CPE UEs, vehicular-mounted devices, XR wearables, UAVs and ATG devices, and multiple classes of NTN terminals. It is anticipated that 6GR will be required to support these device types as well as potentially new categories emerging from future applications and services.

To facilitate efficient study and specification work within RAN1/2/4, it is important that RANP provides a clear guidance and requirements for specific device categories.

The following guiding principles for 6G device support are considered:

- 6G should address the needs of diverse device types while maintaining a unified system design and avoiding unnecessary fragmentation.

- 6G should provide support for both existing and new device types.

- 6G should enable efficient operation of these devices without requiring additional infrastructure investments or introducing significant system overhead.

- 6G should adopt a scalable PHY design that can accommodate the needs of diverse device types.

- 6G should minimize RAN-specific optimizations for individual device categories, instead reusing common baseband functionalities wherever possible to avoid fragmentation in chip design and leverage economies of scale.

Key differentiating factors across device types may include:

- Device form factor

- Minimum number of Tx/Rx chains

- Minimum supported bandwidth

- Expected data rates (maximum and minimum)

- Supported power classes

Proposal #12:	6G should provide support for diverse set of device types

    - 6G should address the needs of diverse device types while maintaining a unified system design and avoiding unnecessary fragmentation.

    - 6G should provide support for both existing and new device types.

    - 6G should enable efficient operation of these devices without requiring additional infrastructure investments or introducing significant system overhead.

    - 6G should adopt a scalable PHY design that can accommodate the needs of diverse device types, while minimize RAN-specific optimizations for individual device categories, and instead reusing common baseband functionalities wherever possible to avoid fragmentation in chip design and leverage economies of scale.

RANP to further discuss and provide recommendations on the set of device types to be supported for the initial version of 6G technology including the device-specific requirements on form factor, minimum number of Tx/Rx chains, minimum supported bandwidth, expected data rates (maximum and minimum), supported power classes.

# Conclusions

This contribution provides views on 6G requirements for new and existing services, and also on the support of diverse device types to handle various 6G services. In summary, the following proposals are made:

Proposal #1:	6G day-1 requirements shall focus on key services to ensure delivery of high-quality specifications and support successful 6G commercialization.

Proposal #2:	6G shall support features required to enable eMBB evolving to Immersive Communication services including

    - High data rates aiming notable improvement over 5G in both cell average and cell edge performance.

    - Low latency aiming same level of latency as of 5G.

    - High reliability aiming same level of reliability as of 5G.

Proposal #3:	6G shall support two types of Computing and AI services 1) optimized handling for computing and AI traffic, and 2) computing and AI services for applications and mobile devices. 6GR shall support use case dependent computing/AI requirements, which generally involves new traffic models and new requirements on throughput, latency and reliability. RAN architecture and RAN-CN interface definition shall take into account requirements on computing and AI services.

Proposal #4:	6G shall support Massive Communication IoT services with detailed requirements outlined in RP-251951

Proposal #5:		6G shall support ISAC services with the following requirements

    - ISAC KPIs/requirements can be defined on a use case basis (i.e., object detection and tracking, environment monitoring, motion monitoring) and initial 6G targets can be based the requirements outlined in TS 22.137

    - 6G ISAC shall provide coverage and operation across both wide-area and local deployments, leveraging existing communication infrastructure.

    - 6G ISAC solutions should be designed for efficient integration into existing radio architectures, with minimal additional overhead in signaling and processing.

    - 6G ISAC operations should avoid significant impact on network and device power consumption.

Proposal #6:	6G shall support FWA services aiming for high throughput, coverage, capacity and latency FWA-specific RAN specific optimizations and optimized support for FWA/CPE type of devices

Proposal #7:		6G shall support aerial services with detailed requirements outlined in RP-251948

Proposal #8:	MBMS functionality is not supported by 6GR. 6G could support requirements for multicast and broadcast, similar to 5G, in which case their realization would continue to rely on LTE-based solutions in line with the 5G approach.

Proposal #9:	6G shall support the minimum set of positioning requirements and techniques required to meet regulatory requirements and aim to maximize likelihood of large-scale deployment, emphasizing solutions that are simple to deploy, cost-effective, low in implementation complexity, power-efficient, and with minimal signaling or resource overhead.

Proposal #10:	6G should support emergency communications and public warning/emergency alert services required to meet regulatory requirements. Public Safety services should be supported over the Uu interface without introducing specific optimizations in the RAN, while use cases requiring sidelink should not be redefined in 6G and can rely on 5G solutions.

Proposal #11:	6G should support V2X services.

- 6GR Uu system design should, from day-1, efficiently accommodate V2X type of services characterized by high mobility, stringent latency and reliability requirements.

- V2X sidelink and MBMS should not be redefined in 6GR, at least for the initial release of the technology.

Proposal #12:	6G should provide support for diverse set of device types

    - 6G should address the needs of diverse device types while maintaining a unified system design and avoiding unnecessary fragmentation.

    - 6G should provide support for both existing and new device types.

    - 6G should enable efficient operation of these devices without requiring additional infrastructure investments or introducing significant system overhead.

    - 6G should adopt a scalable PHY design that can accommodate the needs of diverse device types, while minimize RAN-specific optimizations for individual device categories, and instead reusing common baseband functionalities wherever possible to avoid fragmentation in chip design and leverage economies of scale.

RANP to further discuss and provide recommendations on the set of device types to be supported for the initial version of 6G technology including the device-specific requirements on form factor, minimum number of Tx/Rx chains, minimum supported bandwidth, expected data rates (maximum and minimum), supported power classes.

# References

TR 38.914 “Study on 6G Scenarios and requirements”

TR 38.913 “Study on scenarios and requirements for next generation access technologies”

RP-251951 “Minimum requirements for Massive Communication for IoT”, Ericsson, AT&T, BT, CHTTL, Deutsche Telekom, FirstNet, Fujitsu, Intel Corporation, KDDI, KT Corp., NTT DOCOMO, OPPO, Semtech, SK Telecom, Sony, Spark NZ, Telstra, Thales, T-Mobile USA, Verizon, RAN #109, September 2025

TS 22.137 “Service requirements for Integrated Sensing and Communication”

TR 28.870 “Study on 6G Use Cases and Service Requirements”

RP-251948 “Requirements for supporting aerial UEs in 6G”, Ericsson, et al, RAN #109, September 2025