3GPP TSG RAN Meeting #109					RP-252319

Beijing, China, September 15-18, 2025

Agenda Item:	8.2.1.4

Source: Rakuten Mobile

Title: Requirements for 6G NTN interworking with TN

Document for:	Approval

# 1	Introduction

Non-Terrestrial Networks (NTN) are entering operator roadmaps as an extension of terrestrial networks (TN) to achieve ubiquitous handheld coverage and resilience. To accelerate practical deployment while protecting existing terrestrial assets and spectrum investments, this paper proposes service-level requirements for:

- NTN↔TN Mobility:

- NTN↔TN Spectrum Sharing (MRSS):

- Energy aware Coordination between NTN ↔TN.

# 2	Discussion

### 2.1 NTN–TN Mobility (6G NTN - NR/LTE TN)

Handheld UE users will move between NTN beams/cells and TN cells, or will require fallback from NTN to TN due to visibility, blockage or policy. To ensure service continuity while avoiding too many options we recommend mobility support between 6G NTN↔5G TN as well as 6G NTN ↔ 4G TN.

### 2.2 NTN - TN Spectrum Sharing (MRSS), Interference Mitigation, Cross-Border Management

Many operators intend to utilise their existing TN spectrum for early or sustained NTN service where regulation permits. Service requirements should therefore:

- Support TN ↔NTN MRSS in both FDD and TDD, enabling dynamic time/frequency partitioning with rate-matching around TN signals (e.g., SSB/PRS/legacy LTE RS) .

- Provide explicit methods for interference mitigation and management, including:

  - Spectral masks/PFD envelopes, power control, and beam shaping appropriate for NTN-TN spectrum sharing.

  - Scheduling windows (time–frequency coordination opportunities) to separate terrestrial signals from NTN bursts.

- Include cross-border interference avoidance provisions: geo-constraints and coordination hooks to respect national borders and protect adjacent TN deployments.

### 2.3 Energy Savings through NTN/TN Coordination

To ensure sustainable operation as NTN scales:

- Require load-aware coordination between NTN and TN (e.g., traffic steering thresholds, opportunistic idling/sleep windows across domains, and energy-biased scheduling) while maintaining agreed QoS.

# 3 Text proposal

Based on the discussion above we propose the following text proposal.

## 5.x	Functional requirements for a 6G Network with satellite access

### 5.x.x NTN–TN Mobility for service Continuity

6G RAN shall support Idle and connected mode mobility between Non Terrestrial Networks and Terrestrial Networks, including:

- 6G NTN ↔ NR TN/NTN

- 6G NTN ↔ LTE TN

The Mobility between 6G NTN and inter RAT TN should minimize service interruption time; and achieving low handover failure rate in handheld scenarios.

### 5.x.x NTN-TN Spectrum Sharing

MRSS shall enable dynamic time/frequency partitioning and rate-matching policies that:

- Protect terrestrial reference/signalling (e.g., SSB/PRS/legacy LTE RS) when co-present;

- Accommodate NTN waveform/timing characteristics (Doppler/RTT) without degrading TN KPIs.

- MRSS operation between NTN -TN scenarios shall support interference mitigation and management techniques, including:

  - Applicable spectral masks/PFD envelopes and power control for NTN transmissions in shared spectrum;

  - Scheduling windows for TN/NTN separation of resources.

- Cross-border interference avoidance shall be supported via geo-constraints and coordination methods to protect adjacent terrestrial deployments across national borders.

### 5.x.x Energy aware Coordination between NTN and TN

6G RAN shall support energy-aware coordination between NTN and TN with the objective of reducing network/device energy consumption without violating agreed QoS.

Energy-aware coordination may include:

- load-aware traffic steering between NTN and TN;

- coordinated idling/sleep windows across NTN/TN where service conditions permit; and

- Energy-biased scheduling policies in shared spectrum operation.

# 4	Conclusion

In this contribution we propose to add the text proposal in section 3 to TR 38.914.