3GPP TSG RAN Meeting #109		RP-252453

Beijing, China, 15th – 18th September 2025

Agenda item:	9.13

Source:	Apple

Title:	On the introduction of the higher power classes for the existing NR and IOT NTN bands

Release:	Rel-20

Document for:	Information

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN. The corresponding WI was completed in August 2025 introducing core RF requirements for the PC2 (26dBm) and PC1 (31dBm) power classes for both NR and IOT NTN bands. However, only two exemplary bands – L-band n255 and S-band n256 – were considered for the band specific requirements.

As discussed during the corresponding NTN high-power WI, PC2 and PC1 power classes for other NTN bands should be also supported, either through the maintenance track or through the corresponding "basket" spectrum WID. Thus, in this discussion paper we present a general overview on which existing NR and IOT NTN bands need further specification efforts to enable high-power classes.

# 2	NTN band and power classes

Table 2 below summarises existing NTN bands, both NR and IOT, with further information on the release when the band was introduced and which power classes are defined. As can be seen from the table, the Rel-19 NTN high-power class WI enabled PC2 and PC1 only for the "exemplary" bands n255 and n256, which were introduced back in Rel-17. Other Rel-18 and brand new Rel-19 bands (such as n252 or b249) are not covered yet by the 3GPP specifications.

Table 2-1: Summary of the existing NTN NR/IOT bands and the NTN power classes.

| NTN band |  | Release | PC3 | PC2 | PC1 |
| --- | --- | --- | --- | --- | --- |
| NR | n255 | Rel-17 | X | X | X |
|  | n256 | Rel-17 | X | X | X |
|  | n254 | Rel-18 | X |  |  |
|  | n252 | Rel-19 | X |  |  |
|  | n253 | Rel-19 | X |  |  |
|  | n250 | Rel-19 | X |  |  |
|  | n251 | Rel-19 | X |  |  |
| IOT | b255 | Rel-17 | X | X | X |
|  | b256 | Rel-17 | X | X | X |
|  | b254 | Rel-18 | X |  |  |
|  | b253 | Rel-18 | X |  |  |
|  | b252 | Rel-19 | X |  |  |
|  | b249 | Rel-19 | X |  |  |

Thus, we suggest initiating a new Rel-20 spectrum "basket" WI aiming at enabling missing NTN power classes in the existing NTN bands. The potential objectives of the WI may look as follows (the exact list of bands will be clarified further with interested companies and satellite operators):

-	Introduce support of the PC2 power class (26dBm):

-	List of considered NR NTN bands: [n250, n251, n253, n252, n254]

-	List of considered IOT NTN bands: [b249, b253, b252, b254]

-	Define, if needed, A-MPR requirements accounting for the general and band specific requirements

-	Define, if needed, RSD requirements

-	Introduce support of the PC1 power class (31dBm):

-	List of considered NR NTN bands: [n250, n251, n253, n252, n254]

-	List of considered IOT NTN bands: [b249, b253, b252, b254]

-	Define, if needed, A-MPR requirements accounting for the general and band specific requirements

-	Define, if needed, RSD requirements

Proposal:	Initiate a new Rel-20 spectrum WI to enable missing NTN high-power classes for the existing NTN NR/IOT bands.

# 3	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"