3GPP TSG-RAN WG Meeting #109	                                          RP-252461

Beijing, China, 15 – 18, Sep, 2025

Source: 	Xiaomi

Title:  	Views on R20 AI/ML for Mobility

Agenda Item:	9.3.2.1

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting, RAN WG2 work item on “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” [2] was approved.  However, the details of the objectives are related to RAN2 and RAN4 discussion in Aug meeting. In this contribution, we discuss the scope of R20 AI/ML for mobility based on the progress in Aug meeting.

# Discussion

## RAN2 part

In RAN2#131 meeting, the SI of AI mobility was concluded with following recommendation for WI,

Based on what is summarized above, RRM and measurement event prediction are recommended for normative work. The following scenarios and/or sub-cases are recommended for normative work: - For UE sided model (RRM and measurement event prediction), intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case, - For network sided model (RRM prediction), at least RRM sub-case 2 of intra-frequency temporal domain case A for inference input report and all scenarios and sub-cases for data collection.

It is unclear in current WID, what scenarios/cases are supported for UE sided model. RAN2 has done lots of simulation results on prediction accuracy. The simulation results showed the prediction can be close to the ground truth in some cases. The RSRP difference can be very small. Therefore, for UE sided model, (RRM and measurement event prediction), intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case should be included in the R20 WI.

Proposal 1: For UE sided model, (RRM and measurement event prediction), follow RAN2 recommendations, i.e., intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case should be included in the R20 WI.

It’s FFS whether L3 beam prediction is included for UE sided model as following NOTE,

Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

For beam level prediction, TR captures following observations,

Limited simulation results are submitted for intra-cell spatial domain prediction and L3 beam level prediction. For RRM measurement prediction, L3 beam-level prediction is feasible, however there are concerns on UE complexity and uncertainty of impacts/evaluations in other WG for UE sided model.

Companies didn’t have enough evaluation on the prediction performance of L3 beam prediction, due to lack of simulation results. Also, there is concern on the potential impact to RAN4. Since RAN4 didn’t study the L3 beam prediction in the study phase, additional work is expected for RAN4. Therefore, we propose not to include L3 beam prediction in R20 WI.

Proposal 2: L3 beam prediction is not included in R20 WI.

Event prediction can be done in direct or indirect way. RAN2 has agreed to prioritize indirect prediction and the related simulation assumptions. Based on the simulation results, RAN2 concluded that the prediction result is accurate under certain scenarios. In addition, the event prediction can improve the mobility performance or save UE power by reducing the measurement. However, there is no consensus on how to monitor the performance of direct event prediction, since companies can’t find appropriate performance metrics for direct event prediction. It’s important for NW to monitor the prediction performance. Otherwise, the mobility performance would be degraded. Therefore, we propose to only support indirect event prediction in R20 WI.

Proposal 3: Only indirect event prediction is included in the R20 WI.

Regrading NW sided model, it’s FFS which use case is supported in WI, as following NOTE,

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI.

The TR captures following observation for NW sided model,

For network sided model, all scenarios and all RRM sub-cases are feasible based on existing specification. For intra-frequency temporal domain case A sub-case 2 enhancement is needed. For other cases there is no specification impact. However, they can be discussed in WI phase whether enhancement (i.e. multi-instances reporting of beam) is needed and justified.

The model input can be L1 beam-level measurement result, i.e. sub-case 1 or L3 cell level measurement result, i.e. sub-case. Only the L3 cell-level measurement result as model input in temporal case A should be included in R20 WI. Other cases, including L1 beam-level measurement result as model input, can be achieved by legacy procedure without spec impact.

Although the TR indicates they can be discussed in WI phase whether enhancement (i.e. multi-instances reporting of beam) is needed and justified for other use cases. We understand the enhancement is mainly about NW side data collection. We don’t see the need to support any specific enhancement for other use cases as well. Unified data collection framework should be supported for all use cases in NW side model.

Proposal 4: For network sided model (RRM prediction), L3 cell-level measurement result as model input in intra-frequency temporal domain case A is included in R20 WI. Other use cases, e.g., spatial domain can be achieved without spec impact and are not included in R20 WI.

## RAN4 part

During SI phase, RAN4 major focused on Interoperability, testability and RRM requirements.

The SI confirmed the feasibility of AI/ML-based mobility prediction and captured baseline anchors on measurement error modeling (BB via LL-derived L3-RSRP difference; RF via truncated Gaussian with per-layer/chain granularity to be discussed in WI) and on the relative RSRP accuracy definitions for intra/inter-frequency (beam-level: FFS depending on RAN2).

RAN4 also noted no interoperability issue impacting feasibility and agreed to initiate discussion on generalization testing in WI, while leaving LCM-related testability to WI once requirements are clear.

As performance requirements and concrete test cases were not specified in SI, we propose to update the objectives of Rel-20 WID accordingly.

Proposal 5: The objectives for RAN4 can be updated as following:

- Core requirements for both RRM measurement prediction and measurement event prediction [RAN4]:

  - Evaluate RAN4 testability and interoperability

  - Specify RRM core requirements for above AI/ML mobility cases

  - Specify LCM-related requirements, if any

- Performance requirements and test cases for both RRM measurement prediction and measurement event prediction [RAN4]

## Text proposal on the WID

Based on above proposals, the objectives of WID can be updated.

Proposal 6: The objectives of WID can be updated as following,

The objectives of this work item focuses on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, RRM measurement configuration and report framework in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - Following use cases are supported for UE sided model: Temporal domain prediction case A and case B, Frequency domain prediction for collocated cases and indirect event prediction. Only Cell level predication is supported. - Following use cases are supported for NW sided model: L3 cell-level measurement result as model input in intra-frequency temporal domain case A. - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction - Measurement event A1~A6 are in the scope in this release - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY) - Inference configuration - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline Note1: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Network sided data collection, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 2: The above objectives to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - Core requirements for both RRM measurement prediction and measurement event prediction [RAN4]: - Evaluate RAN4 testability and interoperability - Specify RRM core requirements for above AI/ML mobility cases - Specify LCM-related requirements, if any - Performance requirements and test cases for both RRM measurement prediction and measurement event prediction [RAN4]

# Conclusions

Proposal 1: For UE sided model, (RRM and measurement event prediction), follow RAN2 recommendations, i.e., intra-frequency temporal domain case A, intra-frequency temporal domain case B and inter-frequency domain prediction for co-located case should be included in the R20 WI.

Proposal 2: L3 beam prediction is not included in R20 WI.

Proposal 3: Only indirect event prediction is included in the R20 WI.

Proposal 4: For network sided model (RRM prediction), L3 cell-level measurement result as model input in intra-frequency temporal domain case A is included in R20 WI. Other use cases, e.g., spatial domain can be achieved without spec impact and are not included in R20 WI.

Proposal 5: The objectives for RAN4 can be updated as following:

- Core requirements for both RRM measurement prediction and measurement event prediction [RAN4]:

  - Evaluate RAN4 testability and interoperability

  - Specify RRM core requirements for above AI/ML mobility cases

  - Specify LCM-related requirements, if any

- Performance requirements and test cases for both RRM measurement prediction and measurement event prediction [RAN4]

Proposal 6: The objectives of WID can be updated as following,

The objectives of this work item focuses on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, RRM measurement configuration and report framework in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction for both UE and network sided models [RAN2]: - Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction - Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction - Following use cases are supported for UE sided model: Temporal domain prediction case A and case B, Frequency domain prediction for collocated cases and indirect event prediction. Only Cell level predication is supported. - Following use cases are supported for NW sided model: L3 cell-level measurement result as model input in intra-frequency temporal domain case A. - Specify support for measurement event prediction for UE sided models [RAN2]: - Specify signaling and protocol aspect to enable reporting inference outcome of UE sided model for measurement event prediction - Measurement event A1~A6 are in the scope in this release - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach (considering further progress for AI/ML PHY) - Inference configuration - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection, for which AI/ML PHY request/configuration framework is taken as baseline Note1: Data transfer and model delivery for UE sided model are out of scope of this WID. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Network sided data collection, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 2: The above objectives to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - Core requirements for both RRM measurement prediction and measurement event prediction [RAN4]: - Evaluate RAN4 testability and interoperability - Specify RRM core requirements for above AI/ML mobility cases - Specify LCM-related requirements, if any - Performance requirements and test cases for both RRM measurement prediction and measurement event prediction [RAN4]

# Reference

[1] TR 38.744

[2] RP-251864 New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR