3GPP TSG RAN Meeting #109									RP-252493

Beijing, China, September 15-18, 2025

## Status Report to TSG

Agenda item:			9.6.4.3

| WI / SI Name | NR Radio Resource Management (RRM) Phase 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_Ph5 |  |  |  |  |
| Unique ID | 1031079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-242380 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 09/2025 | Performance part: 03/2026 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:100% RAN4: 100% | Performance Part: 40% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Aijun Cao |
|  | Company | CATT |
|  | Email | ajcao@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#116 meeting

The following tdocs have been approved/endorsed in RAN4#116 meeting for core part:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2512123 | Ad-hoc minutes for NR_RRM_Ph5 WI | Apple |
| R4-2512124 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2512125 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |
| R4-2512333 | LS on Rx BSF optimization for NR RRM Phase 5 | CATT |
| R4-2512289 | Draft CR on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple, CATT, Nokia, CMCC, Huawei, HiSilicon |
| R4-2512161 | Reply LS on on CSSF optimization for NR RRM Phase 5 | Apple |
| R4-2512347 | Draft CR for CSSF optimization of RRM Phase 5 | OPPO, Huawei, HiSilicon |

Topic #1: FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Issue 1-1-1: Value for T1

Agreement:

- A fixed value of 90 seconds.

Issue 1-1-2: Re-evaluation mechanism

Agreement:

FFS following options in the maintenance stage:

- Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be:

    - Alt 1: Use the values for T1 as a starting point.

    - Alt 2: 5 s, which is predefined in RAN4 spec.

- Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS.

- Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS.

Agreement in Friday online:

- No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

The following UE capability is defined:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-2 | Fast Rx beam sweeping factor for FR2-1 L3 measurement delay reduction | Supports fast Rx beam sweeping factor reduction for L3 measurement for FR2-1. |  | Yes | N/A | UE does not support fast Rx beam sweeping for FR2-1 L3 measurement delay reduction. | Per band | TDD only | FR2-1 only | N/A | Candidate values: {2, 4, 6} for FR2-1. Note: It is only supported for power class 3. | Optional with capability signalling |

Issue 1-1-4: Requirements when UE reports different BSF values for different bands

Way forward:

Further discuss this issue in the maintenance stage:

  - Option 1:

    - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

Issue 1-1-5: Revision of the agreement on applicability requirement

Agreement in Tuesday ad-hoc:

- The FBS enhancement applies for the case when the target and serving carrier frequency are same.

- FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different.

Agreement in Thursday online:

When the target and serving carrier frequency are different,

- FBS requirement does not apply to inter-frequency measurement without measurement gap.

- FFS inter-frequency measurement with per-FR measurement gap.

- Note: inter-RAT is not in the scope.

Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

Issue 2-1-1: baseline setup for BSF test case

- Proposal 1(Samsung):

  - For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

  - To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

    - How to define the test parameters involving hysteresis and the TTT/evaluation period of the timer can wait for further solid conclusions.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-1-2: whether or not to test FBS quitting

- Option 1(CMCC): it is proposed to discuss whether to test FBS quiting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Topic #2: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Issue 1-1: issues related with solution 1 (measure one serving CC per band)

Issue 1-1-1: LS reply to RAN2 (R2-2504728) on solution 1

Agreement:

Reply LS R4-2512161 has been agreed.

RAN4 would like to thank RAN2 for the LS on the CSSF optimization for NR RRM Phase 5. RAN4 had discussion on the alternatives from RAN2, and following conclusions have been made: - About the network indication of a specific SCC per-band for measurement, RAN4 agreed to adopt an alternative 3 as following: - If UE support R19 CSSF solution 1 (measurement one CC per FR2-1 band) and network enables this feature, - Follow the new RRC network indication (if present) to measure the indicated SCC in an FR2-1 band, if - Neither PCC nor PSCC is in the same FR2-1 band, and - The indicated SCC to be measured shall be one of the SCCs on which UE is configured to report SSB based measurements in the same FR2-1 band. - For case that only MO(s) are configured on SCC(s) in an FR2-1 band, when neither PCC nor PSCC is in the same FR2-1 band, and there is no SCCs on which UE is configured to report SSB based measurements in this FR2-1 band, UE will select and perform serving and intra-frequency neighbour cell measurements on one SCC in that FR2-1 band. SCC selection in this FR2-1 band to measure is up to UE implementation. - RAN4 kindly requests RAN2 to implement RRC signalling enabling network to indicate which SCC the UE shall measure in an FR2-1 band.

Issue 1-1-2: revision on solution 1 according to issue 1-1-1

- Discuss in draft CR

Issue 1-1-3: Expand solution 1 to FR1 case

Agreement:

Not extend the solution 1 to FR1.

Issue 1-2: issues related with solution 3 (3 searchers)

Issue 1-2-1: applicability limitation for solution 3

Agreement:

For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG.

FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

Issue 1-3: issues related UE capability for CSSF enhancement

Issue 1-3-1: detailed UE capability for CSSF enhancement

Agreement:

Solution 1 (measure one CC per band):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-3 | L3 serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap | Support of serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. |  | Yes |  | UE does not support serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. | Per-UE | No | FR2-1 only |  | UE indicating this capability shall meet the corresponding enhanced requirements in TS38.133 Clause 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4 | Optional with capability signaling |

Solution 3 (3 searchers):

This capability has been agreed in last meeting, and following is just to remove the square bracket

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-1 | Simultaneous L3 measurement on three carriers for measurements without measurement gap under CA/DC operation. | Support of measuring serving cell and neighbor cells measurement on three carriers simultaneously for measurements without measurement gap. Support of separate indications for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA PCell is FR1 only}, and {FR1+FR2 NR-DC PCell is FR1 only}. |  | Yes |  | UE does not support measuring serving cell and neighbor cells on three carriers simultaneously for measurements without measurement gap. | Per-UE | No | No |  | UE indicating this capability shall meet the corresponding enhanced requirements defined in TS38.133 Clause 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3 if the feature is supported. | Optional with capability signaling |

TC for solution 1:

Issue 2-2-1: baseline TC setup for CSSF optimization solution 1

- Proposal 1(Nokia): A scenario with at least two carrier frequencies in one FR2 band is required to verify the measurement delay with enhanced CSSF.

- Proposal 2 (Nokia): The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-2-2: sub-TC for CSSF optimization solution 1 with network indication

- Option 1(Apple): for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting.

- Option 2 (Nokia): To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-2-3: TC for CSSF optimization solution 1 in FR1

- Proposal 1(Nokia, CMCC):

  - To define the test cases to verify the measurement delay by measuring one SCC per FR1 band, if this would be agreed in core requirement discussion.

- Agreement:

  - This issue is closed since issue 1-1-3 agreed to not extend the solution 1 to FR1

TC for solution 3:

Issue 2-2-4: baseline TC setup for CSSF optimization solution 3

- Proposal 1(Huawei):

  - To unify the test set of Solution 3 test cases, the test set of Solution is able to refer to four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Topic #3: Fast SCell activation for UE supporting Rel-18 EMR

Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-1-1: UE capability for EMR-based fast SCell activation

Agreement in Friday online:

- Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Test case list

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-2: Applicability rules of test case

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-3: Test case design

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-4: Report mechanism of EMR results in the test cases

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#116 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2509054 | Topic summary for [116][211] NR_RRM_Ph5_Part1 | Moderator (Apple) |
| R4-2509055 | Topic summary for [116][212] NR_RRM_Ph5_Part2 | Moderator (CATT) |
| R4-2509158 | On remaining FBS requirements | Nokia |
| R4-2509159 | DraftCR for FBS activation conditions | Nokia |
| R4-2509309 | DraftCR to update the applicability requirements for UE supporting L3 fast beam sweeping operation in FR2-1 | CATT |
| R4-2509310 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | CATT |
| R4-2509311 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CADC | CATT |
| R4-2509312 | Discussion on performance requirements for FR2-1 L3 measurement delay reduction | CATT |
| R4-2509313 | Discussion on performance requirements of fase SCell activation for UE supporting R18 EMR | CATT |
| R4-2509314 | DraftCR to define test case for SSB based Inter-frequency measurement delay reduction without MG with fast beam sweeping | CATT |
| R4-2509440 | On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple |
| R4-2509441 | Draft CR on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple |
| R4-2509460 | Discussion of RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | Apple |
| R4-2509504 | On feature list for RRM phase 5 WI | Apple |
| R4-2509505 | On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Apple |
| R4-2509506 | draft CR for Rel-19 CSSF enhancement solution 1 (one CC measurement per band) | Apple |
| R4-2509507 | On testing for CSSF enhancement in CA/DC | Apple |
| R4-2509508 | Big CR for NR Radio Resource Management (RRM) Phase 5 | Apple |
| R4-2509509 | Reply LS on on CSSF optimization for NR RRM Phase 5 | Apple |
| R4-2509612 | FR2-1 SSB based L3 measurement delay reduction with CSSF optimization | Nokia, Nokia Shanghai Bell |
| R4-2509613 | draftCR on CSSF optimization solutions | Nokia, Nokia Shanghai Bell |
| R4-2509614 | On performance part of L3 measurement delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2509615 | draftCR on test case for intra-frequency measurement with CSSF solution 1 | Nokia, Nokia Shanghai Bell |
| R4-2509637 | Discussion on RRM requirements for FR2-1 L3 measurement delay reduction | LG Electronics Inc. |
| R4-2509680 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | OPPO |
| R4-2509681 | Discussion on RRM core requirements of Fast SCell activation for UE supporting Rel-18 EMR | OPPO |
| R4-2509683 | DraftCR on test cases for Multi-Rx based handover | OPPO |
| R4-2509684 | Discussion on performance requirements for fast SCell activation for UE supporting Rel-18 EMR | OPPO |
| R4-2509688 | On optimizing CSSF for FR2-1 L3 measurement delay | OPPO |
| R4-2509689 | Draft CR for CSSF optimization of RRM Phase 5 | OPPO |
| R4-2509690 | Draft CR on TC of inter-frequency measurement without MG with CSSF solution 3 | OPPO |
| R4-2509927 | DraftCR: Test cases requirements for RRC re-establishment and release with redirection | Intel Corporation |
| R4-2510081 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | China Telecom |
| R4-2510082 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | China Telecom |
| R4-2510083 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2510084 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | China Telecom |
| R4-2510154 | Discussion on L3 measurement delay reduction by optimizing Rx beam sweeping factor | CMCC |
| R4-2510155 | Discussion on L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC | CMCC |
| R4-2510156 | Discussion on fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2510157 | Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode | CMCC |
| R4-2510158 | Discussion on RRM performance requirements for fast SCell activation for UE supporting Rel-18 EMR | CMCC |
| R4-2510173 | DraftCR on activation condition for optimizing Rx beam sweeping factor | CMCC |
| R4-2510176 | DraftCR on TC on Rx BSF optimization for SSB based Intra-frequency measurement without MG (TC#1) | CMCC |
| R4-2510186 | Discussion on remaining issues on FR 2-1 L3 measurement delay by optimizaing CSSF outside gap in CADC | vivo |
| R4-2510187 | Discussion on remanining issues on FR 2-1 L3 measurement delay by optimizing Rx beam sweeping factor | vivo |
| R4-2510385 | Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC | Ericsson |
| R4-2510386 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Ericsson |
| R4-2510387 | (NR_RRM_Ph5-Perf) Draft CR on SSB based Inter-frequency measurement without MG with CSSF optimization solution 1 | Ericsson |
| R4-2510388 | (NR_RRM_Ph5-Perf) Draft CR on SSB based Intra-frequency measurement without MG with CSSF solution 3 | Ericsson |
| R4-2510451 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2510452 | Discussion on the fast SCell activation for UE supporting Rel-18 EMR | ZTE Corporation, Sanechips |
| R4-2510495 | Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR | vivo |
| R4-2510547 | Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC | Huawei, HiSilicon |
| R4-2510548 | Discussion on performance part for RRM Phase 5 | Huawei, HiSilicon |
| R4-2510555 | Draft CR on CSSF reduction Solution 3 SA mode for Rel-19 | Huawei, HiSilicon |
| R4-2510556 | Draft CR on L3 measurement delay by optimizing CSSF performance for Rel-19 | Huawei, HiSilicon |
| R4-2510591 | Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Huawei, HiSilicon |
| R4-2510592 | DraftCR on FBS L3 measurement for inter-frequency without gap | Huawei, HiSilicon |
| R4-2510593 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2510594 | DraftCR on TC for Intra-frequency measurement with MG | Huawei, HiSilicon |
| R4-2510595 | Discussion on performance for Fast SCell activation for UE supporting Rel-18 EMR | Huawei, HiSilicon |
| R4-2510676 | Discussion on CSSF optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2510677 | Discussion on Rx beam factor optimization of R19 RRM enhancements | ZTE Corporation, Sanechips |
| R4-2510724 | Draft CR on Test case of Inter-RAT measurement without MG of R19 CSSF enh | ZTE Corporation, Sanechips |
| R4-2510789 | RRM performance requirements of Fast Scell activation for UE supporting Rel-18 EMR | Ericsson |
| R4-2510790 | RRM core requirement of fast scell activaiton for UE supporting Rel-18 EMR | Ericsson |
| R4-2511088 | Discussion on L3 measurement enhancement by optimizing Rx BSF | Samsung |
| R4-2511089 | Discussion on test case on SSB based inter-frequency measurement delay reduction | Samsung |
| R4-2511090 | Draft CR on measurement delay reduction for SSB based Inter-frequency measurement | Samsung |
| R4-2511409 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR core part | Nokia |
| R4-2511410 | CR to 38.133 on Fast SCell activation with EMR | Nokia |
| R4-2511411 | Discussion on Fast SCell activation for UE supporting Rel-18 EMR performance part | Nokia |
| R4-2511514 | Fast SCell activation based on Rel-18 eEMR | Qualcomm Incorporated |
| R4-2511515 | Fast SCell activation based on Rel-18 eEMR | Qualcomm Incorporated |
| R4-2511567 | Enhancement on FR2 CSSF | Qualcomm Incorporated |
| R4-2511649 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor | MediaTek inc. |
| R4-2511650 | Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF | MediaTek inc. |
| R4-2511655 | Draft big CR for NR Radio Resource Management (RRM) Phase 5 | Apple |
| R4-2512123 | Ad-hoc minutes for NR_RRM_Ph5 | Apple |
| R4-2512124 | WF on RRM requirements for NR_RRM_Ph5_Part1 | Apple |
| R4-2512125 | WF on RRM requirements for NR_RRM_Ph5_Part2 | CATT |
| R4-2512127 | WF on FS_NR_AIML_Mob_Part1 | Nokia |
| R4-2512161 | Reply LS on on CSSF optimization for NR RRM Phase 5 | Apple |
| R4-2512240 | Draft CR for CSSF optimization of RRM Phase 5 | OPPO, Huawei, HiSilicon |
| R4-2512289 | Draft CR on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor | Apple, CATT, Nokia, CMCC, Huawei, HiSilicon |
| R4-2512325 | Draft CR for CSSF optimization of RRM Phase 5 | OPPO, Huawei, HiSilicon |
| R4-2512333 | LS on Rx BSF optimization for NR RRM Phase 5 | CATT |
| R4-2512347 | Draft CR for CSSF optimization of RRM Phase 5 | OPPO, Huawei, HiSilicon |
| R4-2512348 | Draft big CR for NR Radio Resource Management (RRM) Phase 5 | Apple |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template