3GPP TSG RAN WG1 #122	        		   			R1-2506661

# Bangalore, India, August 25th – 29th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0722 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on enhancements of network energy savings for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Uncessesary parameter od-ssb-physCellId in clause 4.4. - Missing component of “adaptation” of SS/PBCH block transmissions on SCell in title of clause 4.4. - Clarify that od-ss-PBCH-BlockPower may not be provided in clause 7. - Clarify that the SSB bandwidth may be punctured in clause 10. - Missing reference to activated/adapted SS/PBCH blocks in clause 4.4 in determining symbols that cannot change to UL in clause 11.1.1. - Missing description that k-ssb provided by SIB1-RequestConfig also indicates a frequency offset for CORESET#0 in clause 23. - Incorrect text color in clause 8.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove parameter od-ssb-physCellId in clause 4.4. - Add “adaptation” to the title of clause 4.4. - Capture that od-ss-PBCH-BlockPower may not be provided in clause 7. - Capture that the SSB bandwidth may be punctured in clause 10. - Add reference to activated/adapted SS/PBCH blocks in clause 4.4 in determining symbols that cannot change to UL in clause 11.1.1. - Add description that k-ssb provided by SIB1-RequestConfig also indicates a frequency offset for CORESET#0 in clause 23. - Correct text color in clause 8.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 7, 8.1, 10, 11.1.1, 23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged parts are omitted ***

## 4.4	Activation/adaptation/deactivation of SS/PBCH block transmissions on a secondary cell

A UE can be indicated transmission of first SS/PBCH blocks with frequency location [9, TS 38.104] provided by absoluteFrequencySSB on an SCell. The UE can be indicated, by od-ssb-config [12, TS 38.331] or by a MAC CE [11, TS 38.321], activation of transmission for second SS/PBCH blocks in a configured DL BWP of the SCell [19, TS 38.300]. The UE can be indicated, by a MAC CE, adaptation of parameters for the activated transmission of the second SS/PBCH blocks when the SCell is activated. A number of half frames with transmission of the second SS/PBCH blocks is indicated by a MAC CE from values provided by od-ssb-nrofBurst, if provided; otherwise, the transmission of the second SS/PBCH blocks occurs until it is deactivated by od-ssb-config or a MAC CE [11, TS 38.321], where

-	the physical cell identity of the second SS/PBCH blocks is indicated by physCellId in ServingCellConfigCommon

-	the indexes of transmitted second SS/PBCH blocks are indicated by a MAC CE from candidate values provided by od-ssb-PositionsInBurst, if provided; otherwise, by ssb-PositionsInBurst

-	the frequency location of the second SS/PBCH blocks is indicated by od-absoluteFrequencySSB, if provided; otherwise, by absoluteFrequencySSB

-	the SCS configuration of the second SS/PBCH blocks is indicated by od-ssbSubcarrierSpacing, if provided; otherwise, by ssbSubcarrierSpacing

-	the power of the second SS/PBCH blocks is indicated by od-ss-PBCH-BlockPower, if provided; otherwise, by ss-PBCH-BlockPower

-	the periodicity of the transmission of the second SS/PBCH blocks are indicated by a MAC CE from candidate values by od-ssb-Periodicity

-	the half frames for the transmission of the second SS/PBCH blocks are determined based on an indication by a MAC CE

-	the transmission of the second SS/PBCH blocks is in frames with SFN determined from SFN+SFN_offset⋅10 mod P=0, where P is the periodicity for the transmission of the second SS/PBCH blocks, and SFN_offset is the indicated SFN offset by the MAC CE from candidate values by od-ssb-sfn-Offset, if provided; else, SFNoffset=0. An index of a half frame with transmission of the second SS/PBCH blocks in a corresponding frame is indicated by the MAC CE from candidate values by od-ssb- halfFrameIndex, if provided; else the index is 0

When the activation or adaptation of the second SS/PBCH blocks transmission is by a MAC CE, respectively, and with reference to slots for PUCCH transmission, the UE expects that the transmission of the second SS/PBCH blocks according to the indicated parameters starts from a first slot including the candidate SS/PBCH block occasion corresponding to the first transmitted second SS/PBCH block index and located in a first half frame within the half frames for the transmissions of the second SS/PBCH blocks, that is at least m+3 Nslotsubframe,μ+1 slots after slot n, where n is a slot when a PDSCH reception providing the MAC CE ends, respectively, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211].

When the deactivation of the second SS/PBCH blocks transmission is by a MAC CE, and with reference to slots for PUCCH transmission, the UE expects that the transmission of the second SS/PBCH blocks according to the indicated parameters terminates from the end of

-	a slot n+T, if the slot n+T is not within a first slot to a last slot with activated transmission of second SS/PBCH blocks in a half frame, where T=m+3 Nslotsubframe,μ+1, n is a slot when a PDSCH reception providing the MAC CE ends, n+m is a slot indicated for PUCCH transmission with HARQ-ACK information for the PDSCH reception as described in clause 9.2.3, and Nslotsubframe,μ is a number of slots per subframe for the SCS configuration μ of the PUCCH transmission as defined in [4, TS 38.211]

-	the first slot including the candidate SS/PBCH block corresponding to the index of the last transmitted second SS/PBCH block that is not earlier than the slot n+T, if the slot n+T is within a first slot to a last slot with activated transmission of second SS/PBCH blocks in a half frame

When the UE is not provided absoluteFrequencySSB for the SCell, the UE does not expect the transmission of the second SS/PBCH blocks provided by od-ssb-config to be deactivated while the SCell is activated.

When the first SS/PBCH blocks in a configured DL BWP can be used to obtain SIB1 and the frequency location of the first SS/PBCH blocks corresponds to the GSCN of a synchronization raster entry, the UE expects

-	a frequency location of the second SS/PBCH blocks to be different from the frequency location of the first SS/PBCH blocks and not to correspond to the GSCN of a synchronization raster entry

-	frequency resources of the second SS/PBCH blocks not to overlap with frequency resources of the first SS/PBCH blocks

-	the second SS/PBCH blocks to be within the configured DL BWP as the first SS/PBCH blocks

-	the second SS/PBCH blocks are not used to obtain SIB1

When the first SS/PBCH blocks in a configured DL BWP cannot be used to obtain SIB1, the UE expects

-	a same frequency location for the second SS/PBCH blocks and for the first SS/PBCH blocks

-	a same PBCH payload, other than the SFN index and the half frame index, for a first SS/PBCH block, from the first SS/PBCH blocks, and for a second SS/PBCH block, from the second SS/PBCH blocks, with same SS/PBCH block index as the first SS/PBCH block

The UE may assume that a first SS/PBCH block from the first SS/PBCH blocks and a second SS/PBCH block from the second SS/PBCH blocks are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SS/PBCH block index.

The UE may assume that SS/PBCH blocks from the second SS/PBCH blocks are quasi co-located with respect to Doppler shift, Doppler spread, average gain, average delay, delay spread and, when applicable, spatial RX parameters, when they have a same SS/PBCH block index.

# 5	Radio link monitoring

*** Unchanged parts are omitted ***

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ  is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE, and for each indicated one or two TCI-State or TCI-UL-State or CandidateTCI-State or CandidateTCI-UL-State of a PUSCH, PUCCH, or SRS transmission occasion as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS, or by pathlossReferenceRS-Id included in CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	in clause 7.1.1, if p0AlphaSetforPUSCH is provided, or if p0AlphaSetforPUSCH-SBFD is provided and for a PUSCH transmission in SBFD symbols as described in clause 11.1, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH or by p0AlphaSetforPUSCH-SBFD, respectively, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUSCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	in clause 7.2.1, if p0AlphaSetforPUCCH is provided, or if p0AlphaSetforPUCCH-SBFD is provided and for a PUCCH transmission in SBFD symbols, the values of PO_UE_PUCCHqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH or by p0AlphaSetforPUCCH-SBFD, respectively, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforPUCCH associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	in clause 7.3.1, if p0AlphaSetforSRS is provided,

-	if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of  PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with the indicated TCI-State or TCI-UL-State, or by p0AlphaSetforSRS associated with the CandidateTCI-State or CandidateTCI-UL-State indicated in the LTM Cell Switch Command MAC CE

-	else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and a value of a PLb,f,c,offset, when TCI-State or a TCI-UL-State includes pl-Offset, is provided by pl-Offset associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set

PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set.

In the remaining of this clause, except for clause 7.7.3, if in a PUSCH, PUCCH, or SRS transmission occasion i a UE applies a TCI-State or a TCI-UL-State that includes pl-Offset with value PLb,f,c,offset in dB [11, TS 38.321], the UE sets PLb,f,cqd=PLb,f,cqd-PLb,f,c,offset.

In the remaining of this clause, if a UE obtains a downlink pathloss estimate based on reception of second SS/PBCH blocks, as described in Clause 4.4, ss-PBCH-BlockPower is replaced by od-ss-PBCH-BlockPower, if provided.

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2.

*** Unchanged parts are omitted ***

## 8.1	Random access preamble

Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following:

-	A configuration for PRACH transmission on the cell [4, TS 38.211].

-	A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell.

-	A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions.

A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions.

For Type-1 random access procedure, a UE can be provided, by addl-RACH-Config-Adaptation in RACH-ConfigCommon, parameters for determining time resources and frequency resources for PRACH transmission [4, TS 38.211]. When a PRACH occasion associated with addl-RACH-Config-Adaptation has same frequency resource and same time resource as a PRACH occasion in RACH-ConfigCommon that is not associated with addl-RACH-Config-Adaptation, the former PRACH occasion is not valid and is not considered in the procedures in this clause.

For valid PRACH occasions associated with addl-RACH-Config-Adaptation in RACH-ConfigCommon, the UE can be additionally provided a PRACH mask index, by prach-SubsetMask-Index-Adaptation, that indicates one or more association periods per Kmask association pattern periods according to Table 8.1-0, where Kmask is provided by KforAPPForPRACHsubsetMask.

Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods

| Mask Index | Association periods (APs) per Kmask association pattern periods (APPs) |
| --- | --- |
| 0 | First half of APs in Kmask APPs |
| 1 | First quarter of APs in Kmask APPs |
| 2 | First eighth of APs in Kmask APPs |
| 3 | First sixteenth of APs in Kmask APPs |

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI or a C-RNTI [5, TS 38.212]. For indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI.

In the following, for determining valid PRACH occasions, if a UE is not provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH blocks is for the union of SS/PBCH blocks provided by each combination of od-ssb-PositionsInBurst and od-ssb-Periodicity in od-ssb-Config. If the UE is provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH blocks is for the union of SS/PBCH blocks provided by ssb-Periodicity and ssb-PositionsInBurst in ServingCellConfigCommon and by each combination of od-ssb-PositionsInBurst and od-ssb-Periodicity in od-ssb-Config.

In the following, for determining an association between PRACH occasions and SS/PBCH block indexes, if a UE is not provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH block indexes provided by ssb-PositionsInBurst in ServingCellConfigCommon is for the union of SS/PBCH block indexes provided by each od-ssb-PositionsInBurst in od-ssb-Config. If the UE is provided SS/PBCH block indexes by ssb-PositionsInBurst in ServingCellConfigCommon and is provided od-ssb-Config, reference to SS/PBCH block indexes provided by ssb-PositionsInBurst in ServingCellConfigCommon is for the union of SS/PBCH block indexes provided by ssb-PositionsInBurst in ServingCellConfigCommon and by each od-ssb-PositionsInBurst in od-ssb-Config.

*** Unchanged parts are omitted ***

# 10	UE procedure for receiving control information

*** Unchanged parts are omitted ***

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-	does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received an indication for transmission of second SS/PBCH blocks on a serving cell as described in Clause 4.4, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, from the second SS/PBCH blocks, corresponding to a SS/PBCH block index provided as described in Clause 4.4,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE, if the UE

-	has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and

-	at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate,

the UE is not required to monitor the PDCCH candidate.

*** Unchanged parts are omitted ***

### 11.1.1	UE procedure for determining slot format

*** Unchanged parts are omitted ***

For a set of symbols of a slot that are indicated as downlink/uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink/downlink, respectively, or as flexible. For a set of symbols of a slot that are indicated as SBFD by tdd-UL-DL-ConfigurationCommon, the UE ignores the indication by the SFI-index field value.

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in clause 4.1, or as described in clause 4.4, or by NonCellDefiningSSB or, if the UE is not provided dl-OrJointTCI-StateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

For a set of symbols of a slot corresponding to a valid PRACH occasion and Ngap symbols before the valid PRACH occasion, as described in clause 8.1, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as downlink.

*** Unchanged parts are omitted ***

# 23	UE procedure to request SIB1 reception

Unless otherwise mentioned, the higher layer parameters in this clause and in referenced clauses are provided by SIB1-RequestConfig on a first cell.

A UE can be provided, by NES_CellId, a physical cell identity of a second cell and an ARFCN by ARFCN-ValueNR for SS/PBCH block receptions on the second cell. When

-	the UE receives an SS/PBCH block on the second cell, and

-	kSSB>23 for FR1 or kSSB>11 for FR2 is indicated by the SS/PBCH block on the second cell, and

-	conditions for PRACH transmission associated with the SS/PBCH block on the second cell are satisfied [12, TS 38.331],

the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI and can transmit a PRACH associated with the SS/PBCH block on the second cell based on:

-	a timing adjustment indicated by n-TimingAdvanceOffset, if provided, as described in Clause 4.2

-	a power determined as described in Clause 7.4

-	a procedure determined as in Clause 8.1 based on Type-1 random access procedure

where kSSB for determining the common resource block [4, TS 38.211] and the frequency offset for the CORESET in Clause 13 is provided by k-ssb.

In response to the PRACH transmission, the UE monitors PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by ra_ResponseWindow, as described in Clause 8.2. The UE monitors PDCCH according to a Type1-PDCCH CSS set provided by ra-SearchSpace, if provided; else provided by searchSpaceZero as described in Clause 10.1, in a CORESET provided by controlResourceSetZero.

If the higher layers at the UE identify a RAPID associated with a corresponding PRACH transmission from the UE in a PDSCH reception scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI, the higher layers indicate to the physical layer to monitor PDCCH on the second cell to detect a DCI format 1_0 with CRC scrambled by the SI-RNTI according to a Type0-PDCCH CSS set provided by searchSpaceZero. If the UE is provided sib1-pdcchRestrictionToPRACHAssociatedSSBs, the UE monitors PDCCH only in monitoring occasions associated with the SS/PBCH block; otherwise, the UE assumes that PDCCH is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. The UE monitors PDCCH to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI after a number of slots provided by od-sib1-windowStartOffset from the start of the first slot of the window controlled by ra_ResponseWindow, and for a number of slots provided by od-sib1-WindowDuration.

After the PRACH transmission, the UE is not required to monitor PDCCH on the second cell to detect the DCI format 1_0 with CRC scrambled by the SI-RNTI prior to a reception of a PDSCH providing a RAPID associated with a corresponding PRACH transmission scheduled by the DCI format 1_0 with CRC scrambled by the RA-RNTI.

*** Unchanged parts are omitted ***